# Lab - Configure BGP Route Reflector (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

# Topology

# Addressing Table

| Device | Interface | IPv4 Address | IPv6 Address | IPv6 Link-Local |
| --- | --- | --- | --- | --- |
| R1 | G0/0/0 | 10.0.0.2/30 | 2001:db8:feed:1::2/64 | fe80::1:1 |
| R1 | G0/0/1 | 209.165.200.5/30 | 2001:db8:209:4::5/64 | fe80::1:2 |
| R2 | G0/0/0 | 10.0.0.1/30 | 2001:db8:feed:1::1/64 | fe80::2:1 |
|  | G0/0/1 | 10.0.0.5/30 | 2001:db8:feed:2::5/64 | fe80::2:2 |
|  | Lo0 | 172.16.1.1/24 | N/A | N/A |
|  | Lo1 | 172.16.2.1/24 | N/A | N/A |
| R2 | Lo2 | N/A | 2001:db8:a001:1::1/64 | EUI-64 |
| R2 | Lo3 | N/A | 2001:db8:a002:1::1/64 | EUI-64 |
| R3 R3 | G0/0/0 | 10.0.0.6/30 | 2001:db8:feed:2::6/64 | fe80::3:1 |
|  | G0/0/1 | 209.165.200.9/30 | 2001:db8:209:8::9/64 | fe80::3:2 |
| D1 D1 | G1/0/11 | 209.165.200.6/30 | 2001:db8:209:4::6/64 | fe80::d1:1 |
|  | Lo0 | 192.168.1.1/24 | N/A | N/A |
|  | Lo1 | 192.168.2.1/24 | N/A | N/A |
|  | Lo2 | N/A | 2001:db8:cab::1/48 | EUI-64 |
| D2 D2 | G1/0/11 | 209.165.200.10/30 | 2001:db8:209:8::10/64 | fe80::d2:1 |
|  | N/A | 192.168.10.1/24 | N/A | N/A |
|  | N/A | 192.168.20.1/24 | N/A | N/A |
|  | N/A | N/A | 2001:db8:acad::1/48 | EUI-64 |

# Objectives

Part 1: Build the Topology and Configure Basic Device Settings and IP Addressing

Part 2: Examine and Verify OSPFv3 and BGP Reachability

Part 3: Configure R2 as a BGP Route Reflector

Part 4: Verify BGP Adjacencies and Tables

Part 5: Verify IP Routing Tables

# Background / Scenario

In this lab, AS 65000 uses OSPFv3 as its Interior Gateway Protocol (IGP) to provide internal reachability for both IPv4 and IPv6. OSPFv3 is configured with separate address families, allowing a single IGP to support dual-stack operation. Loopback interfaces are used to represent internal IPv4 and IPv6 prefixes within AS 65000. These prefixes simulate customer or internal networks and are later advertised into BGP.

AS 65000 runs BGP using address families (AFs) for both IPv4 and IPv6, enabling a unified BGP process to handle dual-stack routing. External autonomous systems AS 65001 and AS 65002 use traditional BGP for IPv4 and the IPv6 unicast address family for IPv6. Because IPv6 routing in BGP is supported only through address families, all IPv6 BGP configuration and route exchange in this lab occurs within the IPv6 AF context.

Although routers R1, R2, and R3 in AS 65000 are already configured for BGP, no internal BGP (iBGP) neighbor adjacencies have been configured between them at this point in the lab. In a traditional iBGP design, these routers would form a full-mesh of iBGP sessions to ensure that all BGP routes are shared within the autonomous system. However, full-mesh iBGP does not scale well as the number of routers increases. In this lab, instead of configuring a full mesh, R2 will be deployed as a BGP route reflector, allowing R1 and R3 to exchange BGP routing information through R2 using a scalable route reflector design.

Routers D1 and D2 are already configured for BGP and maintain eBGP adjacencies with routers R1 and R3, respectively. These routers originate their local prefixes into BGP and represent external autonomous systems connected to AS 65000.

Note: Although the BGP standards (e.g., RFC 4271) define the abbreviations as EBGP and IBGP, modern vendor documentation often uses the lower-case forms eBGP and iBGP to visually distinguish “external” or “internal” from the core BGP protocol name. We will use lower case e and i to make is easier to visually distinguish these two protocols.

Note: The routers used with this CCNP hands-on lab are Cisco 8200Ls and the two Layer 3 switches are Catalyst 9300L switches using Cisco IOS XE Release 17.12.05 (universalk9 image). Other routers and Layer 3 switches and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs.

Note: Ensure that the routers and switches have been erased and have no startup configurations. If you are unsure contact your instructor.

Instructor Note: Refer to the Instructor Lab Manual for the procedures to initialize and reload devices.

# Required Resources

3 Routers (Cisco Catalyst 8200L with Cisco IOS XE Release 17.12.05c universal image or comparable)

2 Switches (Cisco Catalyst 9300L with Cisco IOS XE Release 17.12.05 universal image or comparable)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

# Instructions

## Build the Network and Configure Basic Device Settings and Interface Addressing

In Part 1, you will set up the network topology and configure basic settings and interface addressing on the routers and switches.

### Cable the network as shown in the topology.

Attach the devices as shown in the topology diagram, and cable as necessary.

### Configure basic settings for each router.

Console into each device, enter global configuration mode, and apply the basic settings and interface addressing using the following startup configurations for each device.

Open configuration window

Router R1

hostname R1

!

no ip domain lookup

line con 0

exec-timeout 0 0

logging sync

!

ipv6 unicast-routing

!

router ospfv3 1

!

address-family ipv4 unicast

router-id 1.1.1.1

exit-address-family

!

address-family ipv6 unicast

router-id 1.1.1.1

exit-address-family

!

interface GigabitEthernet0/0/0

ip address 10.0.0.2 255.255.255.252

ipv6 address 2001:db8:feed:1::2/64

ipv6 address fe80::1:1 link-local

ospfv3 network point-to-point

ospfv3 1 ipv6 area 0

ospfv3 1 ipv4 area 0

no shutdown

exit

!

interface GigabitEthernet0/0/1

ip address 209.165.200.5 255.255.255.252

ipv6 address 2001:db8:209:4::5/64

ipv6 address fe80::1:2 link-local

no shutdown

exit

!

router bgp 65000

bgp log-neighbor-changes

neighbor 2001:DB8:209:4::6 remote-as 65001

neighbor 209.165.200.6 remote-as 65001

!

address-family ipv4

neighbor 209.165.200.6 activate

exit-address-family

!

address-family ipv6

neighbor 2001:DB8:209:4::6 activate

exit-address-family

!

end

Router R2

hostname R2

!

no ip domain lookup

line con 0

exec-timeout 0 0

logging sync

!

ipv6 unicast-routing

!

router ospfv3 1

!

address-family ipv4 unicast

router-id 2.2.2.2

exit-address-family

!

address-family ipv6 unicast

router-id 2.2.2.2

exit-address-family

!

!

interface GigabitEthernet0/0/0

ip address 10.0.0.1 255.255.255.252

ipv6 address 2001:db8:feed:1::1/64

ipv6 address fe80::2:1 link-local

ospfv3 network point-to-point

ospfv3 1 ipv6 area 0

ospfv3 1 ipv4 area 0

no shutdown

exit

!

interface GigabitEthernet0/0/1

ip address 10.0.0.5 255.255.255.252

ipv6 address 2001:db8:feed:2::5/64

ipv6 address fe80::2:2 link-local

ospfv3 network point-to-point

ospfv3 1 ipv6 area 0

ospfv3 1 ipv4 area 0

no shutdown

exit

!

interface loopback0

ip address 172.16.1.1 255.255.255.0

ipv6 enable

ospfv3 1 ipv4 area 0

ospfv3 network point-to-point

no shutdown

exit

!

interface loopback1

ip address 172.16.2.1 255.255.255.0

ipv6 enable

ospfv3 1 ipv4 area 0

ospfv3 network point-to-point

no shutdown

exit

!

interface loopback2

ipv6 address 2001:db8:a001:1::1/64

ospfv3 1 ipv6 area 0

ospfv3 network point-to-point

no shutdown

exit

!

interface loopback3

ipv6 address 2001:db8:a002:1::1/64

ospfv3 1 ipv6 area 0

ospfv3 network point-to-point

no shutdown

exit

!

router bgp 65000

bgp log-neighbor-changes

!

address-family ipv4

network 172.16.0.0 mask 255.255.0.0

exit-address-family

!

address-family ipv6

network 2001:DB8:A000::/36

exit-address-family

!

ip route 172.16.0.0 255.255.0.0 Null0

ipv6 route 2001:db8:a000::/36 Null0

!

end

Router R3

hostname R3

!

no ip domain lookup

line con 0

exec-timeout 0 0

logging sync

!

ip routing

ipv6 unicast-routing

!

router ospfv3 1

!

address-family ipv4 unicast

router-id 3.3.3.3

exit-address-family

!

address-family ipv6 unicast

router-id 3.3.3.3

exit-address-family

!

interface GigabitEthernet0/0/0

ip address 10.0.0.6 255.255.255.252

ipv6 address 2001:db8:feed:2::6/64

ipv6 address fe80::3:1 link-local

ospfv3 network point-to-point

ospfv3 1 ipv6 area 0

ospfv3 1 ipv4 area 0

no shutdown

exit

!

interface GigabitEthernet0/0/1

ip address 209.165.200.9 255.255.255.252

ipv6 address 2001:db8:209:8::9/64

ipv6 address fe80::3:2 link-local

no shutdown

exit

!

router bgp 65000

bgp log-neighbor-changes

neighbor 2001:DB8:209:8::10 remote-as 65002

neighbor 209.165.200.10 remote-as 65002

!

address-family ipv4

neighbor 209.165.200.10 activate

exit-address-family

!

address-family ipv6

neighbor 2001:DB8:209:8::10 activate

exit-address-family

!

end

Switch D1

hostname D1

!

no ip domain lookup

line con 0

exec-timeout 0 0

logging sync

!

ip routing

ipv6 unicast-routing

!

interface GigabitEthernet1/0/11

no switchport

ip address 209.165.200.6 255.255.255.252

ipv6 address 2001:db8:209:4::6/64

ipv6 address fe80::d1:1 link-local

no shutdown

exit

!

interface loopback0

ip address 192.168.1.1 255.255.255.0

! ipv6 enable

no shutdown

exit

!

interface loopback1

ip address 192.168.2.1 255.255.255.0

! ipv6 enable

no shutdown

exit

!

interface loopback2

ipv6 address 2001:db8:cab::1/48

no shutdown

exit

!

router bgp 65001

bgp log-neighbor-changes

neighbor 2001:DB8:209:4::5 remote-as 65000

neighbor 209.165.200.5 remote-as 65000

!

address-family ipv4

network 192.168.1.0 mask 255.255.255.0

network 192.168.2.0 mask 255.255.255.0

neighbor 209.165.200.5 activate

exit-address-family

!

address-family ipv6

network 2001:DB8:CAB::/48

neighbor 2001:DB8:209:4::5 activate

exit-address-family

!

end

Switch D2

hostname D2

!

no ip domain lookup

line con 0

exec-timeout 0 0

logging sync

!

ip routing

ipv6 unicast-routing

!

interface GigabitEthernet1/0/11

no switchport

ip address 209.165.200.10 255.255.255.252

ipv6 address 2001:db8:209:8::10/64

ipv6 address fe80::d2:1 link-local

no shutdown

exit

!

interface loopback0

ip address 192.168.10.1 255.255.255.0

! ipv6 enable

no shutdown

exit

!

interface loopback1

ip address 192.168.20.1 255.255.255.0

! ipv6 enable

no shutdown

exit

!

interface loopback2

ipv6 address 2001:db8:acad::1/48

no shutdown

exit

!

router bgp 65002

bgp log-neighbor-changes

neighbor 2001:DB8:209:8::9 remote-as 65000

neighbor 209.165.200.9 remote-as 65000

!

address-family ipv4

network 192.168.10.0 mask 255.255.255.0

network 192.168.20.0 mask 255.255.255.0

neighbor 209.165.200.9 activate

exit-address-family

!

address-family ipv6

network 2001:DB8:ACAD::/48

neighbor 2001:DB8:209:8::9 activate

exit-address-family

!

end

Save the running configuration to startup-config.

Close configuration window

## Examine and Verify OSPFv3 and BGP Reachability

In the configurations in Part 1, OSPFv3 provides the internal routing foundation for AS 65000, enabling full IPv4 and IPv6 reachability between routers R1, R2, and R3. Each router in AS 65000 can reach every other router at the IP layer. At this stage, OSPFv3 is functioning strictly as the underlay protocol, providing transport reachability for BGP sessions and next hops within the autonomous system.

External connectivity is provided using eBGP. R1 peers with D1 in AS 65001, and R3 peers with D2 in AS 65002. As a result, R1 is receiving IPv4 prefixes (192.168.1.0/24 and 192.168.2.0/24) and the IPv6 prefix (2001:db8:cab::/48) originated by AS 65001, and R3 is receiving IPv4 prefixes (192.168.10.0/24 and 192.168.20.0/24) and the IPv6 prefix (2001:db8:acad::/48) originated by AS 65002.

However, no iBGP neighbor relationships have been configured within AS 65000. Because of this, the BGP routes learned via eBGP on R1 and R3 are not propagated to the rest of the autonomous system. Although router R2 has been configured to advertise aggregate IPv4 prefix (172.16.0.0/16) and IPv6 prefix (2001:db8:cab::/48), it currently has no iBGP neighbors and therefore does not exchange BGP routing information with R1 or R3.

In this part of the lab, you will verify OSPFv3 reachability within AS 65000 and confirm the current BGP routing state.

### Verify IP routing tables on R1, R2, and R3.

Examine the IPv4 and IPv6 routing tables on R1.

R1# show ip route | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 3 subnets, 2 masks

C        10.0.0.0/30 is directly connected, GigabitEthernet0/0/0

L        10.0.0.2/32 is directly connected, GigabitEthernet0/0/0

O        10.0.0.4/30 [110/2] via 10.0.0.1, 01:01:25, GigabitEthernet0/0/0

172.16.0.0/24 is subnetted, 2 subnets

O        172.16.1.0 [110/2] via 10.0.0.1, 01:01:25, GigabitEthernet0/0/0

O        172.16.2.0 [110/2] via 10.0.0.1, 01:01:25, GigabitEthernet0/0/0

B     192.168.1.0/24 [20/0] via 209.165.200.6, 00:56:19

B     192.168.2.0/24 [20/0] via 209.165.200.6, 00:56:19

209.165.200.0/24 is variably subnetted, 2 subnets, 2 masks

C        209.165.200.4/30 is directly connected, GigabitEthernet0/0/1

L        209.165.200.5/32 is directly connected, GigabitEthernet0/0/1

R1# show ipv6 route | begin 2001

C   2001:DB8:209:4::/64 [0/0]

via GigabitEthernet0/0/1, directly connected

L   2001:DB8:209:4::5/128 [0/0]

via GigabitEthernet0/0/1, receive

B   2001:DB8:CAB::/48 [20/0], tag 65001

via FE80::D1:1, GigabitEthernet0/0/1

O   2001:DB8:A001:1::/64 [110/2]

via FE80::2:1, GigabitEthernet0/0/0

O   2001:DB8:A002:1::/64 [110/2]

via FE80::2:1, GigabitEthernet0/0/0

C   2001:DB8:FEED:1::/64 [0/0]

via GigabitEthernet0/0/0, directly connected

L   2001:DB8:FEED:1::2/128 [0/0]

via GigabitEthernet0/0/0, receive

O   2001:DB8:FEED:2::/64 [110/2]

via FE80::2:1, GigabitEthernet0/0/0

L   FF00::/8 [0/0]

via Null0, receive

On R1, the OSPF route to the 10.0.0.4/30 network provides reachability to R3 through R2, confirming that OSPFv3 is successfully exchanging internal routing information within AS 65000. This internal OSPF connectivity allows R1 to reach all routers in the autonomous system at the IP layer.

The 172.16.1.0/24 and 172.16.2.0/24 networks are also learned via OSPFv3 from R2. These networks originate from loopback interfaces on R2 and are advertised individually into OSPFv3. Although R2 has been configured to advertise the summarized prefix 172.16.0.0/16 into BGP, R1 does not see this aggregate route in its routing table because there is no iBGP adjacency between R1 and R2. As a result, BGP routing information generated by R2 is not exchanged within AS 65000 at this time.

R1 does see the 192.168.1.0/24 and 192.168.2.0/24 networks in its routing table as BGP routes, learned via eBGP from D1 in AS 65001. These routes are present only on R1 because they were learned from an external neighbor and have not been propagated to other routers within AS 65000.

This is similar for the information in R3's IPv6 routing table.

Examine the IP routing tables on R3.

R3# show ip route | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 3 subnets, 2 masks

O        10.0.0.0/30 [110/2] via 10.0.0.5, 01:05:52, GigabitEthernet0/0/0

C        10.0.0.4/30 is directly connected, GigabitEthernet0/0/0

L        10.0.0.6/32 is directly connected, GigabitEthernet0/0/0

172.16.0.0/24 is subnetted, 2 subnets

O        172.16.1.0 [110/2] via 10.0.0.5, 01:05:52, GigabitEthernet0/0/0

O        172.16.2.0 [110/2] via 10.0.0.5, 01:05:52, GigabitEthernet0/0/0

B     192.168.10.0/24 [20/0] via 209.165.200.10, 01:04:16

B     192.168.20.0/24 [20/0] via 209.165.200.10, 01:04:16

209.165.200.0/24 is variably subnetted, 2 subnets, 2 masks

C        209.165.200.8/30 is directly connected, GigabitEthernet0/0/1

L        209.165.200.9/32 is directly connected, GigabitEthernet0/0/1

R3# show ipv6 route | begin 2001

IPv6 Routing Table - default - 9 entries

C   2001:DB8:209:8::/64 [0/0]

via GigabitEthernet0/0/1, directly connected

L   2001:DB8:209:8::9/128 [0/0]

via GigabitEthernet0/0/1, receive

O   2001:DB8:A001:1::/64 [110/2]

via FE80::2:2, GigabitEthernet0/0/0

O   2001:DB8:A002:1::/64 [110/2]

via FE80::2:2, GigabitEthernet0/0/0

B   2001:DB8:ACAD::/48 [20/0], tag 65002

via FE80::D2:1, GigabitEthernet0/0/1

O   2001:DB8:FEED:1::/64 [110/2]

via FE80::2:2, GigabitEthernet0/0/0

C   2001:DB8:FEED:2::/64 [0/0]

via GigabitEthernet0/0/0, directly connected

L   2001:DB8:FEED:2::6/128 [0/0]

via GigabitEthernet0/0/0, receive

L   FF00::/8 [0/0]

via Null0, receive

Examine the IP routing tables on R2.

In this lab, R2 is the single origination point for the aggregated prefixes to simplify the configuration and highlight the role of iBGP. The network 172.16.0.0/16 is an aggregation all subnets such as Loopback0 172.16.1.0/24 and Loopback1 172.16.2.0/24 networks. In production networks, these summaries are often originated on multiple routers to avoid a single point of failure.

When an AS advertises only an aggregate prefix to its external BGP peers, internal routing changes do not trigger external BGP updates, as long as some route within the aggregate remains reachable. AS 65001 advertises aggregated prefixes so that internal routing changes do not trigger external BGP updates, which reflects real-world design practices used to improve stability and reduce BGP churn on the Internet. Traffic will be forwarded to AS then dropped if prefix is not present.

R2# show ip route | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 4 subnets, 2 masks

C        10.0.0.0/30 is directly connected, GigabitEthernet0/0/0

L        10.0.0.1/32 is directly connected, GigabitEthernet0/0/0

C        10.0.0.4/30 is directly connected, GigabitEthernet0/0/1

L        10.0.0.5/32 is directly connected, GigabitEthernet0/0/1

172.16.0.0/16 is variably subnetted, 5 subnets, 3 masks

S        172.16.0.0/16 is directly connected, Null0

C        172.16.1.0/24 is directly connected, Loopback0

L        172.16.1.1/32 is directly connected, Loopback0

C        172.16.2.0/24 is directly connected, Loopback1

L        172.16.2.1/32 is directly connected, Loopback1

R2# show ipv6 route | begin 2001

S   2001:DB8:A000::/36 [1/0]

via Null0, directly connected

C   2001:DB8:A001:1::/64 [0/0]

via Loopback2, directly connected

L   2001:DB8:A001:1::1/128 [0/0]

via Loopback2, receive

C   2001:DB8:A002:1::/64 [0/0]

via Loopback3, directly connected

L   2001:DB8:A002:1::1/128 [0/0]

via Loopback3, receive

C   2001:DB8:FEED:1::/64 [0/0]

via GigabitEthernet0/0/0, directly connected

L   2001:DB8:FEED:1::1/128 [0/0]

via GigabitEthernet0/0/0, receive

C   2001:DB8:FEED:2::/64 [0/0]

via GigabitEthernet0/0/1, directly connected

L   2001:DB8:FEED:2::5/128 [0/0]

via GigabitEthernet0/0/1, receive

L   FF00::/8 [0/0]

via Null0, receive

On R2, there are no OSPF-learned routes in the routing tables because, from R2’s perspective, there are no remote OSPF networks to learn. Its OSPF-enabled interfaces and loopbacks are all directly connected.

At this stage, no iBGP neighbor adjacencies are configured, so R2 has no BGP routes in either the IPv4 or IPv6 routing tables.

The static routes to Null0 ensure that the aggregate prefixes 172.16.0.0/16 and 2001:DB8:A000::/36 exist in R2’s routing tables, because BGP will only advertise a network when an exact match exists in the routing table. Using Null0 is a safe way to advertise aggregate routes and prevent traffic from being forwarded if no more-specific route exists

### Verify IP routing tables on D1 and D2.

Examine the IP routing tables on D1.

On D1, the IPv4 and IPv6 routing tables currently contain only directly connected and local routes. These include the loopback networks 192.168.1.0/24, 192.168.2.0/24, and 2001:DB8:CAB::/48, as well as the directly connected link to R1.

Although an eBGP session is established with R1 in AS 65000, D1 is not receiving any BGP routes from R1 at this time. This is expected because R1 is not advertising any BGP-learned or internal routes back toward D1. As a result, D1 has no BGP routes installed in its routing tables.

D1# show ip route | begin Gateway

Gateway of last resort is not set

192.168.1.0/24 is variably subnetted, 2 subnets, 2 masks

C        192.168.1.0/24 is directly connected, Loopback0

L        192.168.1.1/32 is directly connected, Loopback0

192.168.2.0/24 is variably subnetted, 2 subnets, 2 masks

C        192.168.2.0/24 is directly connected, Loopback1

L        192.168.2.1/32 is directly connected, Loopback1

209.165.200.0/24 is variably subnetted, 2 subnets, 2 masks

C        209.165.200.4/30 is directly connected, GigabitEthernet1/0/11

L        209.165.200.6/32 is directly connected, GigabitEthernet1/0/11

D1# show ipv6 route | begin 2001

C   2001:DB8:209:4::/64 [0/0]

via GigabitEthernet1/0/11, directly connected

L   2001:DB8:209:4::6/128 [0/0]

via GigabitEthernet1/0/11, receive

C   2001:DB8:CAB::/48 [0/0]

via Loopback2, directly connected

L   2001:DB8:CAB::1/128 [0/0]

via Loopback2, receive

L   FF00::/8 [0/0]

via Null0, receive

Examine similar output for the IP routing tables on D2.

D2# show ip route | begin Gateway

Gateway of last resort is not set

192.168.10.0/24 is variably subnetted, 2 subnets, 2 masks

C        192.168.10.0/24 is directly connected, Loopback0

L        192.168.10.1/32 is directly connected, Loopback0

192.168.20.0/24 is variably subnetted, 2 subnets, 2 masks

C        192.168.20.0/24 is directly connected, Loopback1

L        192.168.20.1/32 is directly connected, Loopback1

209.165.200.0/24 is variably subnetted, 2 subnets, 2 masks

C        209.165.200.8/30 is directly connected, GigabitEthernet1/0/11

L        209.165.200.10/32 is directly connected, GigabitEthernet1/0/11

D2# show ipv6 route | begin 2001

C   2001:DB8:209:8::/64 [0/0]

via GigabitEthernet1/0/11, directly connected

L   2001:DB8:209:8::10/128 [0/0]

via GigabitEthernet1/0/11, receive

C   2001:DB8:ACAD::/48 [0/0]

via Loopback2, directly connected

L   2001:DB8:ACAD::1/128 [0/0]

via Loopback2, receive

L   FF00::/8 [0/0]

via Null0, receive

### Examine BGP adjacencies and tables for eBGP on R1/D1 and R3/D2.

The eBGP adjacencies between R1 and D1, and between R3 and D2, have already been indirectly verified in the previous section by observing that R1 and R3 have installed IPv4 and IPv6 BGP routes learned from AS 65001 and AS 65002, respectively. The presence of these routes in the IP routing tables confirms that the eBGP sessions are established and operational.

The following commands are provided for reference and optional verification. To keep the lab concise, command output is omitted.

BGP Adjacencies

show ip bgp summary

show bgp ipv6 unicast summary

BGP Tables

show bgp ipv4 unicast

show bgp ipv6 unicast

These commands allow you to confirm the state of the eBGP sessions and view the BGP routing information exchanged with the external autonomous systems.

### Examine BGP adjacencies for iBGP on R1, R2, and R3.

Examine the BGP adjacencies on R1. Router R1 has an established eBGP adjacency with D1 in AS 65001 for both IPv4 and IPv6. The BGP summary output confirms that the sessions are stable and operational. R1 is receiving two IPv4 prefixes and one IPv6 prefix from its eBGP neighbor, which correspond to the networks originated by D1. This confirms that R1 is successfully learning external routes from AS 65001 and installing them into its BGP table.

R1 currently has no iBGP adjacencies because iBGP neighbor adjacencies have not yet been configured at this stage of this lab.

R1# show ip bgp summary | begin Neighbor

Neighbor        V      AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

209.165.200.6   4   65001     158     155        3    0    0 02:19:08        2

R1# show bgp ipv6 unicast summary | begin Neighbor

Neighbor        V      AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

2001:DB8:209:4::6

4   65001     159     156        2    0    0 02:19:05        1

R1#

Examine the BGP adjacencies on R3 to see similar output.

R3# show ip bgp summary | begin Neighbor

Neighbor        V      AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

209.165.200.10  4   65002     162     161        3    0    0 02:23:45        2

R3# show bgp ipv6 unicast summary | begin Neighbor

Neighbor        V      AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

2001:DB8:209:8::10

4   65002     163     161        2    0    0 02:23:45        1

Router R2 is internal to AS 65000 and is intended to participate in iBGP only. Because no iBGP neighbor adjacencies have been configured at this stage of the lab, R2 has no BGP adjacencies for either IPv4 or IPv6, and therefore no BGP routing information is exchanged or displayed.

R2# show ip bgp summary

R2#

R2# show bgp ipv6 unicast summary

R2#

## Configure R2 as a BGP Route Reflector

At this point in the lab, a full-mesh iBGP topology could be configured between routers R1, R2, and R3, requiring iBGP neighbor adjacencies between R1–R2, R2–R3, and R1–R3. While this approach works in small networks, it does not scale well as the number of routers increases. A full mesh requires n(n−1)/2 iBGP neighbor relationships, which increases configuration complexity, operational overhead, and the likelihood of configuration errors.

To address these scalability challenges, BGP route reflectors can be used. In this lab, R2 will be configured as a route reflector server, while R1 and R3 will be configured as route reflector clients. Each route reflector client establishes a single iBGP adjacency with the route reflector. The route reflector then reflects eligible BGP routes between its clients, allowing routes learned from one client to be advertised to other clients without requiring a full-mesh of iBGP sessions.

This changes how BGP routes are distributed but does not change how packets are forwarded. Once routes are learned via BGP, packet forwarding still relies on the underlying IGP (OSPFv3 in this lab) to resolve next hops and provide end-to-end connectivity between routers. Packet forwarding continues to rely on the IGP (e.g. OSPFv3) and next-hop reachability.

This design significantly reduces the number of iBGP neighbor configurations, simplifies troubleshooting, and improves scalability, while preserving correct BGP behavior within the autonomous system. Route reflection is therefore a common and essential technique in larger enterprise and service provider networks.

Note: While BGP route reflectors greatly improve scalability, they can also introduce a single point of failure if only one route reflector is deployed. To mitigate this risk in production networks, it is common to deploy multiple route reflectors and configure each client to peer with two or more route reflectors. This design maintains route visibility and resiliency while preserving the scalability benefits of route reflection

### Configure R2 as the route reflector.

R2 will function as the route reflector for AS 65000. It will maintain iBGP sessions with R1 and R3 and reflect eligible BGP routes between them.

Cluster IDs are used to prevent routing loops between route reflectors. They are especially important when multiple route reflectors are deployed. Although only one route reflector is used in this lab, the cluster ID is included for completeness and realism.

R2(config)# router bgp 65000

R2(config-router)# bgp cluster-id 999

R2(config-router)# neighbor 10.0.0.2 remote-as 65000

R2(config-router)# neighbor 10.0.0.6 remote-as 65000

R2(config-router)# !

R2(config-router)# neighbor 2001:DB8:FEED:1::2 remote-as 65000

R2(config-router)# neighbor 2001:DB8:FEED:2::6 remote-as 65000

R2(config-router)# !

R2(config-router)# address-family ipv4

R2(config-router-af)#  neighbor 10.0.0.2 activate

R2(config-router-af)#  neighbor 10.0.0.6 activate

R2(config-router-af)#  neighbor 10.0.0.2 route-reflector-client

R2(config-router-af)#  neighbor 10.0.0.6 route-reflector-client

R2(config-router-af)# exit-address-family

R2(config-router)# !

R2(config-router)# address-family ipv6

R2(config-router-af)#  neighbor 2001:db8:feed:1::2 activate

R2(config-router-af)#  neighbor 2001:db8:feed:2::6 activate

R2(config-router-af)#  neighbor 2001:db8:feed:1::2 route-reflector-client

R2(config-router-af)#  neighbor 2001:db8:fedd:2::6 route-reflector-client

R2(config-router-af)# exit-address-family

R2(config-router)# end

These commands are identical to standard iBGP neighbor configurations, with the exception of the route-reflector-client statements:

neighbor 10.0.0.2 route-reflector-client

neighbor 10.0.0.6 route-reflector-client

neighbor 2001:db8:feed:1::2 route-reflector-client

neighbor 2001:db8:feed:2::6 route-reflector-client

The statements tell R2 that these neighbors are clients, so R2 is allowed to reflect (re-advertise) routes learned from one client to the other client(s). This behavior overrides the iBGP split-horizon rule, allowing the route reflector to advertise iBGP-learned routes between its clients and eliminating the need for a full-mesh iBGP configuration.

Router R2, acting as the iBGP route reflector, is responsible for advertising the aggregated IPv4 and IPv6 prefixes for AS 65000 into BGP. Because these prefixes are originated on R2, any BGP client that establishes an iBGP adjacency with R2 will receive these route updates. This ensures that all internal BGP routers have consistent visibility of the autonomous system’s aggregated prefixes without requiring a full-mesh iBGP configuration.

### Configure R1 as a route reflector client.

R1 is configured as a route reflector client by establishing iBGP adjacencies with R2 for both IPv4 and IPv6. Because R1 learns routes via eBGP from AS 65001, the next-hop-self command is used so that routes reflected within AS 65000 have a reachable next hop.

From the route reflector client’s perspective, the BGP configuration is identical to a normal iBGP configuration. The only difference is that the client forms iBGP adjacencies only with the route reflector, rather than with every other iBGP router in the autonomous system.

R1(config)# router bgp 65000

R1(config-router)# neighbor 10.0.0.1 remote-as 65000

R1(config-router)# neighbor 2001:db8:feed:1::1 remote-as 65000

R1(config-router)# !

R1(config-router)# address-family ipv4

R1(config-router-af)# neighbor 10.0.0.1 activate

R1(config-router-af)# neighbor 10.0.0.1 next-hop-self

R1(config-router-af)# exit-address-family

R1(config-router)# !

R1(config-router)# address-family ipv6

R1(config-router-af)# neighbor 2001:db8:feed:1::1 activate

R1(config-router-af)# neighbor 2001:db8:feed:1::1 next-hop-self

R1(config-router-af)# exit-address-family

R1(config-router)# end

### Configure R3 as a route reflector client.

R3 is configured in the same manner as R1.

R3(config)# router bgp 65000

R3(config-router)# neighbor 10.0.0.5 remote-as 65000

R3(config-router)# neighbor 2001:db8:feed:2::5 remote-as 65000

R3(config-router)# !

R3(config-router)# address-family ipv4

R3(config-router-af)# neighbor 10.0.0.5 activate

R3(config-router-af)# neighbor 10.0.0.5 next-hop-self

R3(config-router-af)# exit-address-family

R3(config-router)# !

R3(config-router)# address-family ipv6

R3(config-router-af)# neighbor 2001:db8:feed:2::5 activate

R3(config-router-af)# neighbor 2001:db8:feed:2::5 next-hop-self

R3(config-router-af)# exit-address-family

R3(config-router)# end

## Verify BGP Adjacencies and Tables

At this point, R1 and R3 have established iBGP adjacencies with the route reflector R2. With route reflection enabled, BGP updates learned by any router within AS 65000 are now shared throughout the autonomous system without requiring a full-mesh iBGP configuration. In this section, you will verify that the iBGP adjacencies are established and that IPv4 and IPv6 BGP routes are visible on R1, R2, and R3.

A route reflector client may form iBGP adjacencies with one or more route reflectors, and it may also form iBGP adjacencies with non-route reflector routers if required by the network design. Similarly, a route reflector may form iBGP adjacencies with both route reflector clients and non-client routers, depending on the network design. Route reflection reduces the need for a full-mesh iBGP topology, but it does not prohibit additional iBGP adjacencies when they are operationally or architecturally necessary.

### Verify R1's iBGP adjacency with router reflector R2.

On R1, the only iBGP adjacency is with the route reflector R2, through which R1 receives all internal BGP route updates for AS 65000. The neighbor 10.0.0.1 (AS 65000) represents R2, and the PfxRcd value indicates that R1 is now receiving three IPv4 prefixes via iBGP from the route reflector. This confirms that R2 is reflecting BGP routes learned within AS 65000 to R1.

Similarly, the IPv6 BGP summary shows an established iBGP adjacency with R2 at 2001:DB8:FEED:1::1 (AS 65000), with two IPv6 prefixes received. This verifies that IPv6 BGP route reflection is also functioning correctly.

R1# show ip bgp summary | begin Neighbor

Neighbor      V    AS MsgRcvd MsgSent   TblVer  InQ OutQ  Up/DownState/PfxRcd

10.0.0.1      4  65000     14      13        6    0    0   00:08:03        3

209.165.200.6 4  65001    383     382        6    0    0   05:44:20        2

R1# show bgp ipv6 unicast summary | begin Neighbor

Neighbor      V    AS MsgRcvd MsgSent   TblVer  InQ OutQ  Up/DownState/PfxRcd

2001:DB8:209:4::6

4  65001     385     383        4    0    0 05:44:17        1

2001:DB8:FEED:1::1

4  65000      14      13        4    0    0 00:07:57        2

### Verify R2's iBGP adjacency with router reflector R2.

The BGP summary output on R3 shows similar behavior to R1.

R3# show ip bgp summary | begin Neighbor

Neighbor      V    AS MsgRcvd MsgSent   TblVer  InQ OutQ  Up/DownState/PfxRcd

10.0.0.5       4  65000    20      16        6    0    0     00:10:29      3

209.165.200.10 4  65002   389     390        6    0    0     05:50:11      2

R3# show bgp ipv6 unicast summary | begin Neighbor

Neighbor      V    AS MsgRcvd MsgSent   TblVer  InQ OutQ  Up/DownState/PfxRcd

2001:DB8:209:8::10

4  65002     391     389        4    0    0 05:50:11        1

2001:DB8:FEED:2::5

4  65000      20      16        4    0    0 00:10:27        2

### Verify route reflector R2's iBGP adjacencies with clients R1 and R3.

The BGP summary output on R2, acting as the route reflector, shows established iBGP adjacencies with both route reflector clients, R1 and R3. R2 is receiving BGP prefixes from each client for both IPv4 and IPv6, confirming that the route reflector is successfully exchanging routing information with all clients. This verifies that R2 is positioned to reflect BGP routes between R1 and R3 within AS 65000. R2 functions solely as an internal BGP router within AS 65000 and has no eBGP neighbor relationships.

R2# show ip bgp summary | begin Neighbor

Neighbor     V    AS MsgRcvd MsgSent   TblVer  InQ OutQ  Up/DownState/PfxRcd

10.0.0.2     4  65000     16      17        6    0    0     00:10:31      2

10.0.0.6     4  65000     16      20        6    0    0     00:10:20      2

R2# show bgp ipv6 unicast summary | begin Neighbor

Neighbor      V    AS MsgRcvd MsgSent   TblVer  InQ OutQ  Up/DownState/PfxRcd

2001:DB8:FEED:1::2

4  65000      16      17        4    0    0 00:10:25        1

2001:DB8:FEED:2::6

4  65000      16      20        4    0    0 00:10:18        1

### Verify Routers as route reflector clients on R2.

The show ip bgp neighbor command on R2 explicitly identifies R1 and R3 as route reflector clients, as indicated by the “Route-Reflector Client” field in the neighbor output.

R2# show ip bgp neighbor 10.0.0.2

BGP neighbor is 10.0.0.2,  remote AS 65000, internal link

BGP version 4, remote router ID 209.165.200.5

BGP state = Established, up for 01:30:25

Last read 00:00:54, last write 00:00:26, hold time is 180, keepalive interval is 60 seconds

Last update received: 01:30:25

Neighbor sessions:

1 active, is not multisession capable (disabled)

Neighbor capabilities:

Route refresh: advertised and received(new)

Four-octets ASN Capability: advertised and received

Address family IPv4 Unicast: advertised and received

Enhanced Refresh Capability: advertised and received

Multisession Capability:

Stateful switchover support enabled: NO for session 1

Message statistics:

InQ depth is 0

OutQ depth is 0

Sent       Rcvd

Opens:                  1          1

Notifications:          0          0

Updates:                3          2

Keepalives:           100        101

Route Refresh:          0          0

Total:                104        104

Do log neighbor state changes (via global configuration)

Default minimum time between advertisement runs is 0 seconds

For address family: IPv4 Unicast

Session: 10.0.0.2

BGP table version 6, neighbor version 6/0

Output queue size : 0

Index 1, Advertise bit 0

Route-Reflector Client

1 update-group member

<output omitted>

R2# show ip bgp neighbor 10.0.0.6 | include Route-Reflector

Route-Reflector Client

## Verify IP Routing Tables

In this part, you will examine the IP routing tables on each router to validate that the route reflector configuration is working as expected. The routing tables should now reflect consistent IPv4 and IPv6 reachability across AS 65000, confirming that BGP routes are being successfully shared through the route reflector

### Examine the IP routing tables on route reflector client R1.

The BGP routing tables on R1 now show all BGP-originated routes from AS 65000, AS 65001, and AS 65002, confirming that the route reflector configuration is working correctly. Routes learned from AS 65002 are installed with R3 as the next hop, indicating that R1 is receiving these routes via iBGP reflection through R2 and that next-hop reachability has been preserved.

R1# show ip route bgp | begin Gateway

Gateway of last resort is not set

172.16.0.0/16 is variably subnetted, 3 subnets, 2 masks

B        172.16.0.0/16 [200/0] via 10.0.0.1, 00:08:04

B     192.168.1.0/24 [20/0] via 209.165.200.6, 05:43:28

B     192.168.2.0/24 [20/0] via 209.165.200.6, 05:43:28

B     192.168.10.0/24 [200/0] via 10.0.0.6, 00:07:53

B     192.168.20.0/24 [200/0] via 10.0.0.6, 00:07:53

R1# show ipv6 route bgp | begin 2001

B   2001:DB8:CAB::/48 [20/0], tag 65001

via FE80::D1:1, GigabitEthernet0/0/1

B   2001:DB8:A000::/36 [200/0]

via 2001:DB8:FEED:1::1

B   2001:DB8:ACAD::/48 [200/0], tag 65002

via 2001:DB8:FEED:2::6

### Examine the IP routing tables on route reflector client R3.

Similar to R1, the BGP routing tables on R3 show all BGP-originated routes from AS 65000, AS 65001, and AS 65002, confirming full route visibility within AS 65000.

R3# show ip route bgp | begin Gateway

Gateway of last resort is not set

172.16.0.0/16 is variably subnetted, 3 subnets, 2 masks

B        172.16.0.0/16 [200/0] via 10.0.0.5, 00:10:30

B     192.168.1.0/24 [200/0] via 10.0.0.2, 00:10:30

B     192.168.2.0/24 [200/0] via 10.0.0.2, 00:10:30

B     192.168.10.0/24 [20/0] via 209.165.200.10, 05:49:21

B     192.168.20.0/24 [20/0] via 209.165.200.10, 05:49:21

R3# show ipv6 route bgp | begin 2001

B   2001:DB8:CAB::/48 [200/0], tag 65001

via 2001:DB8:FEED:1::2

B   2001:DB8:A000::/36 [200/0]

via 2001:DB8:FEED:2::5

B   2001:DB8:ACAD::/48 [20/0], tag 65002

via FE80::D2:1, GigabitEthernet0/0/1

### Examine the IP routing tables on route reflector R2.

The BGP routing tables on R2, acting as the iBGP-only route reflector, show BGP routes learned from both R1 and R3. As the route reflector, R2 uses this information to reflect BGP routes between R1 and R3, providing full BGP route visibility within AS 65000.

R2# show ip route bgp | begin Gateway

Gateway of last resort is not set

B     192.168.1.0/24 [200/0] via 10.0.0.2, 00:10:32

B     192.168.2.0/24 [200/0] via 10.0.0.2, 00:10:32

B     192.168.10.0/24 [200/0] via 10.0.0.6, 00:10:21

B     192.168.20.0/24 [200/0] via 10.0.0.6, 00:10:21

R2# show ipv6 route bgp | begin 2001

B   2001:DB8:CAB::/48 [200/0], tag 65001

via 2001:DB8:FEED:1::2

B   2001:DB8:ACAD::/48 [200/0], tag 65002

via 2001:DB8:FEED:2::6

### Examine the IP routing tables on edge routers D1 and D2.

The BGP routing tables on D1 and D2 show that they are now receiving routes from both AS 65001 and AS 65002 via AS 65000, confirming that the internal BGP configuration in AS 65000 is functioning correctly. The presence of these routes indicates that AS 65000 is operating as a transit autonomous system, successfully advertising learned BGP routes between external autonomous systems.

D1# show ip route bgp | begin Gateway

Gateway of last resort is not set

B     172.16.0.0/16 [20/0] via 209.165.200.5, 00:10:52

B     192.168.10.0/24 [20/0] via 209.165.200.5, 00:10:22

B     192.168.20.0/24 [20/0] via 209.165.200.5, 00:10:22

D1# show ipv6 route bgp | begin 2001

B   2001:DB8:A000::/36 [20/0], tag 65000

via FE80::1:2, GigabitEthernet1/0/11

B   2001:DB8:ACAD::/48 [20/0], tag 65000

via FE80::1:2, GigabitEthernet1/0/11

D2# show ip route bgp | begin Gateway

Gateway of last resort is not set

B     172.16.0.0/16 [20/0] via 209.165.200.9, 00:10:51

B     192.168.1.0/24 [20/0] via 209.165.200.9, 00:10:51

B     192.168.2.0/24 [20/0] via 209.165.200.9, 00:10:51

D2# show ipv6 route bgp | begin 2001

B   2001:DB8:CAB::/48 [20/0], tag 65000

via FE80::3:2, GigabitEthernet1/0/11

B   2001:DB8:A000::/36 [20/0], tag 65000

via FE80::3:2, GigabitEthernet1/0/11

of document

# Device Configs - Final

# Router R1

R1# show run brief

Building configuration...

Current configuration : 2629 bytes

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R1

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

no ip domain lookup

login on-success log

ipv6 unicast-routing

subscriber templating

multilink bundle-name authenticated

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

hash sha512

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

diagnostic bootup level minimal

license udi pid ISR4331/K9 sn FDO2218007Z

memory free low-watermark processor 61443

spanning-tree extend system-id

redundancy

mode none

interface GigabitEthernet0/0/0

ip address 10.0.0.2 255.255.255.252

negotiation auto

ipv6 address FE80::1:1 link-local

ipv6 address 2001:DB8:FEED:1::2/64

ospfv3 network point-to-point

ospfv3 1 ipv4 area 0

ospfv3 1 ipv6 area 0

interface GigabitEthernet0/0/1

ip address 209.165.200.5 255.255.255.252

negotiation auto

ipv6 address FE80::1:2 link-local

ipv6 address 2001:DB8:209:4::5/64

interface GigabitEthernet0/0/2

no ip address

negotiation auto

interface Serial0/1/0

no ip address

interface Serial0/1/1

no ip address

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

negotiation auto

router ospfv3 1

address-family ipv4 unicast

router-id 1.1.1.1

exit-address-family

address-family ipv6 unicast

router-id 1.1.1.1

exit-address-family

router bgp 65000

bgp log-neighbor-changes

neighbor 10.0.0.1 remote-as 65000

neighbor 2001:DB8:209:4::6 remote-as 65001

neighbor 2001:DB8:FEED:1::1 remote-as 65000

neighbor 209.165.200.6 remote-as 65001

address-family ipv4

neighbor 10.0.0.1 activate

neighbor 10.0.0.1 next-hop-self

no neighbor 2001:DB8:209:4::6 activate

no neighbor 2001:DB8:FEED:1::1 activate

neighbor 209.165.200.6 activate

exit-address-family

address-family ipv6

neighbor 2001:DB8:209:4::6 activate

neighbor 2001:DB8:FEED:1::1 activate

neighbor 2001:DB8:FEED:1::1 next-hop-self

exit-address-family

ip forward-protocol nd

no ip http server

ip http secure-server

ip ssh bulk-mode 131072

control-plane

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line aux 0

line vty 0 4

login

transport input ssh

line vty 5 14

login

transport input ssh

end

R1#

# Router R2

R2# show run

Building configuration...

Current configuration : 3412 bytes

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R2

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

no ip domain lookup

login on-success log

ipv6 unicast-routing

subscriber templating

multilink bundle-name authenticated

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

hash sha512

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

diagnostic bootup level minimal

license udi pid ISR4321/K9 sn FDO20410713

memory free low-watermark processor 61443

spanning-tree extend system-id

redundancy

mode none

interface Loopback0

ip address 172.16.1.1 255.255.255.0

ipv6 enable

ospfv3 network point-to-point

ospfv3 1 ipv4 area 0

interface Loopback1

ip address 172.16.2.1 255.255.255.0

ipv6 enable

ospfv3 network point-to-point

ospfv3 1 ipv4 area 0

interface Loopback2

no ip address

ipv6 address 2001:DB8:A001:1::1/64

ospfv3 network point-to-point

ospfv3 1 ipv6 area 0

interface Loopback3

no ip address

ipv6 address 2001:DB8:A002:1::1/64

ospfv3 network point-to-point

ospfv3 1 ipv6 area 0

interface GigabitEthernet0/0/0

ip address 10.0.0.1 255.255.255.252

negotiation auto

ipv6 address FE80::2:1 link-local

ipv6 address 2001:DB8:FEED:1::1/64

ospfv3 network point-to-point

ospfv3 1 ipv4 area 0

ospfv3 1 ipv6 area 0

interface GigabitEthernet0/0/1

ip address 10.0.0.5 255.255.255.252

negotiation auto

ipv6 address FE80::2:2 link-local

ipv6 address 2001:DB8:FEED:2::5/64

ospfv3 network point-to-point

ospfv3 1 ipv4 area 0

ospfv3 1 ipv6 area 0

interface Serial0/1/0

no ip address

interface Serial0/1/1

no ip address

interface Serial0/2/0

no ip address

interface Serial0/2/1

no ip address

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

negotiation auto

router ospfv3 1

address-family ipv4 unicast

router-id 2.2.2.2

exit-address-family

address-family ipv6 unicast

router-id 2.2.2.2

exit-address-family

router bgp 65000

bgp cluster-id 999

bgp log-neighbor-changes

neighbor 10.0.0.2 remote-as 65000

neighbor 10.0.0.6 remote-as 65000

neighbor 2001:DB8:FEED:1::2 remote-as 65000

neighbor 2001:DB8:FEED:2::6 remote-as 65000

address-family ipv4

network 172.16.0.0

neighbor 10.0.0.2 activate

neighbor 10.0.0.2 route-reflector-client

neighbor 10.0.0.6 activate

neighbor 10.0.0.6 route-reflector-client

no neighbor 2001:DB8:FEED:1::2 activate

no neighbor 2001:DB8:FEED:2::6 activate

exit-address-family

address-family ipv6

network 2001:DB8:A000::/36

neighbor 2001:DB8:FEED:1::2 activate

neighbor 2001:DB8:FEED:1::2 route-reflector-client

neighbor 2001:DB8:FEED:2::6 activate

exit-address-family

ip forward-protocol nd

no ip http server

ip http secure-server

ip route 172.16.0.0 255.255.0.0 Null0

ip ssh bulk-mode 131072

ipv6 route 2001:DB8:A000::/36 Null0

control-plane

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line aux 0

line vty 0 4

login

transport input ssh

line vty 5 14

login

transport input ssh

end

R2#

# Router R3

R3# show run brief

Building configuration...

Current configuration : 3169 bytes

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R3

boot-start-marker

boot-end-marker

no aaa new-model

no ip domain lookup

login on-success log

ipv6 unicast-routing

subscriber templating

multilink bundle-name authenticated

crypto pki trustpoint TP-self-signed-3903873913

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-3903873913

revocation-check none

rsakeypair TP-self-signed-3903873913

hash sha512

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

hash sha512

crypto pki certificate chain TP-self-signed-3903873913

certificate self-signed 01

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

diagnostic bootup level minimal

license udi pid ISR4221/K9 sn FGL23313183

memory free low-watermark processor 61616

spanning-tree extend system-id

enable secret 9 $9$N18TC/LwqoCoGU$ssssH5FBUysDQMnxvkYsfuIeHjw.2YpMV4M/AHAH4vk

redundancy

mode none

interface GigabitEthernet0/0/0

description To GW2 (R2)

ip address 10.0.0.6 255.255.255.252

negotiation auto

ipv6 address FE80::3:1 link-local

ipv6 address 2001:DB8:ACAD:A002::2/64

ipv6 address 2001:DB8:FEED:2::6/64

ospfv3 network point-to-point

ospfv3 1 ipv6 area 0

ospfv3 1 ipv4 area 0

interface GigabitEthernet0/0/1

description To ISP3 (D2)

ip address 209.165.200.9 255.255.255.252

negotiation auto

ipv6 address FE80::3:2 link-local

ipv6 address 2001:DB8:209:8::9/64

ipv6 address 2001:DB8:ACAD:2001::1/64

ospfv3 network point-to-point

ospfv3 123 ipv6 area 2

ospfv3 123 ipv4 area 2

interface Serial0/1/0

no ip address

shutdown

interface Serial0/1/1

no ip address

shutdown

router ospfv3 1

address-family ipv4 unicast

router-id 3.3.3.3

exit-address-family

address-family ipv6 unicast

router-id 3.3.3.3

exit-address-family

router ospfv3 123

address-family ipv4 unicast

router-id 3.3.3.1

exit-address-family

address-family ipv6 unicast

router-id 3.3.3.1

exit-address-family

router bgp 65000

bgp log-neighbor-changes

neighbor 10.0.0.5 remote-as 65000

neighbor 2001:DB8:209:8::10 remote-as 65002

neighbor 2001:DB8:FEED:2::5 remote-as 65000

neighbor 209.165.200.10 remote-as 65002

address-family ipv4

neighbor 10.0.0.5 activate

neighbor 10.0.0.5 next-hop-self

no neighbor 2001:DB8:209:8::10 activate

no neighbor 2001:DB8:FEED:2::5 activate

neighbor 209.165.200.10 activate

exit-address-family

address-family ipv6

neighbor 2001:DB8:209:8::10 activate

neighbor 2001:DB8:FEED:2::5 activate

neighbor 2001:DB8:FEED:2::5 next-hop-self

exit-address-family

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip ssh bulk-mode 131072

control-plane

line con 0

exec-timeout 0 0

logging synchronous

transport input none

stopbits 1

line aux 0

line vty 0 4

login

transport input ssh

line vty 5 14

login

transport input ssh

end

R3#

# Switch D1

D1# show run brief

Building configuration...

Current configuration : 5343 bytes

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform punt-keepalive disable-kernel-core

hostname D1

vrf definition Mgmt-vrf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

switch 1 provision c9300l-24t-4g

ip routing

no ip domain lookup

login on-success log

ipv6 nd cache expire refresh

ipv6 unicast-routing

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

hash sha512

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

license boot level network-advantage addon dna-advantage

memory free low-watermark processor 130178

diagnostic bootup level minimal

spanning-tree mode rapid-pvst

spanning-tree extend system-id

redundancy

mode sso

crypto engine compliance shield disable

transceiver type all

monitoring

class-map match-any system-cpp-police-ewlc-control

description EWLC Control

class-map match-any system-cpp-police-topology-control

description Topology control

class-map match-any system-cpp-police-sw-forward

description Sw forwarding, L2 LVX data packets, LOGGING, Transit Traffic

class-map match-any system-cpp-default

description EWLC Data, Inter FED Traffic

class-map match-any system-cpp-police-sys-data

description Openflow, Exception, EGR Exception, NFL Sampled Data, RPF Failed

class-map match-any system-cpp-police-punt-webauth

description Punt Webauth

class-map match-any system-cpp-police-l2lvx-control

description L2 LVX control packets

class-map match-any system-cpp-police-forus

description Forus Address resolution and Forus traffic

class-map match-any system-cpp-police-multicast-end-station

description MCAST END STATION

class-map match-any system-cpp-police-high-rate-app

description High Rate Applications

class-map match-any system-cpp-police-multicast

description MCAST Data

class-map match-any system-cpp-police-l2-control

description L2 control

class-map match-any system-cpp-police-dot1x-auth

description DOT1X Auth

class-map match-any system-cpp-police-data

description ICMP redirect, ICMP_GEN and BROADCAST

class-map match-any system-cpp-police-stackwise-virt-control

description Stackwise Virtual OOB

class-map match-any non-client-nrt-class

class-map match-any system-cpp-police-routing-control

description Routing control and Low Latency

class-map match-any system-cpp-police-protocol-snooping

description Protocol snooping

class-map match-any system-cpp-police-dhcp-snooping

description DHCP snooping

class-map match-any system-cpp-police-ios-routing

description L2 control, Topology control, Routing control, Low Latency

class-map match-any system-cpp-police-system-critical

description System Critical and Gold Pkt

class-map match-any system-cpp-police-ios-feature

description ICMPGEN,BROADCAST,ICMP,L2LVXCntrl,ProtoSnoop,PuntWebauth,MCASTData,Transit,DOT1XAuth,Swfwd,LOGGING,L2LVXData,ForusTraffic,ForusARP,McastEndStn,Openflow,Exception,EGRExcption,NflSampled,RpfFailed

policy-map system-cpp-policy

interface Loopback0

ip address 192.168.1.1 255.255.255.0

interface Loopback1

ip address 192.168.2.1 255.255.255.0

interface Loopback2

no ip address

ipv6 address 2001:DB8:CAB::1/48

interface GigabitEthernet0/0

vrf forwarding Mgmt-vrf

no ip address

negotiation auto

interface GigabitEthernet1/0/1

interface GigabitEthernet1/0/2

interface GigabitEthernet1/0/3

interface GigabitEthernet1/0/4

interface GigabitEthernet1/0/5

interface GigabitEthernet1/0/6

interface GigabitEthernet1/0/7

interface GigabitEthernet1/0/8

interface GigabitEthernet1/0/9

interface GigabitEthernet1/0/10

interface GigabitEthernet1/0/11

no switchport

ip address 209.165.200.6 255.255.255.252

ipv6 address FE80::D1:1 link-local

ipv6 address 2001:DB8:209:4::6/64

interface GigabitEthernet1/0/12

interface GigabitEthernet1/0/13

interface GigabitEthernet1/0/14

interface GigabitEthernet1/0/15

interface GigabitEthernet1/0/16

interface GigabitEthernet1/0/17

interface GigabitEthernet1/0/18

interface GigabitEthernet1/0/19

interface GigabitEthernet1/0/20

interface GigabitEthernet1/0/21

interface GigabitEthernet1/0/22

interface GigabitEthernet1/0/23

interface GigabitEthernet1/0/24

interface GigabitEthernet1/1/1

interface GigabitEthernet1/1/2

interface GigabitEthernet1/1/3

interface GigabitEthernet1/1/4

interface AppGigabitEthernet1/0/1

interface Vlan1

no ip address

router bgp 65001

bgp log-neighbor-changes

neighbor 2001:DB8:209:4::5 remote-as 65000

neighbor 209.165.200.5 remote-as 65000

address-family ipv4

network 192.168.1.0

network 192.168.2.0

no neighbor 2001:DB8:209:4::5 activate

neighbor 209.165.200.5 activate

exit-address-family

address-family ipv6

network 2001:DB8:CAB::/48

neighbor 2001:DB8:209:4::5 activate

exit-address-family

ip forward-protocol nd

ip http server

ip http secure-server

ip ssh bulk-mode 131072

control-plane

service-policy input system-cpp-policy

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line vty 0 4

login

transport input ssh

line vty 5 31

login

transport input ssh

end

D1#

# Switch D2

D2# show run brief

Building configuration...

Current configuration : 5351 bytes

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform punt-keepalive disable-kernel-core

hostname D2

vrf definition Mgmt-vrf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

switch 1 provision c9300l-24t-4g

ip routing

no ip domain lookup

login on-success log

ipv6 nd cache expire refresh

ipv6 unicast-routing

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

hash sha512

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

license boot level network-advantage addon dna-advantage

memory free low-watermark processor 130178

diagnostic bootup level minimal

spanning-tree mode rapid-pvst

spanning-tree extend system-id

redundancy

mode sso

crypto engine compliance shield disable

transceiver type all

monitoring

class-map match-any system-cpp-police-ewlc-control

description EWLC Control

class-map match-any system-cpp-police-topology-control

description Topology control

class-map match-any system-cpp-police-sw-forward

description Sw forwarding, L2 LVX data packets, LOGGING, Transit Traffic

class-map match-any system-cpp-default

description EWLC Data, Inter FED Traffic

class-map match-any system-cpp-police-sys-data

description Openflow, Exception, EGR Exception, NFL Sampled Data, RPF Failed

class-map match-any system-cpp-police-punt-webauth

description Punt Webauth

class-map match-any system-cpp-police-l2lvx-control

description L2 LVX control packets

class-map match-any system-cpp-police-forus

description Forus Address resolution and Forus traffic

class-map match-any system-cpp-police-multicast-end-station

description MCAST END STATION

class-map match-any system-cpp-police-high-rate-app

description High Rate Applications

class-map match-any system-cpp-police-multicast

description MCAST Data

class-map match-any system-cpp-police-l2-control

description L2 control

class-map match-any system-cpp-police-dot1x-auth

description DOT1X Auth

class-map match-any system-cpp-police-data

description ICMP redirect, ICMP_GEN and BROADCAST

class-map match-any system-cpp-police-stackwise-virt-control

description Stackwise Virtual OOB

class-map match-any non-client-nrt-class

class-map match-any system-cpp-police-routing-control

description Routing control and Low Latency

class-map match-any system-cpp-police-protocol-snooping

description Protocol snooping

class-map match-any system-cpp-police-dhcp-snooping

description DHCP snooping

class-map match-any system-cpp-police-ios-routing

description L2 control, Topology control, Routing control, Low Latency

class-map match-any system-cpp-police-system-critical

description System Critical and Gold Pkt

class-map match-any system-cpp-police-ios-feature

description ICMPGEN,BROADCAST,ICMP,L2LVXCntrl,ProtoSnoop,PuntWebauth,MCASTData,Transit,DOT1XAuth,Swfwd,LOGGING,L2LVXData,ForusTraffic,ForusARP,McastEndStn,Openflow,Exception,EGRExcption,NflSampled,RpfFailed

policy-map system-cpp-policy

interface Loopback0

ip address 192.168.10.1 255.255.255.0

interface Loopback1

ip address 192.168.20.1 255.255.255.0

interface Loopback2

no ip address

ipv6 address 2001:DB8:ACAD::1/48

interface GigabitEthernet0/0

vrf forwarding Mgmt-vrf

no ip address

negotiation auto

interface GigabitEthernet1/0/1

interface GigabitEthernet1/0/2

interface GigabitEthernet1/0/3

interface GigabitEthernet1/0/4

interface GigabitEthernet1/0/5

interface GigabitEthernet1/0/6

interface GigabitEthernet1/0/7

interface GigabitEthernet1/0/8

interface GigabitEthernet1/0/9

interface GigabitEthernet1/0/10

interface GigabitEthernet1/0/11

no switchport

ip address 209.165.200.10 255.255.255.252

ipv6 address FE80::D2:1 link-local

ipv6 address 2001:DB8:209:8::10/64

interface GigabitEthernet1/0/12

interface GigabitEthernet1/0/13

interface GigabitEthernet1/0/14

interface GigabitEthernet1/0/15

interface GigabitEthernet1/0/16

interface GigabitEthernet1/0/17

interface GigabitEthernet1/0/18

interface GigabitEthernet1/0/19

interface GigabitEthernet1/0/20

interface GigabitEthernet1/0/21

interface GigabitEthernet1/0/22

interface GigabitEthernet1/0/23

interface GigabitEthernet1/0/24

interface GigabitEthernet1/1/1

interface GigabitEthernet1/1/2

interface GigabitEthernet1/1/3

interface GigabitEthernet1/1/4

interface AppGigabitEthernet1/0/1

interface Vlan1

no ip address

router bgp 65002

bgp log-neighbor-changes

neighbor 2001:DB8:209:8::9 remote-as 65000

neighbor 209.165.200.9 remote-as 65000

address-family ipv4

network 192.168.10.0

network 192.168.20.0

no neighbor 2001:DB8:209:8::9 activate

neighbor 209.165.200.9 activate

exit-address-family

address-family ipv6

network 2001:DB8:ACAD::/48

neighbor 2001:DB8:209:8::9 activate

exit-address-family

ip forward-protocol nd

ip http server

ip http secure-server

ip ssh bulk-mode 131072

control-plane

service-policy input system-cpp-policy

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line vty 0 4

login

transport input ssh

line vty 5 31

login

transport input ssh

end

D2#