# Lab – Implement eBGP and iBGP (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

# Topology

# Addressing Table

| Device | Interface | IPv4 Address | IPv6 Address | IPv6 Link-Local |
| --- | --- | --- | --- | --- |
| R1 | G0/0/0 | 10.0.0.2/30 | 2001:db8:feed:1::2/64 | fe80::1:1 |
| R1 | G0/0/1 | 209.165.200.5/30 | 2001:db8:209:4::5/64 | fe80::1:2 |
| R2 | G0/0/0 | 10.0.0.1/30 | 2001:db8:feed:1::1/64 | fe80::2:1 |
|  | G0/0/1 | 10.0.0.5/30 | 2001:db8:feed:2::5/64 | fe80::2:2 |
|  | Lo0 | 172.16.1.1/24 | N/A | N/A |
|  | Lo1 | 172.16.2.1/24 | N/A | N/A |
| R2 | Lo2 | N/A | 2001:db8:a001:1::1/64 | EUI-64 |
| R2 | Lo3 | N/A | 2001:db8:a002:1::1/64 | EUI-64 |
| R3 R3 | G0/0/0 | 10.0.0.6/30 | 2001:db8:feed:2::6/64 | fe80::3:1 |
|  | G0/0/1 | 209.165.200.9/30 | 2001:db8:209:8::9/64 | fe80::3:2 |
| D1 D1 | G1/0/11 | 209.165.200.6/30 | 2001:db8:209:4::6/64 | fe80::d1:1 |
|  | Lo0 | 192.168.1.1/24 | N/A | N/A |
|  | Lo1 | 192.168.2.1/24 | N/A | N/A |
|  | Lo2 | N/A | 2001:db8:cab::1/48 | EUI-64 |
| D2 D2 | G1/0/11 | 209.165.200.10/30 | 2001:db8:209:8::10/64 | fe80::d2:1 |
|  | N/A | 192.168.10.1/24 | N/A | N/A |
|  | N/A | 192.168.20.1/24 | N/A | N/A |
|  | N/A | N/A | 2001:db8:acad::1/48 | EUI-64 |

# Objectives

Part 1: Build the Topology and Configure Basic Device Settings and IP Addressing

Part 2: Configure eBGP Edge Routers

Part 3: Configure iBGP on AS 65000 Routers

Part 4: Verify iBGP and eBGP Reachability

Part 5: Configure AS 65000 as a Non-Transit AS

# Background / Scenario

In this lab, AS 65000 uses OSPFv3 as its Interior Gateway Protocol (IGP) to provide internal reachability for both IPv4 and IPv6. OSPFv3 is configured with separate address families, allowing a single IGP to support dual-stack operation. Loopback interfaces are used to represent internal IPv4 and IPv6 prefixes within AS 65000; these prefixes simulate customer or internal networks and are later advertised into BGP.

AS 65000 runs BGP using address families (AFs) for both IPv4 and IPv6, enabling a unified BGP process to handle dual-stack routing. External autonomous systems AS 65001 and AS 65002 use traditional BGP for IPv4 and the IPv6 unicast address family for IPv6. Because IPv6 routing in BGP is supported only through address families, all IPv6 BGP configuration and route exchange in this lab occurs within the IPv6 AF context.

This design reflects modern dual-stack enterprise and service provider networks, where an IGP such as OSPFv3 provides internal reachability while BGP, using address families, controls inter-AS routing and policy.

Note: Although the BGP standards (e.g., RFC 4271) define the abbreviations as EBGP and IBGP, modern vendor documentation often uses the lower-case forms eBGP and iBGP to visually distinguish “external” or “internal” from the core BGP protocol name. We will use lower case e and i to make it easier to visually distinguish these two protocols.

Note: The routers used with this CCNP hands-on lab is a Cisco 8200L and the two Layer 3 switches are Catalyst 9300L switches using Cisco IOS XE Release 17.12.05 (universalk9 image). Other routers and Layer 3 switches and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs.

Note: Ensure that the routers and switches have been erased and have no startup configurations. If you are unsure contact your instructor.

Instructor Note: Refer to the Instructor Lab Manual for the procedures to initialize and reload devices.

# Required Resources

3 Routers (Cisco Catalyst 8200L with Cisco IOS XE Release 17.12.05c universal image or comparable)

2 Switches (Cisco Catalyst 9300L with Cisco IOS XE Release 17.12.05 universal image or comparable)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

# Instructions

## Build the Network and Configure Basic Device Settings and Interface Addressing

In Part 1, you will set up the network topology and configure basic settings and interface addressing on routers and switches.

### Cable the network as shown in the topology.

Attach the devices as shown in the topology diagram, and cable as necessary.

### Configure basic settings for each router.

Console into each device, enter global configuration mode, and apply the basic settings and interface addressing using the following startup configurations for each device.

Open configuration window

Router R1

hostname R1

no ip domain lookup

line con 0

exec-timeout 0 0

logging sync

ipv6 unicast-routing

router ospfv3 1

address-family ipv4 unicast

router-id 1.1.1.1

exit-address-family

address-family ipv6 unicast

router-id 1.1.1.1

exit-address-family

interface GigabitEthernet0/0/0

ip address 10.0.0.2 255.255.255.252

ipv6 address 2001:db8:feed:1::2/64

ipv6 address fe80::1:1 link-local

ospfv3 network point-to-point

ospfv3 1 ipv6 area 0

ospfv3 1 ipv4 area 0

no shutdown

exit

interface GigabitEthernet0/0/1

ip address 209.165.200.5 255.255.255.252

ipv6 address 2001:db8:209:4::5/64

ipv6 address fe80::1:2 link-local

no shutdown

exit

end

Router R2

hostname R2

no ip domain lookup

line con 0

exec-timeout 0 0

logging sync

ipv6 unicast-routing

router ospfv3 1

address-family ipv4 unicast

router-id 2.2.2.2

exit-address-family

address-family ipv6 unicast

router-id 2.2.2.2

exit-address-family

interface GigabitEthernet0/0/0

ip address 10.0.0.1 255.255.255.252

ipv6 address 2001:db8:feed:1::1/64

ipv6 address fe80::2:1 link-local

ospfv3 network point-to-point

ospfv3 1 ipv6 area 0

ospfv3 1 ipv4 area 0

no shutdown

exit

interface GigabitEthernet0/0/1

ip address 10.0.0.5 255.255.255.252

ipv6 address 2001:db8:feed:2::5/64

ipv6 address fe80::2:2 link-local

ospfv3 network point-to-point

ospfv3 1 ipv6 area 0

ospfv3 1 ipv4 area 0

no shutdown

exit

interface loopback0

ip address 172.16.1.1 255.255.255.0

ipv6 enable

ospfv3 1 ipv4 area 0

ospfv3 network point-to-point

no shutdown

exit

interface loopback1

ip address 172.16.2.1 255.255.255.0

ipv6 enable

ospfv3 1 ipv4 area 0

ospfv3 network point-to-point

no shutdown

exit

interface loopback2

ipv6 address 2001:db8:a001:1::1/64

ospfv3 1 ipv6 area 0

ospfv3 network point-to-point

no shutdown

exit

interface loopback3

ipv6 address 2001:db8:a002:1::1/64

ospfv3 1 ipv6 area 0

ospfv3 network point-to-point

no shutdown

exit

end

Router R3

hostname R3

no ip domain lookup

line con 0

exec-timeout 0 0

logging sync

ip routing

ipv6 unicast-routing

router ospfv3 1

address-family ipv4 unicast

router-id 3.3.3.3

exit-address-family

address-family ipv6 unicast

router-id 3.3.3.3

exit-address-family

interface GigabitEthernet0/0/0

ip address 10.0.0.6 255.255.255.252

ipv6 address 2001:db8:feed:2::6/64

ipv6 address fe80::3:1 link-local

ospfv3 network point-to-point

ospfv3 1 ipv6 area 0

ospfv3 1 ipv4 area 0

no shutdown

exit

interface GigabitEthernet0/0/1

ip address 209.165.200.9 255.255.255.252

ipv6 address 2001:db8:209:8::9/64

ipv6 address fe80::3:2 link-local

no shutdown

exit

end

Switch D1

hostname D1

no ip domain lookup

line con 0

exec-timeout 0 0

logging sync

ip routing

ipv6 unicast-routing

interface GigabitEthernet1/0/11

no switchport

ip address 209.165.200.6 255.255.255.252

ipv6 address 2001:db8:209:4::6/64

ipv6 address fe80::d1:1 link-local

no shutdown

exit

interface loopback0

ip address 192.168.1.1 255.255.255.0

ipv6 enable

no shutdown

exit

interface loopback1

ip address 192.168.2.1 255.255.255.0

ipv6 enable

no shutdown

exit

interface loopback2

ipv6 address 2001:db8:cab::1/48

no shutdown

exit

end

Switch D2

hostname D2

no ip domain lookup

line con 0

exec-timeout 0 0

logging sync

ip routing

ipv6 unicast-routing

interface GigabitEthernet1/0/11

no switchport

ip address 209.165.200.10 255.255.255.252

ipv6 address 2001:db8:209:8::10/64

ipv6 address fe80::d2:1 link-local

no shutdown

exit

interface loopback0

ip address 192.168.10.1 255.255.255.0

ipv6 enable

no shutdown

exit

interface loopback1

ip address 192.168.20.1 255.255.255.0

ipv6 enable

no shutdown

exit

interface loopback2

ipv6 address 2001:db8:acad::1/48

no shutdown

exit

end

Save the running configuration to startup-config.

Close configuration window

## Configure eBGP on Edge Routers

In this part, you will configure eBGP sessions between R1 and D1, and between R3 and D2. These eBGP adjacencies represent connections between different autonomous systems and allow each edge router in AS 65000 to learn routes originated by its external neighbor. At this stage, only eBGP is configured. No internal BGP (iBGP) sessions exist within AS 65000.

As a result, each edge router will learn routes from its directly connected external AS, but those routes will not be shared across AS 65000. This behavior highlights the fundamental role of eBGP and sets the foundation for understanding why iBGP is required to carry routes within an autonomous system.

Note: The edge routers (R1 and R3) are used only to establish eBGP connectivity and do not originate any BGP routes. Prefix origination and aggregation for AS 65000 will be configured later on R2, an iBGP router, allowing routing policy to be centralized.

### Configure and verify eBGP on R1 to peer with D1.

Before configuring BGP, confirm that the interface addressing is correct and you can reach the neighbor using the following commands.

R1# show ip interface brief | include up

GigabitEthernet0/0/0   10.0.0.2        YES manual up                    up

GigabitEthernet0/0/1   209.165.200.5   YES manual up                    up

R1# show ipv6 interface brief

GigabitEthernet0/0/0   [up/up]

FE80::1:1

2001:DB8:FEED:1::2

GigabitEthernet0/0/1   [up/up]

FE80::1:2

2001:DB8:209:4::5

<output omitted>

R1# ping 209.165.200.6

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 209.165.200.6, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/1 ms

R1# ping 2001:db8:209:4::6

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 2001:DB8:209:4::6, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/1 ms

Enter BGP process for AS 65000. The bgp log-neighbor-changes command enables syslog messages when BGP neighbor adjacencies change state. It is not enabled by default but is commonly configured to aid in troubleshooting.

R1(config)# router bgp 65000

R1(config-router)# bgp log-neighbor-changes

Configure the IPv4 and IPv6 eBGP neighbor (global neighbor definition). This defines the neighbor and its remote AS for both address families.

R1(config-router)# neighbor 209.165.200.6 remote-as 65001

R1(config-router)# neighbor 2001:db8:209:4::6 remote-as 65001

Note: In this lab, some BGP neighbor parameters (such as remote-as) are configured in router BGP configuration mode, while route exchange is controlled by activating the neighbor under each address family. This is a common and valid approach. Alternatively, all neighbor configurations, including remote-as can be done entirely within the address-family itself. Both methods are supported by IOS/IOS-XE and result in the same operational behavior.

Activate the IPv4 neighbor under the IPv4 unicast address family. In BGP with address families, the neighbor is defined first, then activated under the appropriate address family to exchange routes. With AF-based BGP, the neighbor must be activated in the AF to exchange routes.

R1(config-router)# address-family ipv4 unicast

R1(config-router-af)# neighbor 209.165.200.6 activate

R1(config-router-af)# exit-address-family

Activate the IPv6 neighbor under the IPv6 unicast address family.

R1(config-router)# address-family ipv6 unicast

R1(config-router-af)# neighbor 2001:db8:209:4::6 activate

R1(config-router-af)# exit-address-family

R1(config-router)# end

Verify the BGP IPv4 session status. At this point, the BGP session is not established because the remote peer has not yet been configured; BGP sessions require configuration on both routers before adjacencies can form. You’ll likely see the neighbor listed with State = Idle/Active, or no neighbor at all.

R1# show ip bgp summary

BGP router identifier 209.165.200.5, local AS number 65000

BGP table version is 1, main routing table version 1

Neighbor        V    AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

209.165.200.6   4 65001       0       0        1    0    0 00:04:33 Idle

Verify the BGP IPv6 session status. This is the same as BGP IPv4. The BGP session is not established yet because the remote peer has not been configured.

R1# show bgp ipv6 unicast summary

BGP router identifier 209.165.200.5, local AS number 65000

BGP table version is 1, main routing table version 1

Neighbor        V           AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

2001:DB8:209:4::6

4        65001       0       0        1    0    0 never    Idle

### Configure and verify eBGP on D1 to peer with R1.

Enable the BGP process on D1 for AS 65001 and enable syslog messages when BGP neighbor adjacencies change state.

D1(config)# router bgp 65001

D1(config-router)# bgp log-neighbor-changes

Configure the IPv4 and IPv6 eBGP neighbor. This defines the neighbor and its remote AS for both address families. Notice the message stating IPv4 adjacency is established with R1. IPv6 adjacency is established later after activating R1 as an IPv6 neighbor.

D1(config-router)# neighbor 209.165.200.5 remote-as 65000

%BGP-5-ADJCHANGE: neighbor 209.165.200.5 Up

D1(config-router)# neighbor 2001:db8:209:4::5 remote-as 65000

Originate the IPv4 prefixes with network statements. These prefixes must exist in D1’s routing table (they are looback addresses on D1).

D1(config-router)# network 192.168.1.0 mask 255.255.255.0

D1(config-router)# network 192.168.2.0 mask 255.255.255.0

Configure the IPv6 eBGP neighbor under the IPv6 address family. IPv6 BGP requires the IPv6 address family. The activate parameter is required to establish IPv6 adjacency with R1 and to exchange IPv6 routes. The IPv6 prefix is originated from D1 using the network command.

D1(config-router)# address-family ipv6 unicast

D1(config-router-af)# neighbor 2001:db8:209:4::5 activate

*Jan 20 18:43:37.068: %BGP-5-ADJCHANGE: neighbor 2001:DB8:209:4::5 Up

D1(config-router-af)# network 2001:db8:cab::/48

D1(config-router-af)# exit-address-family

D1(config-router)# end

The following output verifies the status of the BGP session between D1 and R1. In the State/PfxRcd column, a numeric value indicates that the BGP session is established, with the number representing how many prefixes have been received from the neighbor rather than showing the word Established. Currently, R1 is not advertising any prefixes.

D1# show ip bgp summary

BGP router identifier 192.168.2.1, local AS number 65001

BGP table version is 3, main routing table version 3

2 network entries using 496 bytes of memory

2 path entries using 272 bytes of memory

1/1 BGP path/bestpath attribute entries using 296 bytes of memory

0 BGP route-map cache entries using 0 bytes of memory

0 BGP filter-list cache entries using 0 bytes of memory

BGP using 1064 total bytes of memory

BGP activity 3/0 prefixes, 3/0 paths, scan interval 60 secs

2 networks peaked at 10:29:19 Jan 15 2026 UTC (00:10:42.513 ago)

Neighbor        V      AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

209.165.200.5   4   65000      17      18        3    0    0 00:12:16        0

The following command displays detailed information about the BGP neighbor relationship and explicitly confirms that the session is in the Established state.

D1# show ip bgp neighbors 209.165.200.5

BGP neighbor is 209.165.200.5,  remote AS 65000, external link

BGP version 4, remote router ID 209.165.200.5

BGP state = Established, up for 00:15:52

Last read 00:00:01, last write 00:00:17, hold time is 180, keepalive interval is 60 seconds

<output omitted>

The output confirms that R1 has an established eBGP session with D1 (AS 65001). The value 2 in the State/PfxRcd column indicates that R1 has successfully received two IPv4 prefixes from its neighbor.

R1# show ip bgp summary

BGP router identifier 209.165.200.5, local AS number 65000

<output omitted>

2 networks peaked at 18:47:49 Jan 20 2026 UTC (00:06:11.363 ago)

Neighbor        V      AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

209.165.200.6   4   65001      37      36        3    0    0 00:29:41        2

The following command displays the IPv4 BGP routing table on R1, where you can see the two IPv4 prefixes learned from D1 via eBGP. Both routes are marked as valid and best paths, with D1 shown as the next hop.

R1# show bgp ipv4 unicast

BGP table version is 3, local router ID is 209.165.200.5

Status codes: s suppressed, d damped, h history, * valid, > best, i - internal,

r RIB-failure, S Stale, m multipath, b backup-path, f RT-Filter,

x best-external, a additional-path, c RIB-compressed,

t secondary path, L long-lived-stale,

Origin codes: i - IGP, e - EGP, ? - incomplete

RPKI validation codes: V valid, I invalid, N Not found

Network          Next Hop            Metric LocPrf Weight Path

*>   192.168.1.0      209.165.200.6            0             0 65001 i

*>   192.168.2.0      209.165.200.6            0             0 65001 i

This output confirms that R1 has an established IPv6 eBGP session with D1 (AS 65001). The value 1 in the State/PfxRcd column indicates that R1 has received one IPv6 prefix from its neighbor.

R1# show bgp ipv6 unicast summary

BGP router identifier 209.165.200.5, local AS number 65000

<output omitted>

1 networks peaked at 18:48:08 Jan 20 2026 UTC (00:09:41.215 ago)

Neighbor        V      AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

2001:DB8:209:4::6

4   65001      38      38        2    0    0 00:30:33        1

The following command displays the IPv6 BGP routing table on R1, where you can see the IPv6 prefix learned from D1 via eBGP. The route is marked as valid and best, with D1 shown as the next hop.

R1# show bgp ipv6 unicast

BGP table version is 2, local router ID is 209.165.200.5

Status codes: s suppressed, d damped, h history, * valid, > best, i - internal,

r RIB-failure, S Stale, m multipath, b backup-path, f RT-Filter,

x best-external, a additional-path, c RIB-compressed,

t secondary path, L long-lived-stale,

Origin codes: i - IGP, e - EGP, ? - incomplete

RPKI validation codes: V valid, I invalid, N Not found

Network          Next Hop            Metric LocPrf Weight Path

*>   2001:DB8:CAB::/48

2001:DB8:209:4::6

0             0 65001 i

This output verifies that R1 has installed the IPv4 prefixes learned from D1 into its routing table, with both routes marked as BGP-learned and reachable via the eBGP neighbor.

R1# show ip route bgp | include B

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

B     192.168.1.0/24 [20/0] via 209.165.200.6, 01:53:02

B     192.168.2.0/24 [20/0] via 209.165.200.6, 01:52:32

The IPv6 routing table on R1 shows that the IPv6 prefix advertised by D1 has been successfully installed as a BGP route.

R1# show ipv6 route bgp

IPv6 Routing Table - default - 9 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, H - NHRP, HG - NHRP registered

<output omitted>

B   2001:DB8:CAB::/48 [20/0], tag 65001

via FE80::D1:1, GigabitEthernet0/0/1

### Configure and verify eBGP between R3 and D2.

In this step, you will configure the eBGP peering between R3 and D2. Because this configuration closely mirrors the eBGP setup between R1 and D1, the full set of BGP commands are shown for each router without repeating the same step-by-step explanation.

Before BGP, confirm the interface addressing is correct and you can reach the neighbor using the following commands on R3 and D2:

show ip interface brief

show ipv6 interface brief

ping <ipv4-address-of-peer>

ping ipv6 <ipv6-address-of-peer>

Configure eBGP on R3.

R3(config)# router bgp 65000

R3(config-router)# bgp log-neighbor-changes

R3(config-router)# neighbor 209.165.200.10 remote-as 65002

R3(config-router)# neighbor 2001:db8:209:8::10 remote-as 65002

R3(config-router)# address-family ipv4 unicast

R3(config-router-af)# neighbor 209.165.200.10 activate

R3(config-router-af)# exit-address-family

R3(config-router)# address-family ipv6 unicast

R3(config-router-af)# neighbor 2001:db8:209:8::10 activate

R3(config-router-af)# exit-address-family

R3(config-router)# end

Configure eBGP on D2.

D2(config)# router bgp 65002

D2(config-router)# bgp log-neighbor-changes

D2(config-router)# neighbor 209.165.200.9 remote-as 65000

D2(config-router)# neighbor 2001:db8:209:8::9 remote-as 65000

D2(config-router)# network 192.168.10.0 mask 255.255.255.0

D2(config-router)# network 192.168.20.0 mask 255.255.255.0

D2(config-router)# address-family ipv6 unicast

D2(config-router-af)# neighbor 2001:db8:209:8::9 activate

D2(config-router-af)# network 2001:db8:acad::/48

D2(config-router-af)# exit-address-family

D2(config-router)# end

D2#

Verify R3 has an established eBGP session with D2 (AS 65002).

R3# show ip bgp summary

BGP router identifier 209.165.200.9, local AS number 65000

<output omitted>

2 networks peaked at 16:57:12 Jan 20 2026 UTC (00:12:28.559 ago)

Neighbor        V      AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

209.165.200.10  4   65002      18      18        3    0    0 00:12:28        2

Verify the IPv4 BGP routing table on R3, where you can see the two IPv4 prefixes learned from D2 via eBGP.

R3# show bgp ipv4 unicast

BGP table version is 3, local router ID is 209.165.200.9

Status codes: s suppressed, d damped, h history, * valid, > best, i - internal,

r RIB-failure, S Stale, m multipath, b backup-path, f RT-Filter,

x best-external, a additional-path, c RIB-compressed,

t secondary path, L long-lived-stale,

Origin codes: i - IGP, e - EGP, ? - incomplete

RPKI validation codes: V valid, I invalid, N Not found

Network          Next Hop            Metric LocPrf Weight Path

*>   192.168.10.0     209.165.200.10           0             0 65002 i

*>   192.168.20.0     209.165.200.10           0             0 65002 i

Verify R3 has an established IPv6 eBGP session with D2 (AS 65002).

R3# show bgp ipv6 unicast summary

BGP router identifier 209.165.200.9, local AS number 65000

<output omitted>

1 networks peaked at 16:57:14 Jan 15 2026 UTC (00:12:26.606 ago)

Neighbor        V      AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

2001:DB8:209:8::10

4   65002      18      17        2    0    0 00:12:26        1

Verify the IPv6 BGP routing table on R3, where you can see the IPv6 prefix learned from D2 via eBGP.

R3# show bgp ipv6 unicast

BGP table version is 2, local router ID is 209.165.200.9

Status codes: s suppressed, d damped, h history, * valid, > best, i - internal,

r RIB-failure, S Stale, m multipath, b backup-path, f RT-Filter,

x best-external, a additional-path, c RIB-compressed,

t secondary path, L long-lived-stale,

Origin codes: i - IGP, e - EGP, ? - incomplete

RPKI validation codes: V valid, I invalid, N Not found

Network          Next Hop            Metric LocPrf Weight Path

*>   2001:DB8:ACAD::/48

2001:DB8:209:8::10

0             0 65002 i

Verify that R3 has installed the IPv4 prefixes learned from D2 into its routing table.

R3# show ip route bgp | include B

B     192.168.10.0/24 [20/0] via 209.165.200.10, 00:40:37

B     192.168.20.0/24 [20/0] via 209.165.200.10, 00:40:37

The IPv6 routing table on R3 shows that the IPv6 prefix advertised by D2 has been successfully installed as a BGP route.

R3# show ipv6 route bgp

IPv6 Routing Table - default - 9 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, H - NHRP, HG - NHRP registered

<output omitted>

B   2001:DB8:ACAD::/48 [20/0], tag 65002

via FE80::D2:1, GigabitEthernet0/0/1

## Configure iBGP on AS 65000 Routers

So far, each edge router (R1 and R3) in AS 65000 has learned routes only from its directly connected external neighbor. R1 has not learned any prefixes from AS 65002 and R3 has not learned any prefixes from AS 65001. R2 has not yet been configured for BGP and is not aware of any prefixes outside of AS 65000.

In this part, you will configure full-mesh iBGP between R1, R2, and R3 within AS 65000 so these prefixes (routes) can be exchanged internally between these routers. You will also configure R2 to originate and advertise aggregated IPv4 and IPv6 prefixes, demonstrating how iBGP enables route distribution and centralized policy control within an autonomous system.

### Configure iBGP on Router R1

Before configuring iBGP, there should be full IPv4 and IPv6 reachability between all routers in AS 65000 via OSPFv3, as iBGP relies on the IGP to provide next-hop reachability. Verify IGP reachability to the iBGP peers using the following commands.

R1# show ip route ospfv3

R1# show ipv6 route ospf

R1# show ip route 10.0.0.1

R1# show ip route 10.0.0.6

R1# ping 10.0.0.1

R1# ping 10.0.0.6

R1# show ipv6 route 2001:db8:feed:1::1

R1# show ipv6 route 2001:db8:feed:2::6

R1# ping ipv6 2001:db8:feed:1::1

R1# ping ipv6 2001:db8:feed:2::6

Enter the BGP process for AS 65000.

R1(config)# router bgp 65000

Configure the IPv4 iBGP neighbors (R2 and R3). These commands define the iBGP peers using their IPv4 interface addresses.

R1(config-router)# neighbor 10.0.0.1 remote-as 65000

R1(config-router)# neighbor 10.0.0.6 remote-as 65000

Activate the IPv4 neighbors (R2 and R3) under the IPv4 unicast address family. With address families, neighbors must be activated under the appropriate AF to exchange routes.

R1(config-router)# address-family ipv4 unicast

R1(config-router-af)# neighbor 10.0.0.1 activate

R1(config-router-af)# neighbor 10.0.0.6 activate

Configure next-hop-self for iBGP neighbors. R1 is an edge router that learns routes via eBGP from D1. Setting next-hop-self ensures that routes R1 advertises into iBGP use a next hop reachable inside AS 65000. This prevents iBGP peers from receiving routes with an unreachable external next hop.

R1(config-router-af)# neighbor 10.0.0.1 next-hop-self

R1(config-router-af)# neighbor 10.0.0.6 next-hop-self

R1(config-router-af)# exit-address-family

By using next-hop-self, R1 sets itself as the next hop for routes advertised into iBGP. Because all routers in AS 65000 have full IPv4 and IPv6 reachability through OSPFv3, these next-hop addresses are guaranteed to be reachable by the iBGP peers.

Configure the IPv6 iBGP neighbors (R2 and R3) and activate them under the IPv6 address family. IPv6 BGP is configured under the IPv6 unicast address family. As with the IPv4 address family, within the IPv6 AF neighbors must be activated to exchange routes.

R1(config-router)# neighbor 2001:db8:feed:1::1 remote-as 65000

R1(config-router)# neighbor 2001:db8:feed:2::6 remote-as 65000

R1(config-router)# address-family ipv6 unicast

R1(config-router-af)# neighbor 2001:db8:feed:1::1 activate

R1(config-router-af)# neighbor 2001:db8:feed:1::1 next-hop-self

R1(config-router-af)# neighbor 2001:db8:feed:2::6 activate

R1(config-router-af)# neighbor 2001:db8:feed:2::6 next-hop-self

R1(config-router-af)# exit-address-family

R1(config-router)# end

### Configure iBGP on Router R3

Similar to what you did on R1, you can verify IGP reachability on R3 to the iBGP peers using the following commands.

R3# show ip route ospfv3

R3# show ipv6 route ospf

R3# show ip route 10.0.0.5

R3# show ip route 10.0.0.2

R3# ping 10.0.0.5

R3# ping 10.0.0.2

R3# show ipv6 route 2001:db8:feed:1::5

R3# show ipv6 route 2001:db8:feed:2::2

R3# ping ipv6 2001:db8:feed:1::5

R3# ping ipv6 2001:db8:feed:2::2

The iBGP configuration on R3 mirrors the configuration on R1 with the primary difference being the neighbor IP addresses used to peer with the other routers in AS 65000. As with R1, R3 establishes iBGP sessions with R2 and the remaining edge router and uses next-hop-self to ensure that routes learned via eBGP are advertised with a next hop reachable through OSPFv3. You should see messages confirming IPv4 and IPv6 adjacency with R1.

R3(config)# router bgp 65000

R3(config-router)# neighbor 10.0.0.5 remote-as 65000

R3(config-router)# neighbor 10.0.0.2 remote-as 65000

R3(config-router)# address-family ipv4 unicast

R3(config-router-af)# neighbor 10.0.0.5 activate

R3(config-router-af)# neighbor 10.0.0.2 activate

R3(config-router-af)# neighbor 10.0.0.5 next-hop-self

R3(config-router-af)# neighbor 10.0.0.2 next-hop-self

R3(config-router-af)# exit-address-family

R3(config-router)# neighbor 2001:db8:feed:2::5 remote-as 65000

R3(config-router)# neighbor 2001:db8:feed:1::2 remote-as 65000

R3(config-router)# address-family ipv6 unicast

R3(config-router-af)# neighbor 2001:db8:feed:2::5 activate

R3(config-router-af)# neighbor 2001:db8:feed:2::5 next-hop-self

R3(config-router-af)# neighbor 2001:db8:feed:1::2 activate

R3(config-router-af)# neighbor 2001:db8:feed:1::2 next-hop-self

R3(config-router-af)# exit-address-family

R3(config-router)# end

Jan 20 22:02:29.657: %BGP-5-ADJCHANGE: neighbor 10.0.0.2 Up

Jan 20 22:02:32.729: %BGP-5-ADJCHANGE: neighbor 2001:DB8:FEED:1::2 Up

Note: If you look at the routing tables, you will notice that R1 has routes to reach AS 65002 and R3 has routes to reach AS 65001. However, because R2 does not yet know about these prefixes, you will not be able to verify connectivity. For example, ping an address in AS 65002 from R1.

R1# ping 192.168.10.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.10.1, timeout is 2 seconds:

U.U.U

Success rate is 0 percent (0/5)

### Configure iBGP on Router R2

In this step, you will configure internal BGP (iBGP) on R2, which serves as an internal router within AS 65000 and does not establish any eBGP sessions. R2 is used as the centralized point to originate the aggregated IPv4 and IPv6 prefixes for AS 65000, demonstrating how route origination and aggregation can be handled internally and then distributed to the edge routers through iBGP.

We also configure full-mesh iBGP within AS 65000 so that all BGP routers in the autonomous system can exchange BGP routing information. Because iBGP routers do not forward BGP updates learned from one iBGP neighbor to another by default, each router must establish an iBGP session with every other BGP router in the AS. By configuring R2 with iBGP sessions to both R1 and R3, AS 65000 will have a complete iBGP mesh, allowing internal distribution of routes learned via eBGP as well as prefixes originated within the AS. If R2 were not configured for iBGP, it would not receive BGP routing information from R1 or R3 and therefore would not be able to forward traffic between them based on BGP-learned routes.

Enter the BGP process and enable neighbor-change logging.

R2(config)# router bgp 65000

R2(config-router)# bgp log-neighbor-changes

Configure the IPv4 iBGP neighbors (R1 and R3). These commands define the iBGP peers using their IPv4 interface addresses:

R2(config-router)# neighbor 10.0.0.2 remote-as 65000

%BGP-5-ADJCHANGE: neighbor 10.0.0.2 Up

R2(config-router)# neighbor 10.0.0.6 remote-as 65000

%BGP-5-ADJCHANGE: neighbor 10.0.0.6 Up

Because we are using AF-style BGP, neighbors must be activated under the address family to exchange routes.

R2(config-router)# address-family ipv4 unicast

R2(config-router-af)# neighbor 10.0.0.2 activate

R2(config-router-af)# neighbor 10.0.0.6 activate

- Note: R2 is not learning routes from eBGP directly (R1/R3 are), so next-hop-self is typically more important on the edge routers when they pass eBGP-learned routes into iBGP. On R2, it’s optional—use it only if you want to “normalize” next-hop behavior for the routes R2 advertises.

Originate the IPv4 aggregate (summary address) and exit the IPv4 AF.

R2(config-router-af)# network 172.16.0.0 mask 255.255.0.0

R2(config-router-af)# exit-address-family

R2(config-router)#

Note: In this lab, R2 is the single origination point for the aggregated prefixes to simplify the configuration and highlight the role of iBGP. The network 172.16.0.0/16 is an aggregation of all subnets such as Loopback0 172.16.1.0/24 and Loopback1 172.16.2.0/24 networks. In production networks, these summaries are often originated on multiple routers to avoid a single point of failure.

When an AS advertises only an aggregate prefix to its external BGP peers, internal routing changes do not trigger external BGP updates, as long as some route within the aggregate remains reachable. ASN 65000 advertises aggregated prefixes so that internal routing changes do not trigger external BGP updates, which reflects real-world design practices used to improve stability and reduce BGP churn on the Internet. Traffic will be forwarded to AS 65000 and dropped if a prefix is not present.

Configure and activate the IPv6 iBGP neighbors (R1 and R3) under the IPv6 address family. IPv6 BGP is configured under the IPv6 unicast address family. As with the IPv4 address family, within the IPv6 AF neighbors must be activated to exchange routes.

R2(config-router)# neighbor 2001:db8:feed:2::6 remote-as 65000

R2(config-router)# neighbor 2001:db8:feed:1::2 remote-as 65000

R2(config-router)# address-family ipv6 unicast

R2(config-router-af)# neighbor 2001:db8:feed:1::2 activate

*Jan 20 22:29:26.319: %BGP-5-ADJCHANGE: neighbor 2001:DB8:FEED:1::2 Up

R2(config-router-af)# neighbor 2001:db8:feed:2::6 activate

*Jan 20 22:29:42.089: %BGP-5-ADJCHANGE: neighbor 2001:DB8:FEED:2::6 Up

Originate the IPv6 aggregate 2001:db8:a000::/36 which is a summary for Loopback2 2001:db8:a001::/48 and Loopback3 2001:db8:a002::/48 networks.

R2(config-router-af)# network 2001:db8:a000::/36

R2(config-router-af)# exit-address-family

R2(config-router)# exit

BGP will only advertise a network statement if an exact prefix (prefix and prefix length) exists in the routing table. To ensure 172.16.0.0/16 and 2001:db8:a000::/36 exists in R2’s IPv4/IPv6 routing tables, static routes are configured using Null0 to prevent traffic from being forwarded if no more-specific route exists.

R2(config)# ip route 172.16.0.0 255.255.0.0 Null0

R2(config)# ipv6 route 2001:db8:a000::/36 Null0

## Verify iBGP and eBGP Reachability

At this point in the lab, external BGP (eBGP) has been successfully configured on all edge routers and internal BGP (iBGP) has been implemented as a full mesh within AS 65000. The internal router R2 is now originating aggregated IPv4 and IPv6 prefixes, which are distributed to the edge routers and advertised externally.

In this part, you will verify BGP adjacencies, examine BGP tables, and confirm that each AS has learned the appropriate BGP routes. This includes verifying that AS 65001 and AS 65002 have reachability to each other’s prefixes through AS 65000, demonstrating that AS 65000 is currently functioning as a transit autonomous system.

### Verify AS 65000 reachability via OSPFv3 on R1, R2, and R3.

The following commands display the IPv4 and IPv6 routes learned via OSPFv3 on R1, verifying that there is full internal reachability within AS 65000, which is required for iBGP next-hop resolution and overall connectivity.

R1# show ip route ospfv3 | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 3 subnets, 2 masks

O        10.0.0.4/30 [110/2] via 10.0.0.1, 09:30:27, GigabitEthernet0/0/0

172.16.0.0/16 is variably subnetted, 3 subnets, 2 masks

O        172.16.1.0/24 [110/2] via 10.0.0.1, 09:30:27, GigabitEthernet0/0/0

O        172.16.2.0/24 [110/2] via 10.0.0.1, 09:30:27, GigabitEthernet0/0/0

R1# show ipv6 route ospf | begin 2001

O   2001:DB8:A001:1::/64 [110/2]

via FE80::2:1, GigabitEthernet0/0/0

O   2001:DB8:A002:1::/64 [110/2]

via FE80::2:1, GigabitEthernet0/0/0

O   2001:DB8:FEED:2::/64 [110/2]

via FE80::2:1, GigabitEthernet0/0/0

Use similar commands on R3 to verify the same reachability for R3 within AS 65000.

R2 does not display any OSPFv3-learned routes because all networks within the OSPFv3 routing domain are directly connected to R2, so there are no remote OSPF routes to install in the routing table.

R2# show ip route ospfv3 | begin Gateway

Gateway of last resort is not set

R2# show ipv6 route ospf

IPv6 Routing Table - default - 12 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, H - NHRP, HG - NHRP registered

Hg - NHRP registration summary, HE - NHRP External, I1 - ISIS L1

I2 - ISIS L2, IA - ISIS interarea, IS - ISIS summary, D - EIGRP

EX - EIGRP external, ND - ND Default, NDp - ND Prefix, DCE - Destination

NDr - Redirect, O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1

OE2 - OSPF ext 2, ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2

a - Application, m - OMP

### Verify eBGP and iBGP adjacencies on AS 65000 edge routers R1 and R3.

The output confirms that R1 maintains its existing eBGP adjacency with D1 (AS 65001 – 209.165.200.6), and now also has iBGP adjacencies with R2 (10.0.0.1) and R3 (10.0.0.6) within AS 65000. With iBGP fully configured, R1 participates in a full-mesh iBGP topology for both IPv4 and IPv6, allowing routes learned via eBGP to be distributed internally across the autonomous system.

R1# show ip bgp summary | begin Neighbor

Neighbor        V           AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down State/PfxRcd

10.0.0.1        4        65000     219     223        6    0    0 03:16:07        1

10.0.0.6        4        65000     253     254        6    0    0 03:46:57        2

209.165.200.6   4        65001     594     593        6    0    0 08:54:48        2

R1#

R1# show bgp ipv6 unicast summary | begin Neighbor

Neighbor        V      AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

2001:DB8:209:4::6

4   65001     589     587        4    0    0 08:49:55        1

2001:DB8:FEED:1::1

4   65000     205     205        4    0    0 03:02:34        1

2001:DB8:FEED:2::6

4   65000     252     254        4    0    0 03:46:53        1

Use the same BGP verification commands on R3 to confirm that it has established eBGP adjacencies with D2 and iBGP adjacencies with R1 and R2 for both IPv4 and IPv6, completing the full-mesh iBGP topology within AS 65000.

### Verify eBGP adjacencies on AS 65001 and AS 65002 routers D1 and D2

The BGP summary output on D1 confirms an established eBGP adjacency with R1 in AS 65000, showing that D1 is successfully exchanging IPv4 and IPv6 routes with AS 65000.

D1# show ip bgp summary | begin Neighbor

Neighbor        V           AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

209.165.200.5   4        65000     595     596        6    0    0 08:57:10        3

D1# show bgp ipv6 unicast summary | begin Neighbor

Neighbor        V           AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

2001:DB8:209:4::5

4        65000     590     591        4    0    0 08:52:17        2

Use the same commands on D2 to confirm it has established eBGP adjacency with R3 in AS 65000, showing that D2 is successfully exchanging IPv4 and IPv6 routes with AS 65000.

### Verify BGP tables on AS 65000 routers R1, R2, and R3.

In this step, we examine the BGP tables on the routers in AS 65000 to verify that routes learned via eBGP and iBGP are present and that next-hop information is being handled correctly. Because AS 65000 is running full-mesh iBGP and OSPFv3 as its IGP, internal next-hop addresses must be reachable through OSPFv3, while external next hops reflect the directly connected eBGP peers. By inspecting the BGP tables, we can see the difference between routes learned via eBGP and those learned via iBGP, as well as how AS 65000 is functioning as a transit AS.

R1# show bgp ipv4 unicast

BGP table version is 6, local router ID is 209.165.200.5

Status codes: s suppressed, d damped, h history, * valid, > best, i - internal,

<output omitted>

Network          Next Hop            Metric LocPrf Weight Path

*>i  172.16.0.0       10.0.0.1                 0    100      0 i

*>   192.168.1.0      209.165.200.6            0             0 65001 i

*>   192.168.2.0      209.165.200.6            0             0 65001 i

*>i  192.168.10.0     10.0.0.6                 0    100      0 65002 i

*>i  192.168.20.0     10.0.0.6                 0    100      0 65002 i

###### Explaining Specific BGP Entries on R1

R1’s routing table shows BGP routes learned from three different sources.

iBGP route for a prefix originated inside AS 65000

*>i  172.16.0.0       10.0.0.1                 0    100      0 i

This route represents the aggregated prefix originated by R2 within AS 65000. The i flag indicates the route was learned via iBGP, and the next hop is 10.0.0.1, which is R2’s internal address. Because OSPFv3 provides full reachability within AS 65000, R1 can reach this next hop through the IGP, allowing the route to be installed and used. Under the Path column in the table, the i without an AS_PATH indicates this prefix originated within AS 65000 and has not traversed any external autonomous systems.

iBGP route for a prefix originating in AS 65002

*>i  192.168.10.0     10.0.0.6                 0    100      0 65002 i

This route was learned via eBGP on R3 from AS 65002 and advertised to R1 using iBGP. The next hop has been rewritten to 10.0.0.6 (R3’s internal address) due to the next-hop-self configuration on the edge router. This ensures that the next hop is reachable inside AS 65000 via OSPFv3. The AS_PATH shows 65002, indicating the route originated in AS 65002 and was learned internally via iBGP after entering AS 65000 through an edge router.

eBGP route learned directly from AS 65001

*>   192.168.1.0      209.165.200.6            0             0 65001 i

This route was learned directly via eBGP from D1 in AS 65001. Because it is an eBGP-learned route, the next hop remains the external neighbor’s IP address (209.165.200.6). No i flag is present, indicating this is not an internal BGP route. The AS_PATH shows 65001, indicating the route originated in AS 65001 and was learned internally via iBGP after entering AS 65000 through an edge router.

Examine the BGP IPv6 table on R1 to view similar information as you saw in the BGP IPv4 table.

R1# show bgp ipv6 unicast | begin Network

Network          Next Hop            Metric LocPrf Weight Path

*>   2001:DB8:CAB::/48

2001:DB8:209:4::6

0             0 65001 i

*>i  2001:DB8:A000::/36

2001:DB8:FEED:1::1

0    100      0 i

*>i  2001:DB8:ACAD::/48

2001:DB8:FEED:2::6

0    100      0 65002 i

The BGP tables on R3 mirror what we observed on R1. Routes originating within AS 65000 and learned via iBGP use internal next hops resolved through OSPFv3, while routes learned directly from AS 65002 via eBGP use the external neighbor as the next hop.

- This confirms that iBGP is distributing internal and external routes consistently across AS 65000, with edge routers preserving eBGP next hops and internal routes using IGP-reachable addresses.

R3# show bgp ipv4 unicast | begin Network

Network          Next Hop            Metric LocPrf Weight Path

*>i  172.16.0.0       10.0.0.5                 0    100      0 i

*>i  192.168.1.0      10.0.0.2                 0    100      0 65001 i

*>i  192.168.2.0      10.0.0.2                 0    100      0 65001 i

*>   192.168.10.0     209.165.200.10           0             0 65002 i

*>   192.168.20.0     209.165.200.10           0             0 65002 i

R3# show bgp ipv6 unicast | begin Network

Network          Next Hop            Metric LocPrf Weight Path

*>i  2001:DB8:CAB::/48

2001:DB8:FEED:1::2

0    100      0 65001 i

*>i  2001:DB8:A000::/36

2001:DB8:FEED:2::5

0    100      0 i

*>   2001:DB8:ACAD::/48

2001:DB8:209:8::10

0             0 65002 i

Examine the BGP table on R2. R2 operates exclusively as an iBGP router and is responsible for originating the aggregated IPv4 and IPv6 prefixes for AS 65000, which are then distributed to the edge routers via iBGP

R2# show bgp ipv4 unicast | begin Network

Network          Next Hop            Metric LocPrf Weight Path

*>   172.16.0.0       0.0.0.0                  0         32768 i

*>i  192.168.1.0      10.0.0.2                 0    100      0 65001 i

*>i  192.168.2.0      10.0.0.2                 0    100      0 65001 i

*>i  192.168.10.0     10.0.0.6                 0    100      0 65002 i

*>i  192.168.20.0     10.0.0.6                 0    100      0 65002 i

R2# show bgp ipv6 unicast | begin Network

Network          Next Hop            Metric LocPrf Weight Path

*>i  2001:DB8:CAB::/48

2001:DB8:FEED:1::2

0    100      0 65001 i

*>   2001:DB8:A000::/36

::                       0         32768 i

*>i  2001:DB8:ACAD::/48

2001:DB8:FEED:2::6

0    100      0 65002 i

On R2, the aggregated prefixes 172.16.0.0/16 (IPv4) and 2001:db8:a000::/36 (IPv6) show a next hop of 0.0.0.0 and :: because these routes are locally originated by R2 using the network command.

In BGP, a next hop of 0.0.0.0 (IPv4) or :: (IPv6) means “this router is the source of the route”, which is also why these entries have the default weight of 32768, a value automatically assigned to locally originated routes.

All other routes in R2’s BGP table are marked as iBGP entries (*>i) and use next hops that are internal AS 65000 addresses (for example, 10.0.0.2 or 10.0.0.6, and their IPv6 equivalents). These next hops point to the edge routers (R1 or R3) that learned the prefixes via eBGP and then advertised them into AS 65000 using iBGP, with reachability provided by the underlying OSPFv3 IGP.

R2 originates and aggregates local prefixes, while edge routers handle eBGP, and iBGP carries reachability across the AS using IGP-resolvable next hops.

### Verify BGP tables on AS 65000 routers R1, R2. and R3.

In a typical network design, only routers that participate in the BGP transit path need to maintain BGP routing information, whether learned via eBGP or iBGP. Other internal routers, running an IGP such as OSPFv3, do not require full BGP routes and instead forward traffic using a default route or summarized routes toward an ABR or ASBR.

Some routers outside the transit forwarding path may still participate in BGP for control-plane reasons, such as acting as route reflectors, originating prefixes using BGP network statements, or applying routing policy, even if they are not directly forwarding transit traffic

In this lab, AS 65000 functions as a transit AS, so R1, R2, and R3 maintain BGP routes to ensure end-to-end reachability between external autonomous systems.

The IP routing tables on R1 show only the best BGP routes selected for forwarding. These entries demonstrate how eBGP and iBGP routes are resolved using next-hop reachability provided by OSPFv3, allowing R1 to forward traffic between external autonomous systems while maintaining aggregated routing information for AS 65000.

R1# show ip route bgp | begin Gateway

Gateway of last resort is not set

172.16.0.0/16 is variably subnetted, 3 subnets, 2 masks

B        172.16.0.0/16 [200/0] via 10.0.0.1, 02:53:12

B     192.168.1.0/24 [20/0] via 209.165.200.6, 08:53:14

B     192.168.2.0/24 [20/0] via 209.165.200.6, 08:52:44

B     192.168.10.0/24 [200/0] via 10.0.0.6, 03:46:57

B     192.168.20.0/24 [200/0] via 10.0.0.6, 03:46:57

R1# show ipv6 route bgp | begin 2001

B   2001:DB8:CAB::/48 [20/0], tag 65001

via FE80::D1:1, GigabitEthernet0/0/1

B   2001:DB8:A000::/36 [200/0]

via 2001:DB8:FEED:1::1

B   2001:DB8:ACAD::/48 [200/0], tag 65002

via 2001:DB8:FEED:2::6

Unlike the edge routers, R2 does not maintain any eBGP adjacencies. Instead, it learns all external prefixes via iBGP from R1 and R3. As a result, all BGP routes in R2’s IPv4 and IPv6 routing tables use internal next-hop addresses within AS 65000, confirming that R2 relies on OSPFv3 for next-hop reachability while participating in BGP for control-plane route distribution.

R2# show ip route bgp | begin Gateway

Gateway of last resort is not set

B     192.168.1.0/24 [200/0] via 10.0.0.2, 03:17:08

B     192.168.2.0/24 [200/0] via 10.0.0.2, 03:17:08

B     192.168.10.0/24 [200/0] via 10.0.0.6, 03:17:01

B     192.168.20.0/24 [200/0] via 10.0.0.6, 03:17:01

R2# show ipv6 route bgp | begin 2001

B   2001:DB8:CAB::/48 [200/0], tag 65001

via 2001:DB8:FEED:1::2

B   2001:DB8:ACAD::/48 [200/0], tag 65002

via 2001:DB8:FEED:2::6

The BGP routing entries on R3 confirm that, like R1, R3 functions as both an edge and transit router, forwarding traffic between external autonomous systems while relying on iBGP and OSPFv3 for internal reachability.

R3# show ip route bgp | begin Gateway

Gateway of last resort is not set

172.16.0.0/16 is variably subnetted, 3 subnets, 2 masks

B        172.16.0.0/16 [200/0] via 10.0.0.5, 02:54:52

B     192.168.1.0/24 [200/0] via 10.0.0.2, 03:48:37

B     192.168.2.0/24 [200/0] via 10.0.0.2, 03:48:37

B     192.168.10.0/24 [20/0] via 209.165.200.10, 07:36:45

B     192.168.20.0/24 [20/0] via 209.165.200.10, 07:36:45

R3# show ipv6 route bgp | begin 2001

B   2001:DB8:CAB::/48 [200/0], tag 65001

via 2001:DB8:FEED:1::2

B   2001:DB8:A000::/36 [200/0]

via 2001:DB8:FEED:2::5

B   2001:DB8:ACAD::/48 [20/0], tag 65002

via FE80::D2:1, GigabitEthernet0/0/1

The full routing tables on R1 show both OSPF-learned specific routes and BGP-learned aggregated routes for the same address space.

Within AS 65000, OSPFv3 advertises the more specific internal prefixes (such as 172.16.1.0/24, 172.16.2.0/24, and their IPv6 equivalents), while BGP advertises a summarized aggregate prefix (172.16.0.0/16 and 2001:DB8:A000::/36) that represents the entire AS. Because OSPF routes have a lower administrative distance than iBGP-learned routes, the router installs the OSPF routes for internal forwarding, while the BGP aggregate remains present to support inter-AS reachability and policy control.

This behavior allows AS 65000 to use detailed IGP routing internally while presenting a simplified, aggregated view of the network to external autonomous systems.

As an edge eBGP router, R1 advertises only BGP routes to its external neighbor, D1. OSPFv3 routes, while used internally for forwarding, are not advertised across the eBGP boundary

R1# show ip route | begin Gateway

Gateway of last resort is not set

<output omitted

172.16.0.0/16 is variably subnetted, 3 subnets, 2 masks

B        172.16.0.0/16 [200/0] via 10.0.0.1, 03:06:17

O        172.16.1.0/24 [110/2] via 10.0.0.1, 03:08:39, GigabitEthernet0/0/0

O        172.16.2.0/24 [110/2] via 10.0.0.1, 03:08:39, GigabitEthernet0/0/0

<output omitted>

R1# show ipv6 route | begin 2001

<output omitted>

B   2001:DB8:A000::/36 [200/0]

via 2001:DB8:FEED:1::1

O   2001:DB8:A001:1::/64 [110/2]

via FE80::2:1, GigabitEthernet0/0/0

O   2001:DB8:A002:1::/64 [110/2]

via FE80::2:1, GigabitEthernet0/0/0

<output omitted>

### Verify BGP tables on AS 65001 and AS 65002 edge routers D1 and D2 .

The BGP tables on D1 show three distinct types of routes.

D1’s locally originated prefixes (192.168.1.0/24, 192.168.2.0/24, and 2001:DB8:CAB::/48) appear with a next hop of 0.0.0.0 (or ::) and an empty AS path, indicating that these routes were originated within AS 65001.

D1 learns the aggregated prefixes from AS 65000 (172.16.0.0/16 and 2001:DB8:A000::/36) via its eBGP neighbor R1, with the AS path showing only 65000 and the next hop pointing to R1.

D1 receives prefixes from AS 65002 via AS 65000, where the AS path 65000 65002 clearly shows that AS 65000 is acting as a transit AS between AS 65001 and AS 65002.

Note: The presence of the AS path 65000 65002 for prefixes originating in AS 65002 confirms that AS 65000 is acting as a transit AS, carrying traffic between two external autonomous systems rather than only serving its own customers.

D1# show bgp ipv4 unicast | begin Network

Network          Next Hop            Metric LocPrf Weight Path

*>   172.16.0.0       209.165.200.5                          0 65000 i

*>   192.168.1.0      0.0.0.0                  0         32768 i

*>   192.168.2.0      0.0.0.0                  0         32768 i

*>   192.168.10.0     209.165.200.5                          0 65000 65002 i

*>   192.168.20.0     209.165.200.5                          0 65000 65002 i

D1# show bgp ipv6 unicast | begin Network

Network          Next Hop            Metric LocPrf Weight Path

*>   2001:DB8:CAB::/48

::                       0         32768 i

*>   2001:DB8:A000::/36

2001:DB8:209:4::5

0 65000 i

*>   2001:DB8:ACAD::/48

2001:DB8:209:4::5

0 65000 65002 i

The BGP tables on D2 show a similar pattern to those on D1.

D2# show bgp ipv4 unicast | begin Network

Network          Next Hop            Metric LocPrf Weight Path

*>   172.16.0.0       209.165.200.9                          0 65000 i

*>   192.168.1.0      209.165.200.9                          0 65000 65001 i

*>   192.168.2.0      209.165.200.9                          0 65000 65001 i

*>   192.168.10.0     0.0.0.0                  0         32768 i

*>   192.168.20.0     0.0.0.0                  0         32768 i

D2# show bgp ipv6 unicast | begin Network

Network          Next Hop            Metric LocPrf Weight Path

*>   2001:DB8:CAB::/48

2001:DB8:209:8::9

0 65000 65001 i

*>   2001:DB8:A000::/36

2001:DB8:209:8::9

0 65000 i

*>   2001:DB8:ACAD::/48

::                       0         32768 i

### Verify BGP table on AS 65001 and AS 65002 edge routers D1 and D2.

The IP routing tables on D1 confirm reachability to external prefixes learned via BGP. D1 installs the aggregated prefixes from AS 65000 as well as prefixes originating in AS 65002, all reachable via its eBGP neighbor, R1. This verifies that traffic destined for other autonomous systems is successfully forwarded through AS 65000, further confirming that AS 65000 is operating as a transit AS for both IPv4 and IPv6 traffic.

D1# show ip route bgp | begin Gateway

Gateway of last resort is not set

B     172.16.0.0/16 [20/0] via 209.165.200.5, 02:55:34

B     192.168.10.0/24 [20/0] via 209.165.200.5, 03:49:20

B     192.168.20.0/24 [20/0] via 209.165.200.5, 03:49:20

D1# show ipv6 route bgp | begin 2001

IPv6 Routing Table - default - 7 entries

B   2001:DB8:A000::/36 [20/0], tag 65000

via FE80::1:2, GigabitEthernet1/0/11

B   2001:DB8:ACAD::/48 [20/0], tag 65000

via FE80::1:2, GigabitEthernet1/0/11

The IP routing tables on D2 show a similar result to those on D1.

D2# show ip route bgp | begin Gateway

Gateway of last resort is not set

B     172.16.0.0/16 [20/0] via 209.165.200.9, 02:56:13

B     192.168.1.0/24 [20/0] via 209.165.200.9, 03:49:58

B     192.168.2.0/24 [20/0] via 209.165.200.9, 03:49:58

D2# show ipv6 route bgp | begin 2001

B   2001:DB8:CAB::/48 [20/0], tag 65000

via FE80::3:2, GigabitEthernet1/0/11

B   2001:DB8:A000::/36 [20/0], tag 65000

via FE80::3:2, GigabitEthernet1/0/11

## Configure AS 65000 as a Non-Transit AS

In BGP, a transit AS provides connectivity between other autonomous systems by advertising routes learned from one AS to another. Up to this point, AS 65000 has been operating as a transit AS. In this part, we will apply a simple BGP policy that prevents AS 65000 from forwarding external routes between AS 65001 and AS 65002, effectively converting it into a non-transit AS.

From the perspective of AS 65001 and AS 65002, the difference between a transit and non-transit AS is reflected in which external prefixes are reachable. When AS 65000 operates as a transit AS, D1 and D2 learn not only AS 65000’s aggregated prefixes but also prefixes originating in the other external AS, allowing end-to-end connectivity between AS 65001 and AS 65002. After AS 65000 is converted to a non-transit AS, D1 and D2 will continue to reach AS 65000’s own prefixes, but routes to the other external AS will no longer be advertised, eliminating inter-AS transit through AS 65000.

### Create an as-path access list on R1.

Configure the AS-path access list on R1.

R1(config)# ip as-path access-list 10 permit ^$

This command creates an AS-path access list, which is used by BGP to match routes based on its AS path attribute.

ip as-path access-list 10

Creates AS-path access list number 10. AS-path access lists are defined in global configuration mode and are later applied to BGP neighbors.

permit

Allows routes that match the specified AS-path pattern.

^$

This is a regular expression that matches an empty AS path:

^ represents the beginning of the AS path

$ represents the end of the AS path

Together, ^$ matches routes that originate locally within the AS, meaning they were generated by AS 65000 itself and have not passed through any other autonomous systems.

This AS-path access list matches only routes originated by AS 65000 and excludes any routes learned from external autonomous systems. When applied to an eBGP neighbor, it ensures that R1 advertises only AS 65000’s own prefixes, which is a key step in preventing AS 65000 from acting as a transit AS.

### Apply the AS-Path Filter to R1’s eBGP Neighbor

Configure the AS-path filter in both BGP Adress Families.

R1(config)# router bgp 65000

R1(config-router)# address-family ipv4 unicast

R1(config-router-af)# neighbor 209.165.200.6 filter-list 10 out

R1(config-router-af)# exit-address-family

R1(config-router)# address-family ipv6 unicast

R1(config-router-af)# neighbor 2001:db8:209:4::6 filter-list 10 out

R1(config-router-af)# exit-address-family

This command applies the AS-path access list 10 to routes advertised outbound to the eBGP neighbor.

neighbor <ip-address> filter-list 10 out

Ensures the filter affects only routes sent to D1, not routes received from it.

Because AS-path access list 10 permits only an empty AS path (^$), R1 will now advertise only routes originated by AS 65000 to D1.

Routes learned via iBGP from AS 65001 or AS 65002 will no longer be advertised to D1.

The configuration is applied separately under both AFs ensuring consistent behavior for both protocol families. This method is used in production networks to prevent unintended transit while preserving customer connectivity.

### Configure non-transit filtering on R3.

The configuration on R3 mirrors the configuration on R1.

R3(config)# ip as-path access-list 10 permit ^$

R3(config)# router bgp 65000

R3(config-router)# address-family ipv4 unicast

R3(config-router-af)# neighbor 209.165.200.10 filter-list 10 out

R3(config-router-af)# exit-address-family

R3(config-router)# address-family ipv6 unicast

R3(config-router-af)# neighbor 2001:db8:209:8::10 filter-list 10 out

R3(config-router-af)# exit-address-family

R3(config-router)# end

### Verify BGP Tables After Disabling Transit in AS 65000

After applying the outbound AS-path filters on R1 and R3, we verify the impact by examining the BGP tables on D1 (AS 65001) and D2 (AS 65002). Notice that D1 no longer has any BGP entries for prefixes originating in AS 65002, confirming that AS 65000 is no longer advertising transit routes.

D1# show bgp ipv4 unicast | begin Network

Network          Next Hop            Metric LocPrf Weight Path

*>   172.16.0.0       209.165.200.5                          0 65000 i

*>   192.168.1.0      0.0.0.0                  0         32768 i

*>   192.168.2.0      0.0.0.0                  0         32768 i

D1# show bgp ipv6 unicast | begin Network

Network          Next Hop            Metric LocPrf Weight Path

*>   2001:DB8:CAB::/48

::                       0         32768 i

*>   2001:DB8:A000::/36

2001:DB8:209:4::5

0 65000 i

You should see similar output for D2.

D2# show bgp ipv4 unicast | begin Network

Network          Next Hop            Metric LocPrf Weight Path

*>   172.16.0.0       209.165.200.9                          0 65000 i

*>   192.168.10.0     0.0.0.0                  0         32768 i

*>   192.168.20.0     0.0.0.0                  0         32768 i

D2# show bgp ipv6 unicast | begin Network

Network          Next Hop            Metric LocPrf Weight Path

*>   2001:DB8:A000::/36

2001:DB8:209:8::9

0 65000 i

*>   2001:DB8:ACAD::/48

::                       0         32768 i

### Verify IP Routing Tables After Disabling Transit in AS 65000

The IP routing tables on D1 confirm that AS 65000 is no longer acting as a transit AS. D1 now installs only the aggregated prefixes originated by AS 65000 in both the IPv4 and IPv6 routing tables. Routes to prefixes originating in AS 65002 are no longer present, demonstrating that traffic destined for other external autonomous systems is no longer forwarded through AS 65000. This verifies that the outbound AS-path filtering on R1 has successfully disabled transit behavior while preserving reachability to AS 65000’s own networks.

D1# show ip route bgp | begin Gateway

Gateway of last resort is not set

B     172.16.0.0/16 [20/0] via 209.165.200.5, 07:08:12

D1# show ipv6 route bgp | begin 2001

B   2001:DB8:A000::/36 [20/0], tag 65000

via FE80::1:2, GigabitEthernet1/0/11

You should see similar output for D2.

D2# show ip route bgp | begin Gateway

Gateway of last resort is not set

B     172.16.0.0/16 [20/0] via 209.165.200.9, 07:11:15

D2# show ipv6 route bgp | begin 2001

B   2001:DB8:A000::/36 [20/0], tag 65000

via FE80::3:2, GigabitEthernet1/0/11

of document

# Device Configs – Final

Note: When neighbors are defined globally, IOS-XE may alter the running configuration by adding no neighbor <address> activate under an address family to explicitly document that a neighbor is not enabled for that address family. This is normal IOS-XE behavior and does not change how BGP operates; it simply makes the per-address-family activation state explicit.

# Router R1

R1# show run brief

Building configuration...

Current configuration : 2931 bytes

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R1

boot-start-marker

boot-end-marker

no aaa new-model

no ip domain lookup

login on-success log

ipv6 unicast-routing

subscriber templating

vtp domain TSHOOT

vtp mode transparent

multilink bundle-name authenticated

pae

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

hash sha512

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

diagnostic bootup level minimal

license udi pid C8200L-1N-4T sn FJC2730263E

memory free low-watermark processor 61443

spanning-tree extend system-id

redundancy

mode none

interface GigabitEthernet0/0/0

ip address 10.0.0.2 255.255.255.252

negotiation auto

ipv6 address FE80::1:1 link-local

ipv6 address 2001:DB8:FEED:1::2/64

ospfv3 network point-to-point

ospfv3 1 ipv4 area 0

ospfv3 1 ipv6 area 0

interface GigabitEthernet0/0/1

ip address 209.165.200.5 255.255.255.252

negotiation auto

ipv6 address FE80::1:2 link-local

ipv6 address 2001:DB8:209:4::5/64

interface GigabitEthernet0/0/2

no ip address

negotiation auto

interface GigabitEthernet0/0/3

no ip address

negotiation auto

interface Serial0/1/0

no ip address

interface Serial0/1/1

no ip address

router ospfv3 1

address-family ipv4 unicast

router-id 1.1.1.1

exit-address-family

address-family ipv6 unicast

router-id 1.1.1.1

exit-address-family

router bgp 65000

bgp log-neighbor-changes

neighbor 10.0.0.1 remote-as 65000

neighbor 10.0.0.6 remote-as 65000

neighbor 2001:DB8:209:4::6 remote-as 65001

neighbor 2001:DB8:FEED:1::1 remote-as 65000

neighbor 2001:DB8:FEED:2::6 remote-as 65000

neighbor 209.165.200.6 remote-as 65001

address-family ipv4

neighbor 10.0.0.1 activate

neighbor 10.0.0.1 next-hop-self

neighbor 10.0.0.6 activate

neighbor 10.0.0.6 next-hop-self

no neighbor 2001:DB8:209:4::6 activate

no neighbor 2001:DB8:FEED:1::1 activate

no neighbor 2001:DB8:FEED:2::6 activate

neighbor 209.165.200.6 activate

neighbor 209.165.200.6 filter-list 10 out

exit-address-family

address-family ipv6

neighbor 2001:DB8:209:4::6 activate

neighbor 2001:DB8:209:4::6 filter-list 10 out

neighbor 2001:DB8:FEED:1::1 activate

neighbor 2001:DB8:FEED:1::1 next-hop-self

neighbor 2001:DB8:FEED:2::6 activate

neighbor 2001:DB8:FEED:2::6 next-hop-self

exit-address-family

ip forward-protocol nd

no ip http server

ip http secure-server

ip as-path access-list 10 permit ^$

ip ssh bulk-mode 131072

control-plane

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line aux 0

line vty 0 4

login

transport input ssh

line vty 5 14

login

transport input ssh

end

R1#

# Router R2

R2# show run

Building configuration...

Current configuration : 3342 bytes

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R2

boot-start-marker

boot-end-marker

no aaa new-model

no ip domain lookup

login on-success log

ipv6 unicast-routing

subscriber templating

multilink bundle-name authenticated

pae

crypto pki trustpoint TP-self-signed-1301900760

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-1301900760

revocation-check none

rsakeypair TP-self-signed-1301900760

hash sha512

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

hash sha512

crypto pki certificate chain TP-self-signed-1301900760

certificate self-signed 01

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

diagnostic bootup level minimal

license udi pid C8200L-1N-4T sn FJC2730264H

memory free low-watermark processor 61443

spanning-tree extend system-id

redundancy

mode none

interface Loopback0

ip address 172.16.1.1 255.255.255.0

ipv6 enable

ospfv3 network point-to-point

ospfv3 1 ipv4 area 0

interface Loopback1

ip address 172.16.2.1 255.255.255.0

ipv6 enable

ospfv3 network point-to-point

ospfv3 1 ipv4 area 0

interface Loopback2

no ip address

ipv6 address 2001:DB8:A001:1::1/64

ospfv3 network point-to-point

ospfv3 1 ipv6 area 0

interface Loopback3

no ip address

ipv6 address 2001:DB8:A002:1::1/64

ospfv3 network point-to-point

ospfv3 1 ipv6 area 0

interface GigabitEthernet0/0/0

ip address 10.0.0.1 255.255.255.252

negotiation auto

ipv6 address FE80::2:1 link-local

ipv6 address 2001:DB8:FEED:1::1/64

ospfv3 network point-to-point

ospfv3 1 ipv4 area 0

ospfv3 1 ipv6 area 0

interface GigabitEthernet0/0/1

ip address 10.0.0.5 255.255.255.252

negotiation auto

ipv6 address FE80::2:2 link-local

ipv6 address 2001:DB8:FEED:2::5/64

ospfv3 network point-to-point

ospfv3 1 ipv4 area 0

ospfv3 1 ipv6 area 0

interface GigabitEthernet0/0/2

no ip address

negotiation auto

interface GigabitEthernet0/0/3

no ip address

negotiation auto

router ospfv3 1

address-family ipv4 unicast

router-id 2.2.2.2

exit-address-family

address-family ipv6 unicast

router-id 2.2.2.2

exit-address-family

router bgp 65000

bgp log-neighbor-changes

neighbor 10.0.0.2 remote-as 65000

neighbor 10.0.0.6 remote-as 65000

neighbor 2001:DB8:FEED:1::2 remote-as 65000

neighbor 2001:DB8:FEED:2::6 remote-as 65000

address-family ipv4

network 172.16.0.0

neighbor 10.0.0.2 activate

neighbor 10.0.0.6 activate

no neighbor 2001:DB8:FEED:1::2 activate

no neighbor 2001:DB8:FEED:2::6 activate

exit-address-family

address-family ipv6

network 2001:DB8:A000::/36

neighbor 2001:DB8:FEED:1::2 activate

neighbor 2001:DB8:FEED:2::6 activate

exit-address-family

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip route 172.16.0.0 255.255.0.0 Null0

ip ssh bulk-mode 131072

ipv6 route 2001:DB8:A000::/36 Null0

control-plane

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line aux 0

line vty 0 4

login

transport input ssh

line vty 5 14

login

transport input ssh

end

R2#

# Router R3

R3# show run brief

Building configuration...

Current configuration : 3211 bytes

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R3

boot-start-marker

boot-end-marker

no aaa new-model

no ip domain lookup

login on-success log

ipv6 unicast-routing

subscriber templating

multilink bundle-name authenticated

pae

crypto pki trustpoint TP-self-signed-2624250798

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-2624250798

revocation-check none

rsakeypair TP-self-signed-2624250798

hash sha512

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

hash sha512

crypto pki certificate chain TP-self-signed-2624250798

certificate self-signed 01

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

diagnostic bootup level minimal

license udi pid C8200L-1N-4T sn FJC2730265C

memory free low-watermark processor 61443

spanning-tree extend system-id

redundancy

mode none

interface GigabitEthernet0/0/0

ip address 10.0.0.6 255.255.255.252

negotiation auto

ipv6 address FE80::3:1 link-local

ipv6 address 2001:DB8:FEED:2::6/64

ospfv3 network point-to-point

ospfv3 1 ipv4 area 0

ospfv3 1 ipv6 area 0

interface GigabitEthernet0/0/1

ip address 209.165.200.9 255.255.255.252

negotiation auto

ipv6 address FE80::3:2 link-local

ipv6 address 2001:DB8:209:8::9/64

interface GigabitEthernet0/0/2

no ip address

negotiation auto

interface GigabitEthernet0/0/3

no ip address

negotiation auto

interface Serial0/1/0

no ip address

interface Serial0/1/1

no ip address

router ospfv3 1

address-family ipv4 unicast

router-id 3.3.3.3

exit-address-family

address-family ipv6 unicast

router-id 3.3.3.3

exit-address-family

router bgp 65000

bgp log-neighbor-changes

neighbor 10.0.0.2 remote-as 65000

neighbor 10.0.0.5 remote-as 65000

neighbor 2001:DB8:209:8::10 remote-as 65002

neighbor 2001:DB8:FEED:1::2 remote-as 65000

neighbor 2001:DB8:FEED:2::5 remote-as 65000

neighbor 209.165.200.10 remote-as 65002

address-family ipv4

neighbor 10.0.0.2 activate

neighbor 10.0.0.2 next-hop-self

neighbor 10.0.0.5 activate

neighbor 10.0.0.5 next-hop-self

no neighbor 2001:DB8:209:8::10 activate

no neighbor 2001:DB8:FEED:1::2 activate

no neighbor 2001:DB8:FEED:2::5 activate

neighbor 209.165.200.10 activate

neighbor 209.165.200.10 filter-list 10 out

exit-address-family

address-family ipv6

neighbor 2001:DB8:209:8::10 activate

neighbor 2001:DB8:209:8::10 filter-list 10 out

neighbor 2001:DB8:FEED:1::2 activate

neighbor 2001:DB8:FEED:1::2 next-hop-self

neighbor 2001:DB8:FEED:2::5 activate

neighbor 2001:DB8:FEED:2::5 next-hop-self

exit-address-family

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip as-path access-list 10 permit ^$

ip ssh bulk-mode 131072

control-plane

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line aux 0

line vty 0 4

login

transport input ssh

line vty 5 14

login

transport input ssh

end

R3#

# Switch D1

D1# show run brief

Building configuration...

Current configuration : 5369 bytes

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform punt-keepalive disable-kernel-core

hostname D1

vrf definition Mgmt-vrf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

switch 1 provision c9300l-24t-4g

ip routing

no ip domain lookup

login on-success log

ipv6 nd cache expire refresh

ipv6 unicast-routing

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

hash sha512

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

license boot level network-advantage addon dna-advantage

memory free low-watermark processor 130178

diagnostic bootup level minimal

spanning-tree mode rapid-pvst

spanning-tree extend system-id

redundancy

mode sso

crypto engine compliance shield disable

transceiver type all

monitoring

class-map match-any system-cpp-police-ewlc-control

description EWLC Control

class-map match-any system-cpp-police-topology-control

description Topology control

class-map match-any system-cpp-police-sw-forward

description Sw forwarding, L2 LVX data packets, LOGGING, Transit Traffic

class-map match-any system-cpp-default

description EWLC Data, Inter FED Traffic

class-map match-any system-cpp-police-sys-data

description Openflow, Exception, EGR Exception, NFL Sampled Data, RPF Failed

class-map match-any system-cpp-police-punt-webauth

description Punt Webauth

class-map match-any system-cpp-police-l2lvx-control

description L2 LVX control packets

class-map match-any system-cpp-police-forus

description Forus Address resolution and Forus traffic

class-map match-any system-cpp-police-multicast-end-station

description MCAST END STATION

class-map match-any system-cpp-police-high-rate-app

description High Rate Applications

class-map match-any system-cpp-police-multicast

description MCAST Data

class-map match-any system-cpp-police-l2-control

description L2 control

class-map match-any system-cpp-police-dot1x-auth

description DOT1X Auth

class-map match-any system-cpp-police-data

description ICMP redirect, ICMP_GEN and BROADCAST

class-map match-any system-cpp-police-stackwise-virt-control

description Stackwise Virtual OOB

class-map match-any non-client-nrt-class

class-map match-any system-cpp-police-routing-control

description Routing control and Low Latency

class-map match-any system-cpp-police-protocol-snooping

description Protocol snooping

class-map match-any system-cpp-police-dhcp-snooping

description DHCP snooping

class-map match-any system-cpp-police-ios-routing

description L2 control, Topology control, Routing control, Low Latency

class-map match-any system-cpp-police-system-critical

description System Critical and Gold Pkt

class-map match-any system-cpp-police-ios-feature

description ICMPGEN,BROADCAST,ICMP,L2LVXCntrl,ProtoSnoop,PuntWebauth,MCASTData,Transit,DOT1XAuth,Swfwd,LOGGING,L2LVXData,ForusTraffic,ForusARP,McastEndStn,Openflow,Exception,EGRExcption,NflSampled,RpfFailed

policy-map system-cpp-policy

interface Loopback0

ip address 192.168.1.1 255.255.255.0

ipv6 enable

interface Loopback1

ip address 192.168.2.1 255.255.255.0

ipv6 enable

interface Loopback2

no ip address

ipv6 address 2001:DB8:CAB::1/48

interface GigabitEthernet0/0

vrf forwarding Mgmt-vrf

no ip address

negotiation auto

interface GigabitEthernet1/0/1

interface GigabitEthernet1/0/2

interface GigabitEthernet1/0/3

interface GigabitEthernet1/0/4

interface GigabitEthernet1/0/5

interface GigabitEthernet1/0/6

interface GigabitEthernet1/0/7

interface GigabitEthernet1/0/8

interface GigabitEthernet1/0/9

interface GigabitEthernet1/0/10

interface GigabitEthernet1/0/11

no switchport

ip address 209.165.200.6 255.255.255.252

ipv6 address FE80::D1:1 link-local

ipv6 address 2001:DB8:209:4::6/64

interface GigabitEthernet1/0/12

interface GigabitEthernet1/0/13

interface GigabitEthernet1/0/14

interface GigabitEthernet1/0/15

interface GigabitEthernet1/0/16

interface GigabitEthernet1/0/17

interface GigabitEthernet1/0/18

interface GigabitEthernet1/0/19

interface GigabitEthernet1/0/20

interface GigabitEthernet1/0/21

interface GigabitEthernet1/0/22

interface GigabitEthernet1/0/23

interface GigabitEthernet1/0/24

interface GigabitEthernet1/1/1

interface GigabitEthernet1/1/2

interface GigabitEthernet1/1/3

interface GigabitEthernet1/1/4

interface AppGigabitEthernet1/0/1

interface Vlan1

no ip address

router bgp 65001

bgp log-neighbor-changes

neighbor 2001:DB8:209:4::5 remote-as 65000

neighbor 209.165.200.5 remote-as 65000

address-family ipv4

network 192.168.1.0

network 192.168.2.0

no neighbor 2001:DB8:209:4::5 activate

neighbor 209.165.200.5 activate

exit-address-family

address-family ipv6

network 2001:DB8:CAB::/48

neighbor 2001:DB8:209:4::5 activate

exit-address-family

ip forward-protocol nd

ip http server

ip http secure-server

ip ssh bulk-mode 131072

control-plane

service-policy input system-cpp-policy

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line vty 0 4

login

transport input ssh

line vty 5 31

login

transport input ssh

end

D1#

# Switch D2

D2# show run brief

Building configuration...

Current configuration : 5780 bytes

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform punt-keepalive disable-kernel-core

hostname D2

vrf definition Mgmt-vrf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

switch 1 provision c9300l-24p-4g

ip routing

no ip domain lookup

login on-success log

ipv6 nd cache expire refresh

ipv6 unicast-routing

crypto pki trustpoint TP-self-signed-4041479302

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-4041479302

revocation-check none

rsakeypair TP-self-signed-4041479302

hash sha512

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

hash sha512

crypto pki certificate chain TP-self-signed-4041479302

certificate self-signed 01

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

license boot level network-advantage addon dna-advantage

memory free low-watermark processor 130178

diagnostic bootup level minimal

spanning-tree mode rapid-pvst

spanning-tree extend system-id

enable secret 9 $9$trwB8ENb5EIepE$bSXI6ItygxikyAO733vrJCJoVUFZsTfIJEkBf8JCQBg

redundancy

mode sso

crypto engine compliance shield disable

transceiver type all

monitoring

class-map match-any system-cpp-police-ewlc-control

description EWLC Control

class-map match-any system-cpp-police-topology-control

description Topology control

class-map match-any system-cpp-police-sw-forward

description Sw forwarding, L2 LVX data packets, LOGGING, Transit Traffic

class-map match-any system-cpp-default

description EWLC Data, Inter FED Traffic

class-map match-any system-cpp-police-sys-data

description Openflow, Exception, EGR Exception, NFL Sampled Data, RPF Failed

class-map match-any system-cpp-police-punt-webauth

description Punt Webauth

class-map match-any system-cpp-police-l2lvx-control

description L2 LVX control packets

class-map match-any system-cpp-police-forus

description Forus Address resolution and Forus traffic

class-map match-any system-cpp-police-multicast-end-station

description MCAST END STATION

class-map match-any system-cpp-police-high-rate-app

description High Rate Applications

class-map match-any system-cpp-police-multicast

description MCAST Data

class-map match-any system-cpp-police-l2-control

description L2 control

class-map match-any system-cpp-police-dot1x-auth

description DOT1X Auth

class-map match-any system-cpp-police-data

description ICMP redirect, ICMP_GEN and BROADCAST

class-map match-any system-cpp-police-stackwise-virt-control

description Stackwise Virtual OOB

class-map match-any non-client-nrt-class

class-map match-any system-cpp-police-routing-control

description Routing control and Low Latency

class-map match-any system-cpp-police-protocol-snooping

description Protocol snooping

class-map match-any system-cpp-police-dhcp-snooping

description DHCP snooping

class-map match-any system-cpp-police-ios-routing

description L2 control, Topology control, Routing control, Low Latency

class-map match-any system-cpp-police-system-critical

description System Critical and Gold Pkt

class-map match-any system-cpp-police-ios-feature

description ICMPGEN,BROADCAST,ICMP,L2LVXCntrl,ProtoSnoop,PuntWebauth,MCASTData,Transit,DOT1XAuth,Swfwd,LOGGING,L2LVXData,ForusTraffic,ForusARP,McastEndStn,Openflow,Exception,EGRExcption,NflSampled,RpfFailed

policy-map system-cpp-policy

interface Loopback0

ip address 192.168.10.1 255.255.255.0

ipv6 enable

interface Loopback1

ip address 192.168.20.1 255.255.255.0

ipv6 enable

interface Loopback2

no ip address

ipv6 address 2001:DB8:ACAD::1/48

interface GigabitEthernet0/0

vrf forwarding Mgmt-vrf

no ip address

shutdown

negotiation auto

interface GigabitEthernet1/0/1

interface GigabitEthernet1/0/2

interface GigabitEthernet1/0/3

interface GigabitEthernet1/0/4

interface GigabitEthernet1/0/5

interface GigabitEthernet1/0/6

interface GigabitEthernet1/0/7

interface GigabitEthernet1/0/8

interface GigabitEthernet1/0/9

interface GigabitEthernet1/0/10

interface GigabitEthernet1/0/11

no switchport

ip address 209.165.200.10 255.255.255.252

ipv6 address FE80::D2:1 link-local

ipv6 address 2001:DB8:209:8::10/64

interface GigabitEthernet1/0/12

interface GigabitEthernet1/0/13

interface GigabitEthernet1/0/14

interface GigabitEthernet1/0/15

interface GigabitEthernet1/0/16

interface GigabitEthernet1/0/17

interface GigabitEthernet1/0/18

interface GigabitEthernet1/0/19

interface GigabitEthernet1/0/20

interface GigabitEthernet1/0/21

interface GigabitEthernet1/0/22

interface GigabitEthernet1/0/23

interface GigabitEthernet1/0/24

interface GigabitEthernet1/1/1

interface GigabitEthernet1/1/2

interface GigabitEthernet1/1/3

interface GigabitEthernet1/1/4

interface AppGigabitEthernet1/0/1

interface Vlan1

no ip address

router bgp 65002

bgp log-neighbor-changes

neighbor 2001:DB8:209:8::9 remote-as 65000

neighbor 209.165.200.9 remote-as 65000

address-family ipv4

network 192.168.10.0

network 192.168.20.0

no neighbor 2001:DB8:209:8::9 activate

neighbor 209.165.200.9 activate

exit-address-family

address-family ipv6

network 2001:DB8:ACAD::/48

neighbor 2001:DB8:209:8::9 activate

exit-address-family

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip ssh bulk-mode 131072

control-plane

service-policy input system-cpp-policy

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line vty 0 4

login

transport input ssh

line vty 5 31

login

transport input ssh

end

D2#