# Lab - Implement Advanced EIGRP for IPv4 Features (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

# Topology

# Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| R1 | G0/0/0 | 10.12.0.1 | 255.255.255.0 |
| R1 | G0/0/1 | 192.168.3.1 | 255.255.255.0 |
| R1 | S0/1/0 | 10.13.0.1 | 255.255.255.0 |
| R2 | G0/0/0 | 10.12.0.2 | 255.255.255.0 |
| R2 | G0/0/1 | 10.23.0.2 | 255.255.255.0 |
| R3 | G0/0/0 | 10.23.0.3 | 255.255.255.0 |
| R3 | S0/1/0 | 10.13.0.3 | 255.255.255.0 |
| R3 | Loopback0 | 172.16.3.1 | 255.255.255.0 |
| D1 | G1/0/11 | 192.168.3.2 | 255.255.255.0 |
| D1 | Loopback0 | 192.168.1.1 | 255.255.255.0 |
| D1 | Loopback1 | 192.168.1.1 | 255.255.255.0 |
| D1 | Loopback2 | 192.168.0.1 | 255.255.255.0 |

# Objectives

Part 1: Build the Network and Configure Basic Device Settings

Part 2: Implement EIGRP for IPv4

Part 3: Implement Advanced Features

# Background / Scenario

Customizing the operation of EIGRP can yield many benefits, most notably speeding convergence and stabilizing network operations during outages. In this lab you will explore some advanced techniques that can be used to customize and improve EIGRP performance on an enterprise network.

Note: The routers used with CCNP hands-on labs are Cisco 8200L with Cisco IOS XE Release 17.12.5 (universalk9 image). The switches used in the labs are Cisco Catalyst 9300L with Cisco IOS XE Release 17.12.5 (universalk9 image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs.

Note: Make sure that the routers and switches have been erased and have no startup configurations. If you are unsure, contact your instructor.

Instructor Note: Refer to the Instructor Lab Manual for the procedures to initialize and reload devices.

# Required Resources

3 Routers (Cisco 8200L with Cisco IOS XE Release 17.12.5c universal image or comparable)

1 Switch (Cisco 9300L with Cisco IOS XE release 17.12.5 universal image or comparable)

1 PC (Choice of operating system with a terminal emulation program installed)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

# Instructions

## Build the Network and Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings and interface addressing.

### Cable the network as shown in the topology.

Attach the devices as shown in the topology diagram, and cable as necessary.

### Configure basic settings for each device.

    - Console into each device, enter global configuration mode, and apply the basic settings. The startup configurations for each device are provided below.

Open configuration window

Router R1

hostname R1

no ip domain lookup

banner motd # R1, Implement Advanced EIGRP for IPv4 Features #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

exec-timeout 0 0

password cisco123

login

exit

interface g0/0/0

ip address 10.12.0.1 255.255.255.0

no shutdown

exit

interface s0/1/0

ip address 10.13.0.1 255.255.255.0

no shutdown

exit

interface g0/0/1

ip address 192.168.3.1 255.255.255.0

no shutdown

exit

end

Router R2

hostname R2

no ip domain lookup

banner motd # R2, Implement Advanced EIGRP for IPv4 Features #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

exec-timeout 0 0

password cisco123

login

exit

interface g0/0/0

ip address 10.12.0.2 255.255.255.0

no shutdown

exit

interface g0/0/1

ip address 10.23.0.2 255.255.255.0

no shutdown

exit

end

Router R3

hostname R3

no ip domain lookup

banner motd # R3, Implement Advanced EIGRP for IPv4 Features #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

exec-timeout 0 0

password cisco123

login

transport input telnet

exit

interface g0/0/0

ip address 10.23.0.3 255.255.255.0

no shutdown

exit

interface s0/1/0

ip address 10.13.0.3 255.255.255.0

no shutdown

exit

interface loopback 0

ip address 172.16.3.1 255.255.255.0

no shutdown

exit

end

Switch D1

hostname D1

no ip domain lookup

ip routing

banner motd # D1, Implement Advanced EIGRP for IPv4 Features #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

exec-timeout 0 0

password cisco123

login

exit

interface range g1/0/1-24

shutdown

exit

interface g1/0/11

no switchport

ip address 192.168.3.2 255.255.255.0

no shutdown

exit

interface loopback 0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface loopback 1

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface loopback 2

ip address 192.168.0.1 255.255.255.0

no shutdown

exit

end

    - Set the clock on all devices to UTC time.

    - Save the running configuration to startup-config on all devices.

Close configuration window

## Implement EIGRP for IPv4

In this part, you will configure classic EIGRP for IPv4 and verify that all routing tables are converged.

### Configure classic EIGRP for IPv4.

Enable debugging on R1 so you can observe the route exchange process.

R1# debug eigrp packets

(UPDATE, REQUEST, QUERY, REPLY, HELLO, UNKNOWN, PROBE, ACK, STUB, SIAQUERY, SIAREPLY)

EIGRP Packet debugging is on

Configure classic EIGRP for IPv4 on all devices. Use Autonomous System number 98 and advertise only the connected interfaces on each device.

R1# config t

Enter configuration commands, one per line.  End with CNTL/Z.

R1(config)# router eigrp 98

R1(config-router)# network 10.12.0.0 0.0.0.255

R1(config-router)# network 10.13.0.0 0.0.0.255

R1(config-router)# network 192.168.3.0 0.0.0.255

R1(config-router)# exit

R1(config)# end

R2# config t

Enter configuration commands, one per line.  End with CNTL/Z.

R2(config)# router eigrp 98

R2(config-router)# network 10.12.0.0 0.0.0.255

R2(config-router)# network 10.23.0.0 0.0.0.255

R2(config-router)# exit

R2(config)# end

R3# config t

Enter configuration commands, one per line.  End with CNTL/Z.

R3(config)# router eigrp 98

R3(config-router)# network 10.23.0.0 0.0.0.255

R3(config-router)# network 10.13.0.0 0.0.0.255

R3(config-router)# network 172.16.3.0 0.0.0.255

R3(config-router)# exit

R3(config)# end

D1# config t

Enter configuration commands, one per line.  End with CNTL/Z.

D1(config)# router eigrp 98

D1(config-router)# network 192.168.3.0 0.0.0.255

D1(config-router)# network 192.168.2.0 0.0.0.255

D1(config-router)# network 192.168.1.0 0.0.0.255

D1(config-router)# network 192.168.0.0 0.0.0.255

D1(config-router)# exit

D1(config)# end

### Review the debugging results.

Stop debugging.

R1# no debug eigrp packets

Review the debugging results. As soon as you enable EIGRP on R1, EIGRP starts to send and receive hello packets for neighbor discovery. In this example, when R1 receives hello packets from the neighbors R2 and D1, R1 forms new EIGRP adjacencies.

Update packets with routing information are sent to the newly discovered neighbors. When there are link or metric changes, the update packets are multicast to the neighbors.

EIGRP sends query packets to find feasible successors to a destination from a neighbor router. The neighbor router sends a reply packet providing feasible successor information in response to the sender of the query packet.

Acknowledgment (ACK) packets confirm the successful receipt of EIGRP update, query or reply packets.

*Oct 17 01:16:54.040: EIGRP: Sending HELLO on Gi0/0/0 - paklen 20

*Oct 17 01:16:54.040:   AS 98, Flags 0x0:(NULL), Seq 0/0 interfaceQ 0/0 iidbQ un/rely 0/0

*Oct 17 01:16:54.044: EIGRP: Sending HELLO on Se0/1/0 - paklen 20

*Oct 17 01:16:54.044:   AS 98, Flags 0x0:(NULL), Seq 0/0 interfaceQ 0/0 iidbQ un/rely 0/0

*Oct 17 01:16:54.044: EIGRP: Sending HELLO on Gi0/0/1 - paklen 20

*Oct 17 01:16:54.044:   AS 98, Flags 0x0:(NULL), Seq 0/0 interfaceQ 0/0 iidbQ un/rely 0/0

*Oct 17 01:16:54.085: EIGRP: Received HELLO on Gi0/0/0 - paklen 30 nbr 10.12.0.2

*Oct 17 01:16:54.095:   AS 98, Flags 0x0:(NULL), Seq 0/0 interfaceQ 0/0

*Oct 17 01:16:54.104: EIGRP: Add Peer: Total 1 (1/0/0/0/0)

*Oct 17 01:16:54.104: EIGRP: Add Peer: Total 1 (1/0/1/0/0)

*Oct 17 01:16:54.104: %DUAL-5-NBRCHANGE: EIGRP-IPv4 98: Neighbor 10.12.0.2 (GigabitEthernet0/0/0) is up: new adjacency

*Oct 17 01:16:54.113: EIGRP: Enqueueing UPDATE on Gi0/0/0 - paklen 0 nbr 10.12.0.2 tid 0 iidbQ un/rely 0/1 peerQ un/rely 0/0

*Oct 17 01:16:54.114: EIGRP: Received HELLO on Gi0/0/1 - paklen 30 nbr 192.168.3.2

*Oct 17 01:16:54.114:   AS 98, Flags 0x0:(NULL), Seq 0/0 interfaceQ 0/0

*Oct 17 01:16:54.114: EIGRP: Add Peer: Total 1 (1/0/0/0/0)

*Oct 17 01:16:54.114: EIGRP: Add Peer: Total 1 (1/0/1/0/0)

*Oct 17 01:16:54.114: %DUAL-5-NBRCHANGE: EIGRP-IPv4 98: Neighbor 192.168.3.2 (GigabitEthernet0/0/1) is up: new adjacency

*Oct 17 01:16:54.114: EIGRP: Enqueueing UPDATE on Gi0/0/1 - paklen 0 nbr 192.168.3.2 tid 0 iidbQ un/rely 0/1 peerQ un/rely 0/0

*Oct 17 01:16:54.114: EIGRP: Received HELLO on Se0/1/0 - paklen 30 nbr 10.13.0.3

*Oct 17 01:16:54.114:   AS 98, Flags 0x0:(NULL), Seq 0/0 interfaceQ 0/0

*Oct 17 01:16:54.114: EIGRP: Add Peer: Total 1 (1/0/0/0/0)

*Oct 17 01:16:54.114: EIGRP: Add Peer: Total 1 (1/0/1/0/0)

*Oct 17 01:16:54.114: %DUAL-5-NBRCHANGE: EIGRP-IPv4 98: Neighbor 10.13.0.3 (Serial0/1/0) is up: new adjacency

*Oct 17 01:16:54.114: EIGRP: Enqueueing UPDATE on Se0/1/0 - paklen 0 nbr 10.13.0.3 tid 0 iidbQ un/rely 0/1 peerQ un/rely 0/0

*Oct 17 01:16:54.114: EIGRP: Received UPDATE on Se0/1/0 - paklen 0 nbr 10.13.0.3

*Oct 17 01:16:54.114:   AS 98, Flags 0x1:(INIT), Seq 5/0 interfaceQ 0/0 iidbQ un/rely 0/1 peerQ un/rely 0/0

<output omitted>

*Oct 17 01:16:54.160: EIGRP: Requeued unicast on GigabitEthernet0/0/1

*Oct 17 01:16:54.160: EIGRP: Requeued unicast on Serial0/1/0

*Oct 17 01:16:54.160: EIGRP: Sending UPDATE on Gi0/0/0 - paklen 0 nbr 10.12.0.2 tid 0

*Oct 17 01:16:54.160:   AS 98, Flags 0x1:(INIT), Seq 1/0 interfaceQ 0/0 iidbQ un/rely 0/0 peerQ un/rely 0/1

*Oct 17 01:16:54.168: EIGRP: Sending UPDATE on Gi0/0/1 - paklen 0 nbr 192.168.3.2 tid 0

*Oct 17 01:16:54.168:   AS 98, Flags 0x1:(INIT), Seq 2/0 interfaceQ 0/0 iidbQ un/rely 0/0 peerQ un/rely 0/1

*Oct 17 01:16:54.168: EIGRP: Sending UPDATE on Se0/1/0 - paklen 0 nbr 10.13.0.3 tid 0

*Oct 17 01:16:54.168:   AS 98, Flags 0x1:(INIT), Seq 3/5 interfaceQ 0/0 iidbQ un/rely 0/0 peerQ un/rely 0/1

*Oct 17 01:16:54.171: EIGRP: Received ACK on Se0/1/0 - paklen 0 nbr 10.13.0.3

*Oct 17 01:16:54.171:   AS 98, Flags 0x0:(NULL), Seq 0/3 interfaceQ 0/0 iidbQ un/rely 0/0 peerQ un/rely 0/1

*Oct 17 01:16:54.171: EIGRP: Enqueueing UPDATE on Se0/1/0 - paklen 0 tid 0 iidbQ un/rely 0/1 serno 1-3

*Oct 17 01:16:54.172: EIGRP: Enqueueing UPDATE on Se0/1/0 - paklen 0 nbr 10.13.0.3 tid 0 iidbQ un/rely 0/0 peerQ un/rely 0/0 serno 1-3

*Oct 17 01:16:54.183: EIGRP: Received UPDATE on Se0/1/0 - paklen 132 nbr 10.13.0.3

*Oct 17 01:16:54.183:   AS 98, Flags 0x8:(EOT), Seq 6/3 interfaceQ 0/0 iidbQ un/rely 0/0 peerQ un/rely 0/1

*Oct 17 01:16:54.183: EIGRP: Enqueueing ACK on Se0/1/0 - paklen 0 nbr 10.13.0.3 tid 0

*Oct 17 01:16:54.183:   Ack seq 6 iidbQ un/rely 0/0 peerQ un/rely 1/1

*Oct 17 01:16:54.214: EIGRP: Sending ACK on Se0/1/0 - paklen 0 nbr 10.13.0.3 tid 0

*Oct 17 01:16:54.214:   AS 98, Flags 0x0:(NULL), Seq 0/6 interfaceQ 0/0 iidbQ un/rely 0/0 peerQ un/rely 1/1

*Oct 17 01:16:54.214: EIGRP: Sending UPDATE on Se0/1/0 - paklen 88 nbr 10.13.0.3 tid 0

### Verify EIGRP for IPv4 routing.

Verify that each device has a complete routing table for all the networks shown in the topology and Addressing Table.

R1# show ip route eigrp | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 5 subnets, 2 masks

D        10.23.0.0/24 [90/3072] via 10.12.0.2, 00:06:05, GigabitEthernet0/0/0

172.16.0.0/24 is subnetted, 1 subnets

D        172.16.3.0 [90/131072] via 10.12.0.2, 00:06:04, GigabitEthernet0/0/0

D     192.168.0.0/24

[90/130816] via 192.168.3.2, 00:05:37, GigabitEthernet0/0/1

D     192.168.1.0/24

[90/130816] via 192.168.3.2, 00:05:37, GigabitEthernet0/0/1

D     192.168.2.0/24

[90/130816] via 192.168.3.2, 00:05:37, GigabitEthernet0/0/1

R2# show ip route eigrp | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 5 subnets, 2 masks

D        10.13.0.0/24

[90/1782016] via 10.12.0.1, 00:06:47, GigabitEthernet0/0/0

172.16.0.0/24 is subnetted, 1 subnets

D        172.16.3.0 [90/130816] via 10.23.0.3, 00:06:47, GigabitEthernet0/0/1

D     192.168.0.0/24 [90/131072] via 10.12.0.1, 00:06:20, GigabitEthernet0/0/0

D     192.168.1.0/24 [90/131072] via 10.12.0.1, 00:06:20, GigabitEthernet0/0/0

D     192.168.2.0/24 [90/131072] via 10.12.0.1, 00:06:20, GigabitEthernet0/0/0

D     192.168.3.0/24 [90/3072] via 10.12.0.1, 00:06:44, GigabitEthernet0/0/0

R3# show ip route eigrp | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 5 subnets, 2 masks

D        10.12.0.0/24 [90/3072] via 10.23.0.2, 00:07:25, GigabitEthernet0/0/0

D     192.168.0.0/24 [90/131328] via 10.23.0.2, 00:07:01, GigabitEthernet0/0/0

D     192.168.1.0/24 [90/131328] via 10.23.0.2, 00:07:01, GigabitEthernet0/0/0

D     192.168.2.0/24 [90/131328] via 10.23.0.2, 00:07:01, GigabitEthernet0/0/0

D     192.168.3.0/24 [90/3328] via 10.23.0.2, 00:07:25, GigabitEthernet0/0/0

D1# show ip route eigrp | begin Gateway

Gateway of last resort is not set

10.0.0.0/24 is subnetted, 3 subnets

D        10.12.0.0 [90/3072] via 192.168.3.1, 00:07:37, GigabitEthernet1/0/11

D        10.13.0.0

[90/1782016] via 192.168.3.1, 00:07:37, GigabitEthernet1/0/11

D        10.23.0.0 [90/3328] via 192.168.3.1, 00:07:37, GigabitEthernet1/0/11

172.16.0.0/24 is subnetted, 1 subnets

D        172.16.3.0

[90/131328] via 192.168.3.1, 00:07:37, GigabitEthernet1/0/11

Issue the show ip eigrp traffic command to verify EIGRP packet traffic statistics on AS 98. This command provides statistical information about the types of packets sent and received by the devices.

R2# show ip eigrp traffic

EIGRP-IPv4 Traffic Statistics for AS(98)

Hellos sent/received: 649/646

Updates sent/received: 16/13

Queries sent/received: 0/0

Replies sent/received: 0/0

Acks sent/received: 8/16

SIA-Queries sent/received: 0/0

SIA-Replies sent/received: 0/0

Hello Process ID: 468

PDM Process ID: 428

Socket Queue: 0/10000/1/0 (current/max/highest/drops)

Input Queue: 0/10000/1/0 (current/max/highest/drops)

### Choosing the best path.

EIGRP uses the Diffusing Update Algorithm (DUAL) to calculate the best paths to the destination networks. DUAL ensures loop-free routes to the destination networks and synchronizes simultaneously when there is a change in the topology. The network devices unaffected by the topology or link changes do not participate in the recalculations.

DUAL uses the distance information to determine the best route to the destination.

Reported or advertised distance (RD or AD): The metric (distance) to a specific destination network from the neighbor router as advertised by the neighboring router

Feasible distance (FD): The metric (total distance) of the best route to a specific destination network from the local router. The metric of the best route is the sum of the AD from the neighbor router and the metric to the neighbor router from the local router.

Successor Route: The route with the smallest metric

Successor: The successor has the best path the destination. Up to four successors can be displayed in the routing table by default.

Feasible successor route: This route requires the RD of alternative backup route to be smaller than the FD of the best route.

Feasible successor: The next hop routers that satisfy the feature successor condition has the backup path to the destination.

For example, the FD to 172.16.3.0 on R3 is 131072 as noted in the routing table on R1. The best route to the destination networks on R3 is via G0/0/0 on R1.

R1# show ip route eigrp | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 5 subnets, 2 masks

D        10.23.0.0/24 [90/3072] via 10.12.0.2, 00:06:05, GigabitEthernet0/0/0

172.16.0.0/24 is subnetted, 1 subnets

D        172.16.3.0 [90/131072] via 10.12.0.2, 00:06:04, GigabitEthernet0/0/0

D     192.168.0.0/24

[90/130816] via 192.168.3.2, 00:05:37, GigabitEthernet0/0/1

D     192.168.1.0/24

[90/130816] via 192.168.3.2, 00:05:37, GigabitEthernet0/0/1

D     192.168.2.0/24

[90/130816] via 192.168.3.2, 00:05:37, GigabitEthernet0/0/1

To review the alternate routes to the destination networks on R3, you can use the show ip eigrp topology commands.

On R1, enter the show ip eigrp topology command. Note the FD for to the remote network 172.16.3.0/24. Furthermore, note the alternate route to the destination via Serial0/1/0.

The successor route to 172.16.3.0/24 is through G0/0/0 and FD is 131072, as displayed in the routing table. The advertised distance is 130816, which is smaller than FD, meets the requirements for a loop-free topology.

R1# show ip eigrp topology

EIGRP-IPv4 Topology Table for AS(98)/ID(192.168.3.1)

Codes: P - Passive, A - Active, U - Update, Q - Query, R - Reply,

r - reply Status, s - sia Status

P 192.168.3.0/24, 1 successors, FD is 2816

via Connected, GigabitEthernet0/0/1

P 192.168.2.0/24, 1 successors, FD is 130816

via 192.168.3.2 (130816/128256), GigabitEthernet0/0/1

P 10.13.0.0/24, 1 successors, FD is 1792000

via Connected, Serial0/1/0

via 10.12.0.2 (1782272/1782016), GigabitEthernet0/0/0

P 192.168.0.0/24, 1 successors, FD is 130816

via 192.168.3.2 (130816/128256), GigabitEthernet0/0/1

P 192.168.1.0/24, 1 successors, FD is 130816

via 192.168.3.2 (130816/128256), GigabitEthernet0/0/1

P 172.16.3.0/24, 1 successors, FD is 131072

via 10.12.0.2 (131072/130816), GigabitEthernet0/0/0

via 10.13.0.3 (1920000/128256), Serial0/1/0

P 10.23.0.0/24, 1 successors, FD is 3072

via 10.12.0.2 (3072/2816), GigabitEthernet0/0/0

via 10.13.0.3 (1792256/2816), Serial0/1/0

P 10.12.0.0/24, 1 successors, FD is 2816

via Connected, GigabitEthernet0/0/0

As displayed in the results, you can also reach 172.16.3.0/24 via the Serial 0/1/0 interface if the route via G0/0/0 is down.

Now that you have confirmed that you have a feasible successor from R1 to 172.16.3.0/24 network on R3, you can observe the quick EIGRP convergence when a link fails in the successor route. Send 100,000 pings from R1 to the loopback 0 interface (172.16.3.1) on R3. Then simulate a link failure by shutting down the G0/0/0 interface on R2.

R1# ping 172.16.3.1 repeat 100000

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!.

*Oct 17 19:46:00.792: %LINK-3-UPDOWN: Interface GigabitEthernet0/0/0, changed state to down

*Oct 17 19:46:00.806: %DUAL-5-NBRCHANGE: EIGRP-IPv4 98: Neighbor 10.12.0.2 (GigabitEthernet0/0/0) is down: interface down.!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

*Oct 17 19:46:01.792: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0/0, changed state to down!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

Only 2 packets were lost.

To display all possible routes, issue the show ip eigrip topology all-links command. Non-successor route may also be displayed in the topology table. Non-successor routers are routes that are not feasible successor candidates.

R1# show ip eigrp topology all-links

EIGRP-IPv4 Topology Table for AS(98)/ID(192.168.3.1)

Codes: P - Passive, A - Active, U - Update, Q - Query, R - Reply,

r - reply Status, s - sia Status

P 192.168.3.0/24, 1 successors, FD is 2816, serno 3

via Connected, GigabitEthernet0/0/1

P 192.168.2.0/24, 1 successors, FD is 130816, serno 8

via 192.168.3.2 (130816/128256), GigabitEthernet0/0/1

via 10.13.0.3 (1920768/131328), Serial0/1/0

P 10.13.0.0/24, 1 successors, FD is 1792000, serno 2

via Connected, Serial0/1/0

via 10.12.0.2 (1782272/1782016), GigabitEthernet0/0/0

P 192.168.0.0/24, 1 successors, FD is 130816, serno 10

via 192.168.3.2 (130816/128256), GigabitEthernet0/0/1

via 10.13.0.3 (1920768/131328), Serial0/1/0

P 192.168.1.0/24, 1 successors, FD is 130816, serno 9

via 192.168.3.2 (130816/128256), GigabitEthernet0/0/1

via 10.13.0.3 (1920768/131328), Serial0/1/0

P 172.16.3.0/24, 1 successors, FD is 131072, serno 41

via 10.12.0.2 (131072/130816), GigabitEthernet0/0/0

via 10.13.0.3 (1920000/128256), Serial0/1/0

P 10.23.0.0/24, 1 successors, FD is 3072, serno 40

via 10.12.0.2 (3072/2816), GigabitEthernet0/0/0

via 10.13.0.3 (1792256/2816), Serial0/1/0

P 10.12.0.0/24, 1 successors, FD is 2816, serno 39

via Connected, GigabitEthernet0/0/0

via 10.13.0.3 (1792512/3072), Serial0/1/0

## Implement Advanced Features

In this part of the lab, you will customize several different settings within EIGRP and see the impact of those changes on the network.

### Modify timers.

EIGRP uses standard hello-interval and hold-time timers based on the speed of the interface. If the interface speed is a T1 or less, hellos are sent every 60 seconds and the hold-time is set to 180 seconds. If the interface speed is greater than a T1, hellos are sent every 5 seconds and the hold-time is set to 15 seconds. These default times might not be appropriate for some network scenarios.

To see what the timers are set to, issue the show ip eigrp interfaces detail command.

Open configuration window

R1# show ip eigrp interfaces detail

EIGRP-IPv4 Interfaces for AS(98)

Xmit Queue   PeerQ        Mean   Pacing Time   Multicast    Pending

Interface              Peers  Un/Reliable  Un/Reliable  SRTT   Un/Reliable   Flow Timer   Routes

Gi0/0/0                  0        0/0       0/0           0       0/0           50           0

Hello-interval is 5, Hold-time is 15

Split-horizon is enabled

Next xmit serial <none>

Packetized sent/expedited: 5/1

Hello's sent/expedited: 29/2

Un/reliable mcasts: 0/5  Un/reliable ucasts: 7/3

Mcast exceptions: 0  CR packets: 0  ACKs suppressed: 0

Retransmissions sent: 2  Out-of-sequence rcvd: 0

Topology-ids on interface - 0

Authentication mode is not set

Topologies advertised on this interface:  base

Topologies not advertised on this interface:

Gi0/0/1                  1        0/0       0/0           1       0/0           50           0

Hello-interval is 5, Hold-time is 15

Split-horizon is enabled

Next xmit serial <none>

Packetized sent/expedited: 12/1

Hello's sent/expedited: 387/2

Un/reliable mcasts: 0/12  Un/reliable ucasts: 13/5

Mcast exceptions: 0  CR packets: 0  ACKs suppressed: 0

Retransmissions sent: 2  Out-of-sequence rcvd: 0

Topology-ids on interface - 0

Authentication mode is not set

Topologies advertised on this interface:  base

Topologies not advertised on this interface:

Se0/1/0                  1        0/0       0/0           1       0/12          50           0

Hello-interval is 5, Hold-time is 15

Split-horizon is enabled

Next xmit serial <none>

Packetized sent/expedited: 4/0

Hello's sent/expedited: 32/2

Un/reliable mcasts: 0/0  Un/reliable ucasts: 6/5

Mcast exceptions: 0  CR packets: 0  ACKs suppressed: 0

Retransmissions sent: 0  Out-of-sequence rcvd: 0

Topology-ids on interface - 0

Authentication mode is not set

Topologies advertised on this interface:  base

Topologies not advertised on this interface:

For this lab, the timers on R1 interface G0/0/0 and S0/1/0 need to be adjusted to send hellos every 10 seconds and establish a hold time of 30 seconds. EIGRP is unique in that each interface can have a customized hello-interval and hold-time. The times are not needed to match between the ends of a link. Change the timers using the ip hello-interval eigrp ASN seconds and the ip hold-time eigrp ASN seconds interface configuration commands.

R1# config t

Enter configuration commands, one per line.  End with CNTL/Z.

R1(config)# interface g0/0/0

R1(config-if)# ip hello-interval eigrp 98 10

R1(config-if)# ip hold-time eigrp 98 30

R1(config-if)# exit

R1(config)# interface s0/1/0

R1(config-if)# ip hello-interval eigrp 98 10

R1(config-if)# ip hold-time eigrp 98 30

R1(config-if)# end

To verify that the changes were made, check the output of the show ip eigrp interfaces detail command.

R1# show ip eigrp interfaces detail

EIGRP-IPv4 Interfaces for AS(98)

Xmit Queue   PeerQ        Mean   Pacing Time   Multicast    Pending

Interface              Peers  Un/Reliable  Un/Reliable  SRTT   Un/Reliable   Flow Timer   Routes

Gi0/0/0                  0        0/0       0/0           0       0/0           50           0

Hello-interval is 10, Hold-time is 30

Split-horizon is enabled

Next xmit serial <none>

Packetized sent/expedited: 12/2

Hello's sent/expedited: 381/3

Un/reliable mcasts: 0/9  Un/reliable ucasts: 12/7

Mcast exceptions: 0  CR packets: 0  ACKs suppressed: 0

Retransmissions sent: 4  Out-of-sequence rcvd: 0

Topology-ids on interface - 0

Authentication mode is not set

Topologies advertised on this interface:  base

Topologies not advertised on this interface:

Gi0/0/1                  1        0/0       0/0           1       0/0           50           0

Hello-interval is 5, Hold-time is 15

Split-horizon is enabled

Next xmit serial <none>

Packetized sent/expedited: 12/1

Hello's sent/expedited: 387/2

Un/reliable mcasts: 0/12  Un/reliable ucasts: 13/5

Mcast exceptions: 0  CR packets: 0  ACKs suppressed: 0

Retransmissions sent: 2  Out-of-sequence rcvd: 0

Topology-ids on interface - 0

Authentication mode is not set

Topologies advertised on this interface:  base

Topologies not advertised on this interface:

Se0/1/0                  1        0/0       0/0           1       0/12          50           0

Hello-interval is 10, Hold-time is 30

Split-horizon is enabled

Next xmit serial <none>

Packetized sent/expedited: 8/0

Hello's sent/expedited: 273/2

Un/reliable mcasts: 0/0  Un/reliable ucasts: 8/11

Mcast exceptions: 0  CR packets: 0  ACKs suppressed: 0

Retransmissions sent: 0  Out-of-sequence rcvd: 0

Topology-ids on interface - 0

Authentication mode is not set

Topologies advertised on this interface:  base

Topologies not advertised on this interface:

Close configuration window

### Create summarized routes in EIGRP.

Large routing tables take more memory and require more CPU time to process. Reducing the size of the routing table is advantageous in all network scenarios. EIGRP supports summarization of routes at any point in the network. There is no boundary router limitation like the limitations imposed in OSPF. However, in order for the summary route to be valid, EIGRP requires that some component of the summary route be in the routing table for the router doing the summarization.

Issue the show ip route eigrp | begin Gateway command on R3 and note the group of networks from the 192.168 range of addresses. R1 is advertising this contiguous block of networks individually, instead of sending a summary.

Open configuration window

R3# show ip route eigrp | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 5 subnets, 2 masks

D        10.12.0.0/24 [90/3072] via 10.23.0.2, 00:06:35, GigabitEthernet0/0/0

D     192.168.0.0/24 [90/131328] via 10.23.0.2, 00:06:08, GigabitEthernet0/0/0

D     192.168.1.0/24 [90/131328] via 10.23.0.2, 00:06:08, GigabitEthernet0/0/0

D     192.168.2.0/24 [90/131328] via 10.23.0.2, 00:06:08, GigabitEthernet0/0/0

D     192.168.3.0/24 [90/3328] via 10.23.0.2, 00:06:35, GigabitEthernet0/0/0

close configuration window

On R1, configure a summary of the networks between R1 and D1, as well as the networks on D1 on the interfaces connecting to R3 and R2.

Open configuration window

R1# conf t

R1(config)# interface g0/0/0

R1(config-if)# ip summary-address eigrp 98 192.168.0.0 255.255.252.0

*Mar  6 00:07:33.742: %DUAL-5-NBRCHANGE: EIGRP-IPv4 98: Neighbor 10.12.0.2 (GigabitEthernet0/0/0) is resync: summary configured

R1(config-if)# exit

R1(config)# interface s0/1/0

R1(config-if)# ip summary-address eigrp 98 192.168.0.0 255.255.252.0

*Mar  6 00:07:35.220: %DUAL-5-NBRCHANGE: EIGRP-IPv4 98: Neighbor 10.13.0.3 (Serial0/1/0) is resync: summary configured

R1(config-if)# end

close configuration window

Now examine the routing table on R3 again using the show ip route eigrp | begin Gateway command. In the output, you now see a single route taking the place of what had been four distinct routes.

Open configuration window

R3# show ip route eigrp | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 5 subnets, 2 masks

D        10.12.0.0/24 [90/3072] via 10.23.0.2, 00:07:59, GigabitEthernet0/0/0

D     192.168.0.0/22 [90/3328] via 10.23.0.2, 00:00:25, GigabitEthernet0/0/0

close configuration window

### Control EIGRP query propagation with EIGRP stub routers.

EIGRP uses query messages to find a path to networks in the autonomous system. The query messages always require an acknowledgement. But a router will only send a response if it has a potential route that satisfies the query. If it does not have a route, it sends its own queries to its neighbors. This process can lead to long delays in reconvergence after an outage.

Query scoping refers to using various techniques to control how far across a network queries have to be sent. Summarization is one way of controlling query propagation. Another way to control query propagation is to use EIGRP stub routers where appropriate. When a router is single-homed to the rest of the network, and no other networks exist beyond that router, there is no real point in sending it a query looking for lost networks. The stub router declares itself as a stub to the router connected to the rest of the network, which is considered a hub router. The hub router then forwards no queries to the stub router because it knows there are no other networks, beyond those reported, existing beyond the stub router. In the topology for this lab, switch D1 is a stub router and R1 is its hub router.

To verify that switch D1 is receiving EIGRP queries, issue the shutdown command on R2 interface G0/0/1. On switch D1, issue the show ip eigrp events command. This command outputs a timestamped list of actions that EIGRP is taking. In the output, you will find an entry that says switch D1 received a query trying to find the 10.23.0.0/24 network. Take note of the time stamp.

Open configuration window

D1# show ip eigrp events

Event information for AS 98:

1    00:10:46.753 NDB delete: 10.23.0.0/24 1

2    00:10:46.753 RDB delete: 10.23.0.0/24 192.168.3.1

3    00:10:46.737 Metric set: 10.23.0.0/24 metric(Infinity)

4    00:10:46.737 Poison squashed: 10.23.0.0/24 lost if

5    00:10:46.737 Poison squashed: 10.23.0.0/24 rt net gone

6    00:10:46.737 Route installing: 10.23.0.0/24 192.168.3.1

7    00:10:46.737 Send reply: 10.23.0.0/24 192.168.3.1

8    00:10:46.737 Not active net/1=SH: 10.23.0.0/24 1

9    00:10:46.737 FC not sat Dmin/met: metric(Infinity) metric(1782272)

10   00:10:46.737 Find FS: 10.23.0.0/24 metric(1782272)

11   00:10:46.737 Rcv query met/succ met: metric(Infinity) metric(Infinity)

12   00:10:46.737 Rcv query dest/nh: 10.23.0.0/24 192.168.3.1

Issue the no shutdown command on R2 interface G0/0/1.

Configure D1 as an EIGRP stub router.

D1# config t

Enter configuration commands, one per line.  End with CNTL/Z.

D1(config)# router eigrp 98

D1(config-router)# eigrp stub

D1(config-router)# end

D1#

Mar  6 00:11:40.624: %DUAL-5-NBRCHANGE: EIGRP-IPv4 98: Neighbor 192.168.3.1 (GigabitEthernet1/0/11) is down: peer info changed

Mar  6 00:11:45.174: %DUAL-5-NBRCHANGE: EIGRP-IPv4 98: Neighbor 192.168.3.1 (GigabitEthernet1/0/11) is up: new adjacency

Close configuration window

Verify that R1 sees switch D1 as a stub by examining the output of the show ip eigrp neighbor detail command.

Open configuration window

R1# show ip eigrp neighbor detail

EIGRP-IPv4 Neighbors for AS(98)

H   Address                 Interface              Hold Uptime   SRTT   RTO  QSeq

(sec)         (ms)       CntNum

2   192.168.3.2             Gi0/0/1                  14 00:00:39    3   100  010

Version 25.0/2.0, Retrans: 1, Retries: 0, Prefixes: 3

Topology-ids from peer - 0

Topologies advertised to peer:   base

Stub Peer Advertising (CONNECTED SUMMARY ) Routes

Suppressing queries

1   10.13.0.3               Se0/1/0                  11 00:11:44    1   100  028

Time since Restart 00:04:06

Version 25.0/2.0, Retrans: 0, Retries: 0, Prefixes: 3

Topology-ids from peer - 0

Topologies advertised to peer:   base

0   10.12.0.2               Gi0/0/0                  12 00:12:24    1   100  030

Time since Restart 00:04:07

Version 25.0/2.0, Retrans: 2, Retries: 0, Prefixes: 2

Topology-ids from peer - 0

Topologies advertised to peer:   base

Max Nbrs: 0, Current Nbrs: 0

close configuration window

Issue the shutdown command on R2 interface G0/0/1. Take note of the timestamp on the syslog message reporting that the interface is down.

On switch D1, issue the show ip eigrp events command. You will see that no query was received looking for the 10.23.0.0/24 network. The query was stopped at R1, speeding up the convergence process.

Open configuration window

D1# show ip eigrp events

Event information for AS 98:

1    00:12:53.776 NDB delete: 10.23.0.0/24 1

2    00:12:53.776 Poison squashed: 10.23.0.0/24 rt net gone

3    00:12:53.776 RDB delete: 10.23.0.0/24 192.168.3.1

4    00:12:53.776 Not active net/1=SH: 10.23.0.0/24 1

5    00:12:53.776 FC not sat Dmin/met: metric(Infinity) metric(1782272)

6    00:12:53.776 Find FS: 10.23.0.0/24 metric(1782272)

7    00:12:53.776 Rcv update met/succmet: metric(Infinity) metric(Infinity)

8    00:12:53.776 Rcv update dest/nh: 10.23.0.0/24 192.168.3.1

9    00:12:53.776 Ignored route, hopcount: 10.23.0.0 255

10   00:12:50.077 Metric set: 172.16.3.0/24 metric(1910016)

11   00:12:50.077 Route installed: 172.16.3.0/24 192.168.3.1

12   00:12:50.077 Route installing: 172.16.3.0/24 192.168.3.1

13   00:12:50.077 Find FS: 172.16.3.0/24 metric(Infinity)

14   00:12:50.077 Free reply status: 172.16.3.0/24

15   00:12:50.077 Clr handle num/bits: 0 0x0

16   00:12:50.077 Clr handle dest/cnt: 172.16.3.0/24 0

17   00:12:50.077 Rcv reply met/succ met: metric(1910016) metric(1909760)

18   00:12:50.077 Rcv reply dest/nh: 172.16.3.0/24 192.168.3.1

19   00:12:50.077 Metric set: 10.23.0.0/24 metric(1782272)

20   00:12:50.076 Route installed: 10.23.0.0/24 192.168.3.1

21   00:12:50.076 Route installing: 10.23.0.0/24 192.168.3.1

22   00:12:50.076 Find FS: 10.23.0.0/24 metric(Infinity)

23   00:12:50.076 Free reply status: 10.23.0.0/24

close configuration window

Issue the no shutdown command on R2 interface G0/0/1.

### Filter EIGRP routes with a distribute list.

EIGRP supports several different filtering capabilities. The simplest and most direct is to use a distribute list. A distribute list refers to an access list which can be applied to all EIGRP updates being sent by a certain router, or it can be applied to a specific interface to modify updates as they exit. For this exercise, we will filter the 10.12.0.0/24 network from updates being sent out of R2 interface G0/0/1. This will cause a change in R3’s routing table.

Examine the routing table on R3 by issuing the show ip route eigrp | begin Gateway command. In the output, you can see that R3 has calculated the path via R2 at 10.23.0.2 to be the best path to reach the 10.12.0.0/24 network.

Open configuration window

R3# show ip route eigrp | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 5 subnets, 2 masks

D        10.12.0.0/24 [90/3072] via 10.23.0.2, 00:00:17, GigabitEthernet0/0/0

D     192.168.0.0/22 [90/3328] via 10.23.0.2, 00:00:53, GigabitEthernet0/0/0

Close configuration window

Our intent is to change the configuration at R2 so that R3 only learns about the 10.12.0.0/24 network from R1. Create an access list that denies the 10.12.0.0/24 network and permits all other networks.

Open configuration window

R2# config t

R2(config)# ip access-list standard EIGRP-FILTER

R2(config-std-nacl)# deny 10.12.0.0 0.0.255.255

R2(config-std-nacl)# permit any

R2(config-std-nacl)# exit

Enter EIGRP router configuration mode and configure the distribute list to reference the access list you just created, further specifying that the filter should be effective outbound on interface G0/0/1.

R2(config)# router eigrp 98

R2(config-router)# distribute-list EIGRP-FILTER out g0/0/1

R2(config-router)# end

*Mar 6 00:19:56.379: %DUAL-5-NBRCHANGE: EIGRP-IPv4 98: Neighbor 10.23.0.3 (GigabitEthernet0/0/1) is resync: intf route configuration changed

close configuration window

On R3, issue the show ip route eigrp | begin Gateway command. As you can see, the successor for the 10.12.0.0/24 network has changed to R1 at 10.13.0.1.

Open configuration window

R3# show ip route eigrp | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 5 subnets, 2 masks

D        10.12.0.0/24 [90/1792256] via 10.13.0.1, 00:01:30, Serial0/1/0

D     192.168.0.0/22 [90/3328] via 10.23.0.2, 00:03:00, GigabitEthernet0/0/0

Close configuration window

end of document

# Device Configs - Final

# Router R1

R1# show run brief

Current configuration : 2149 bytes

!

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core!

hostname R1

!

boot-start-marker

boot-end-marker

!

no aaa new-model

!

no ip domain lookup

!

login on-success log

!

subscriber templating

!

multilink bundle-name authenticated

!

spanning-tree extend system-id

!

redundancy

mode none

!

interface GigabitEthernet0/0/0

ip address 10.12.0.1 255.255.255.0

ip hello-interval eigrp 98 10

ip hold-time eigrp 98 30

ip summary-address eigrp 98 192.168.0.0 255.255.252.0

negotiation auto

!

interface GigabitEthernet0/0/1

ip address 192.168.3.1 255.255.255.0

negotiation auto

!

interface Serial0/1/0

ip address 10.13.0.1 255.255.255.0

ip hello-interval eigrp 98 10

ip hold-time eigrp 98 30

ip summary-address eigrp 98 192.168.0.0 255.255.252.0

!

interface Serial0/1/1

no ip address

!

router eigrp 98

network 10.12.0.0 0.0.0.255

network 10.13.0.0 0.0.0.255

network 192.168.3.0

!

ip forward-protocol nd

no ip http server

ip http secure-server

!

control-plane

!

banner motd ^C R1, Implement Advanced EIGRP for IPv4 Features ^C

!

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line aux 0

line vty 0 4

exec-timeout 0 0

privilege level 15

password cisco123

login

transport input ssh

line vty 5 14

login

transport input ssh!

end

# Router R2

R2# show run brief

Current configuration : 2062 bytes

!

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core!

hostname R2

!

boot-start-marker

boot-end-marker

!

no aaa new-model

!

no ip domain lookup

!

login on-success log

!

subscriber templating

!

multilink bundle-name authenticated

!

spanning-tree extend system-id

!

redundancy

mode none

!

interface GigabitEthernet0/0/0

ip address 10.12.0.2 255.255.255.0

negotiation auto

!

interface GigabitEthernet0/0/1

ip address 10.23.0.2 255.255.255.0

negotiation auto

!

router eigrp 98

distribute-list EIGRP-FILTER out GigabitEthernet0/0/1

network 10.12.0.0 0.0.0.255

network 10.23.0.0 0.0.0.255

!

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

!

ip ssh bulk-mode 131072

!

ip access-list standard EIGRP-FILTER

10 deny   10.12.0.0 0.0.255.255

20 permit any

!

control-plane

!

banner motd ^C R2, Implement Advanced EIGRP for IPv4 Features ^C

!

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line aux 0

line vty 0 4

exec-timeout 0 0

privilege level 15

password cisco123

login

transport input ssh

line vty 5 14

login

transport input ssh

!

end

# Router R3

R3# show run brief

Building configuration...

Current configuration : 1934 bytes

!

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

!

hostname R3

!

boot-start-marker

boot-end-marker

!

no aaa new-model

!

no ip domain lookup

!

login on-success log

!

subscriber templating

!

multilink bundle-name authenticated

!

spanning-tree extend system-id

!

redundancy

mode non

!

interface Loopback0

ip address 172.16.3.1 255.255.255.0

!

interface GigabitEthernet0/0/0

ip address 10.23.0.3 255.255.255.0

negotiation auto

!

interface GigabitEthernet0/0/1

no ip address

negotiation auto

!

interface Serial0/1/0

ip address 10.13.0.3 255.255.255.0

!

interface Serial0/1/1

no ip address

!

router eigrp 98

network 10.13.0.0 0.0.0.255

network 10.23.0.0 0.0.0.255

network 172.16.3.0 0.0.0.255

!

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

!

ip ssh bulk-mode 131072

!

control-plane

!

banner motd ^C R3, Implement Advanced EIGRP for IPv4 Features ^C

!

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line aux 0

line vty 0 4

exec-timeout 0 0

privilege level 15

password cisco123

login

transport input telnet

line vty 5 14

login

transport input ssh

!

end

# Switch D1

D1# show run

Building configuration...

Current configuration : 5613 bytes

!

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform punt-keepalive disable-kernel-core

!

hostname D1

!

vrf definition Mgmt-vrf

!

address-family ipv4

exit-address-family

!

address-family ipv6

exit-address-family

!

no aaa new-model

switch 1 provision c9300l-24t-4g

!

ip routing

!

no ip domain lookup

!

login on-success log

!

diagnostic bootup level minimal

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

redundancy

mode sso

crypto engine compliance shield disable

!

transceiver type all

monitoring

!

class-map match-any system-cpp-police-ewlc-control

description EWLC Control

class-map match-any system-cpp-police-topology-control

description Topology control

class-map match-any system-cpp-police-sw-forward

description Sw forwarding, L2 LVX data packets, LOGGING, Transit Traffic

class-map match-any system-cpp-default

description EWLC Data, Inter FED Traffic

class-map match-any system-cpp-police-sys-data

description Openflow, Exception, EGR Exception, NFL Sampled Data, RPF Failed

class-map match-any system-cpp-police-punt-webauth

description Punt Webauth

class-map match-any system-cpp-police-l2lvx-control

description L2 LVX control packets

class-map match-any system-cpp-police-forus

description Forus Address resolution and Forus traffic

class-map match-any system-cpp-police-multicast-end-station

description MCAST END STATION

class-map match-any system-cpp-police-high-rate-app

description High Rate Applications

class-map match-any system-cpp-police-multicast

description MCAST Data

class-map match-any system-cpp-police-l2-control

description L2 control

class-map match-any system-cpp-police-dot1x-auth

description DOT1X Auth

class-map match-any system-cpp-police-data

description ICMP redirect, ICMP_GEN and BROADCAST

class-map match-any system-cpp-police-stackwise-virt-control

description Stackwise Virtual OOB

class-map match-any non-client-nrt-class

class-map match-any system-cpp-police-routing-control

description Routing control and Low Latency

class-map match-any system-cpp-police-protocol-snooping

description Protocol snooping

class-map match-any system-cpp-police-dhcp-snooping

description DHCP snooping

class-map match-any system-cpp-police-ios-routing

description L2 control, Topology control, Routing control, Low Latency

class-map match-any system-cpp-police-system-critical

description System Critical and Gold Pkt

class-map match-any system-cpp-police-ios-feature

description ICMPGEN,BROADCAST,ICMP,L2LVXCntrl,ProtoSnoop,PuntWebauth,MCASTData,Transit,DOT1XAuth,Swfwd,LOGGING,L2LVXData,ForusTraffic,ForusARP,McastEndStn,Openflow,Exception,EGRExcption,NflSampled,RpfFailed

!

policy-map system-cpp-policy

!

interface Loopback0

ip address 192.168.2.1 255.255.255.0

!

interface Loopback1

ip address 192.168.1.1 255.255.255.0

!

interface Loopback2

ip address 192.168.0.1 255.255.255.0

!

interface GigabitEthernet0/0

vrf forwarding Mgmt-vrf

no ip address

shutdown

negotiation auto

!

interface GigabitEthernet1/0/1

shutdown

!

interface GigabitEthernet1/0/2

shutdown

!

interface GigabitEthernet1/0/3

shutdown

!

interface GigabitEthernet1/0/4

shutdown

!

interface GigabitEthernet1/0/5

shutdown

!

interface GigabitEthernet1/0/6

shutdown

!

interface GigabitEthernet1/0/7

shutdown

!

interface GigabitEthernet1/0/8

shutdown

!

interface GigabitEthernet1/0/9

shutdown

!

interface GigabitEthernet1/0/10

shutdown

!

interface GigabitEthernet1/0/11

no switchport

ip address 192.168.3.2 255.255.255.0

!

interface GigabitEthernet1/0/12

shutdown

!

interface GigabitEthernet1/0/13

shutdown

!

interface GigabitEthernet1/0/14

shutdown

!

interface GigabitEthernet1/0/15

shutdown

!

interface GigabitEthernet1/0/16

shutdown

!

interface GigabitEthernet1/0/17

shutdown

!

interface GigabitEthernet1/0/18

shutdown

!

interface GigabitEthernet1/0/19

shutdown

!

interface GigabitEthernet1/0/20

shutdown

!

interface GigabitEthernet1/0/21

shutdown

!

interface GigabitEthernet1/0/22

shutdown

!

interface GigabitEthernet1/0/23

shutdown

!

interface GigabitEthernet1/0/24

shutdown

!

interface GigabitEthernet1/1/1

!

interface GigabitEthernet1/1/2

!

interface GigabitEthernet1/1/3

!

interface GigabitEthernet1/1/4

!

interface AppGigabitEthernet1/0/1

!

interface Vlan1

no ip address

!

router eigrp 98

network 192.168.0.0

network 192.168.1.0

network 192.168.2.0

network 192.168.3.0

eigrp stub connected summary

!

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip ssh bulk-mode 131072

!

control-plane

service-policy input system-cpp-policy

!

banner motd ^C D1, Implement Advanced EIGRP for IPv4 Features ^C

!

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line vty 0 4

exec-timeout 0 0

privilege level 15

password cisco123

login

transport input ssh

line vty 5 31

login

transport input ssh

!

end