# Lab – Implement DHCPv6 (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

# Topology

# Addressing Table

| Device | Interface | IPv6 Global Unicast Address | IPv6 Link-Local Address |
| --- | --- | --- | --- |
| R1 | GigabitEthernet 1/0/23 | 2001:db8:acad:23::1/64 | fe80::23:1 |
| R1 |  |  |  |

# Objectives

Part 1: Build the Topology and Configure Basic Device Settings and IP Addressing

Part 2: Stateless Address Autoconfiguration (SLAAC)

Part 3: Implement Stateless DHCPv6 Server on a Router

Part 4: Implement Stateful DHCPv6 Server on a Router

# Background / Scenario

In this lab, you will explore IPv6 address configuration mechanisms, including SLAAC, stateless DHCPv6, and stateful DHCPv6, and observe how Router Advertisements influence host behavior. The router D1 is preconfigured with both an IPv6 global unicast address (GUA) and a link-local address (LLA) on the LAN interface, but IPv6 routing is not yet enabled, so the device is not initially operating as an IPv6 router. While basic IPv4 addressing is present for completeness, IPv4 connectivity is not required to complete this lab. You will progressively enable IPv6 routing and DHCPv6 features to examine how hosts dynamically obtain IPv6 addresses, DNS information, and default gateway information under different configuration models.

Note: The switch used with this CCNP hands-on lab is a Cisco Catalyst 9300L using Cisco IOS XE Release 17.12.05 (universalk9 image). Other Layer 3 switches and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs.

Note: Ensure that the routers and switches have been erased and have no startup configurations. If you are unsure contact your instructor.

Instructor Note: Refer to the Instructor Lab Manual for the procedures to initialize and reload devices.

# Required Resources

1 Switch (Cisco Catalyst 9300L with Cisco IOS XE Release 17.12.05 universal image or comparable)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

# Instructions

## Build the Network and Configure Basic Device Settings and Interface Addressing

In Part 1, you will set up the network topology and configure basic settings and interface addressing on the switch.

### Cable the network as shown in the topology.

Attach the devices as shown in the topology diagram and cable as necessary.

### Configure basic settings for the switch.

Console into the switch, enter global configuration mode, and apply the basic settings and interface addressing using the following startup configuration.

Open configuration window

Switch D1

hostname D1

!

no ip domain lookup

line con 0

exec-timeout 0 0

logging sync

!

inter g 1/0/23

no switchport

description Link to PC

ip add 10.23.0.1 255.255.0.0

ipv6 add 2001:db8:acad:23::1/64

ipv6 add fe80::23:1 link-local

no shutdown

exit

!

ip dhcp pool LAN

network 10.23.0.0 255.255.0.0

default-router 10.23.0.1

domain-name example.netacad.com

exit

!

end

Save the running configuration to startup-config.

PC1 should be able to ping the IPv4 address of the g1/0/23 interface on D1.

- Note: If you receive an APIPA address, perform further troubleshooting.

Close configuration window

## Stateless Address Autoconfiguration (SLAAC)

In this part, you will configure and verify SLAAC (Stateless Address Autoconfiguration) so that PC1 can automatically build its own IPv6 address table and learn a default gateway without using DHCPv6. After enabling IPv6 routing on D1, it will send Router Advertisements (RAs) that provide the IPv6 prefix for the LAN, allowing the PC to generate a global unicast address and begin IPv6 communication. You will then verify that PC1 learned its IPv6 addressing information and can successfully reach D1 using IPv6.

### Examine current IPv6 configuration on D1.

At this stage of the lab, IPv6 routing has not yet been enabled on D1 (ipv6 unicast-routing is not configured), so the router is not sending IPv6 Router Advertisements. The GigabitEthernet1/0/23 interface is already configured with both IPv4 (not applicable) and IPv6 addressing, including a manually assigned IPv6 global unicast address and link-local address, and will serve as the LAN interface for the SLAAC configuration and verification steps.

D1# show run | section interface GigabitEthernet1/0/23

interface GigabitEthernet1/0/23

description Link to PC

no switchport

ip address 10.23.0.1 255.255.0.0

ipv6 address FE80::23:1 link-local

ipv6 address 2001:DB8:ACAD:23::1/64

D1#

### Examine IPv6 interface information on G 1/0/23.

The output from show ipv6 interface confirms that GigabitEthernet1/0/23 is operational and correctly configured with both a link-local address and a global unicast IPv6 address. As a result of having an IPv6 link-local address, the multicast address ff02::1 in the following output confirms that the interface belongs to the all-nodes (all-IPv6 devices) multicast group.

At this point, IPv6 is enabled only at the interface level, and because IPv6 routing is not yet enabled on D1, RAs are not being sent. The same output will be re-examined after ipv6 unicast-routing is configured to reflect IPv6 routing and Neighbor Discovery behavior associated with SLAAC.

D1# show ipv6 interface g 1/0/23

GigabitEthernet1/0/23 is up, line protocol is up

IPv6 is enabled, link-local address is FE80::23:1

No Virtual link-local address(es):

Description: Link to PC

Global unicast address(es):

2001:DB8:ACAD:23::1, subnet is 2001:DB8:ACAD:23::/64

Joined group address(es):

FF02::1

FF02::1:FF00:1

FF02::1:FF23:1

MTU is 1500 bytes

ICMP error messages limited to one every 100 milliseconds

ICMP redirects are enabled

ICMP unreachables are sent

ND DAD is enabled, number of DAD attempts: 1

ND reachable time is 30000 milliseconds (using 30000)

ND NS retransmit interval is 1000 milliseconds

### Verify the IPv6 addressing on PC1.

Examine IPv6 addressing on PC1.

C:\Users\Student> ipconfig

Ethernet adapter Ethernet0:

Connection-specific DNS Suffix  . : netlab.vm

Link-local IPv6 Address . . . . . : fe80::6915:c0d:6c6d:2377%14

Autoconfiguration IPv4 Address. . : 10.23.0.3

Subnet Mask . . . . . . . . . . . : 255.255.0.0

Default Gateway . . . . . . . . . : 10.23.0.1

This output shows that the PC successfully received its IPv4 address via DHCP from D1, as indicated by the address 10.23.0.3/16 and the default gateway 10.23.0.1.

For IPv6, the PC has automatically generated a link-local address (LLA) using a randomly generated interface identifier which is a host-local operating system behavior. The %14 at the end of the IPv6 address is the Windows zone (scope) ID, which uniquely identifies the local interface on which this link-local address is valid.

At this point, the PC has no manually configured or dynamically created IPv6 global unicast address (GUA). There is no dynamically created GUA because PC1 has not yet received any ICMPv6 RAs.

### Turn on debug and enable IPv6 unicast routing.

Before configuring ipv6 unicast-routing, turn on debug ipv6 nd to observe Neighbor Discovery (ND) activity as the router transitions from an IPv6 host to an IPv6 router.

D1# debug ipv6 nd

ICMP Neighbor Discovery events debugging is on

ICMP ND HA events debugging is ON

Enable the router as an IPv6 router using the command ipv6 unicast-routing.

D1# conf t

D1(config)# ipv6 unicast-routing

D1(config)# end

*Jan 21 11:36:03.567: ICMPv6-ND: Node is Router

*Jan 21 11:36:03.567: ICMPv6-ND: (Null0) Periodic RAs Inhibited

*Jan 21 11:36:03.567: ICMPv6-ND: (Null0) Routing change

*Jan 21 11:36:03.567: ICMPv6-ND: (LI-Null0) Routing change

*Jan 21 11:36:03.568: ICMPv6-ND: (GigabitEthernet1/0/23) Routing change

*Jan 21 11:36:03.568: ICMPv6-ND: (GigabitEthernet1/0/23) Periodic RAs Permitted

*Jan 21 11:36:03.568: ICMPv6-ND: (GigabitEthernet1/0/23,FE80::23:1) Link RA context

<output omitted>

The debug output confirms this change by showing that D1 is now operating as a router and that periodic RAs, which were previously inhibited, are now permitted on GigabitEthernet1/0/23, allowing SLAAC to begin on the connected PC.

Note: The exact debug ipv6 nd output may vary depending on the router model, platform, and IOS/IOS XE version in use.

### Examine the router advertisement using debug.

Locate and examine the RA messages in the debug output. To reduce output, disable IPv6 Neighbor Discovery and all other debugging after locating this message.

*Jan 21 11:36:04.079: ICMPv6-ND: (GigabitEthernet1/0/23,FE80::23:1) send RA to FF02::1

*Jan 21 11:36:04.080: ICMPv6-ND: (GigabitEthernet1/0/23,FE80::23:1) Sending RA (1800) to FF02::1

*Jan 21 11:36:04.080: ICMPv6-ND:   MTU = 1500

*Jan 21 11:36:04.080: ICMPv6-ND:   prefix 2001:DB8:ACAD:23::/64 [LA] 2592000/604800

D1# undebug all

(GigabitEthernet1/0/23,FE80::23:1) send RA to FF02::1

This indicates that D1 is sending an RA out GigabitEthernet1/0/23.

The RA is sourced (source IPv6 address) from the router’s link-local address FE80::23:1, which is always used as the source address for RAs.

The destination IPv6 address of the RA is FF02::1 is the all-nodes multicast address, meaning the RA is sent to all IPv6 hosts on the local link.

(GigabitEthernet1/0/23,FE80::23:1) Sending RA (1800) to FF02::1

This confirms the RA transmission and shows the Router Lifetime value of 1800 seconds.

This lifetime tells hosts how long (in seconds) D1 should be considered a valid default router.

MTU = 1500

The RA includes an MTU option, informing hosts that the link MTU is 1500 bytes.

Hosts will use this MTU for IPv6 communication on this interface, helping prevent fragmentation issues.

prefix 2001:DB8:ACAD:23::/64 [LA] 2592000/604800

This advertises the IPv6 prefix 2001:db8:acad:23::/64 for SLAAC.

[L] (On-Link flag) - Indicates the prefix is considered directly reachable on the local link.

[A] (Autonomous flag - Allows hosts to use this prefix to autonomously generate a global unicast address via SLAAC.

2592000 = Valid Lifetime (seconds) - How long the address remains valid.

604800 = Preferred Lifetime (seconds) - How long the address is preferred before becoming deprecated.

### Re-Examine IPv6 interface information on G 1/0/23.

After enabling IPv6 unicast routing on D1, re-examine the output of show ipv6 interface g1/0/23 to verify that the interface is now operating as an IPv6 router and actively sending Router Advertisements (RAs).

D1# show ipv6 interface g 1/0/23

GigabitEthernet1/0/23 is up, line protocol is up

IPv6 is enabled, link-local address is FE80::23:1

No Virtual link-local address(es):

Description: Link to PC

Global unicast address(es):

2001:DB8:ACAD:23::1, subnet is 2001:DB8:ACAD:23::/64

Joined group address(es):

FF02::1

FF02::2

FF02::1:FF00:1

FF02::1:FF23:1

MTU is 1500 bytes

ICMP error messages limited to one every 100 milliseconds

ICMP redirects are enabled

ICMP unreachables are sent

ND DAD is enabled, number of DAD attempts: 1

ND reachable time is 30000 milliseconds (using 30000)

ND advertised reachable time is 0 (unspecified)

ND advertised retransmit interval is 0 (unspecified)

ND router advertisements are sent every 200 seconds

ND router advertisements live for 1800 seconds

ND advertised default router preference is Medium

Hosts use stateless autoconfig for addresses.

The highlighted fields provide key confirmation of SLAAC behavior:

FF02::2 - Indicates that the interface has joined the all-routers multicast group, which is required for sending and receiving IPv6 routing-related messages, including sending Router Advertisements.

ND router advertisements are sent every 200 seconds - Shows the interval at which D1 periodically sends RAs on this interface.

ND router advertisements live for 1800 seconds - Specifies the router lifetime advertised in RAs, telling hosts how long D1 should be considered a valid default router. The host refreshes this timer every time it receives an RA.

ND advertised default router preference is Medium - Indicates the default router preference (DRP) value advertised to hosts, used when multiple routers are present on the same link. Hosts dynamically populate their Default Router List based on the source IPv6 addresses of the RA messages. The DRP (default gateway) can be high, medium (default), or low. This helps the host determine which router to use as the default gateway when it receives multiple RA messages.

Hosts use stateless autoconfig for addresses - Confirms that the RA suggests to hosts to use SLAAC to automatically generate IPv6 global unicast addresses from the advertised prefix. This confirms that the [A] (Autonomous flag) has been set to 1 (default).

### Re-Examine host IPv6 addressing after SLAAC.

Once again, examine IPv6 addressing on PC1. This output verifies that the PC has successfully received a Router Advertisement (RA) and created an IPv6 global unicast address (GUA) using the advertised prefix 2001:db8:acad:23::/64.

C:\Users\Student> ipconfig

Ethernet adapter Ethernet0:

Connection-specific DNS Suffix  : example.netacad.com

IPv6 Address. . . . . . . . . . : 2001:db8:acad:23:61a4:5032:564f:f329

Temporary IPv6 Address. . . . . : 2001:db8:acad:23:3cf5:595a:81a4:9ac8

Link-local IPv6 Address . . . . : fe80::6915:c0d:6c6d:2377%14

IPv4 Address. . . . . . . . . . : 10.23.0.3

Subnet Mask . . . . . . . . . . : 255.255.0.0

Default Gateway . . . . . . . . : fe80::23:1%14

10.23.0.1

Notes:

The IPv6 default gateway is the router’s link-local address fe80::23:1, and the %14 zone ID matches the link-local address, indicating the same Windows interface is being used.

The host operating system is using RFC 4941 (Privacy Extensions for Stateless Address Autoconfiguration), generating randomized interface identifiers instead of EUI-64 (based on the MAC address), which is evident because it does not contain ff:fe in the middle of the interface ID.

RFC 4941 also creates two addresses: a stable IPv6 address (sometimes referred to as a public or permanent address), and a temporary IPv6 address,

This temporary address is typically used when the host initiates outbound communications, which helps protect user privacy by reducing the ability to track the device over time. To further enhance privacy, the interface identifier of the temporary IPv6 address changes periodically, with the timing controlled by the host operating system’s privacy extension settings.

The stable IPv6 address can still be used when other devices need to initiate connections to this host. The interface ID can change but is designed to be stable per the prefix.

When using SLAAC, it is normal to see multiple temporary IPv6 addresses on a host at the same time. Depending on their state (for example, preferred or deprecated), older temporary addresses may remain valid for existing connections while newer temporary addresses are created for new outbound traffic, as part of the host’s IPv6 privacy extension behavior.

### Examine DNS information on PC1.

Use the ipconfig /all command on PC1, to examine the DNS information for IPv6. Notice that PC1 does not have an IPv6 for valid DNS server.

Note: If you are using Windows, you may see DNS addresses beginning with fec0:0:0:ffff::. These are Windows-generated placeholder DNS addresses used when no valid IPv6 DNS servers have been learned. These addresses are part of the deprecated IPv6 site-local address space (fec0::/10). Site-local addresses were removed from IPv6 standards long ago and should not be used on today's networks. Current server and network operating systems do not legitimately advertise these addresses.

Note: “Autoconfiguration Enabled: Yes” indicates that the host operating system is permitted to automatically configure IPv6 addressing. Whether a host actually uses SLAAC depends on the A (Autonomous) flag in Router Advertisements; if the A flag is set to 0, the host will not create a SLAAC address from that prefix even though autoconfiguration remains enabled.

C:\Users\Student> ipconfig /all

Windows IP Configuration

<output omitted>

Ethernet adapter Ethernet0:

Connection-specific DNS Suffix  . : example.netacad.com

Description . . . . . . . . . . . : Intel(R) 82574L Gigabit Network Connection

Physical Address. . . . . . . . . : 00-50-56-A8-68-59

DHCP Enabled. . . . . . . . . . . : Yes

Autoconfiguration Enabled . . . . : Yes

IPv6 Address. . . . . . . . . . . : 2001:db8:acad:23:61a4:5032:564f:f329(Preferred)

Temporary IPv6 Address. . . . . . : 2001:db8:acad:23:3cf5:595a:81a4:9ac8(Preferred)

Link-local IPv6 Address . . . . . : fe80::6915:c0d:6c6d:2377%14(Preferred)

IPv4 Address. . . . . . . . . . . : 10.23.0.3(Preferred)

Subnet Mask . . . . . . . . . . . : 255.255.0.0

Lease Obtained. . . . . . . . . . : Thursday, January 22, 2026 7:13:35 AM

Lease Expires . . . . . . . . . . : Friday, January 23, 2026 7:13:34 AM

Default Gateway . . . . . . . . . : fe80::23:1%14

10.23.0.1

DHCP Server . . . . . . . . . . . : 10.23.0.1

DHCPv6 IAID . . . . . . . . . . . : 50334761

DHCPv6 Client DUID. . . . . . . . : 00-01-00-01-31-03-ED-25-00-50-56-A8-68-59

DNS Servers . . . . . . . . . . . : fec0:0:0:ffff::1%1

fec0:0:0:ffff::2%1

fec0:0:0:ffff::3%1

NetBIOS over Tcpip. . . . . . . . : Enabled

<output omitted>

### Configure the RA to include an IPv6 DNS server address and verify with debug.

Use debug ipv6 nd to observe D1 sending the RA with the IPv6 DNS address.

D1# debug ipv6 nd

ICMP Neighbor Discovery events debugging is on

ICMP ND HA events debugging is ON

Configure the RA to include the IPv6 DNS server address 2001:db8:acad:99::99. This information is configured on the interface and is specific to the RA message sent out this interface.

D1# conf t

D1(config)# interface g 1/0/23

D1(config-if)# ipv6 nd ra dns server 2001:db8:acad:99::9999

Locate and observe that the RA now includes the DNS server address. The DNS lifetime value specifies how long (in seconds) hosts should retain the DNS server information learned from Router Advertisements before it must be refreshed

*Jan 22 16:42:59.836: ICMPv6-ND: Allocate ND config on 28

*Jan 22 16:42:59.837: ICMPv6-ND: (GigabitEthernet1/0/23) RA parameter change

*Jan 22 16:43:00.323: ICMPv6-ND: (GigabitEthernet1/0/23,FE80::23:1) send RA to FF02::1

*Jan 22 16:43:00.323: ICMPv6-ND: (GigabitEthernet1/0/23,FE80::23:1) Sending RA (1800) to FF02::1

*Jan 22 16:43:00.323: ICMPv6-ND:   MTU = 1500

*Jan 22 16:43:00.323: ICMPv6-ND:   DNS lifetime 600

*Jan 22 16:43:00.323: ICMPv6-ND: DNS Server 2001:DB8:ACAD:99::9999

*Jan 22 16:43:00.324: ICMPv6-ND:   prefix 2001:DB8:ACAD:23::/64 [LA] 2592000/604800

Disable all debug output.

D1(config-if)# end

D1# undebug all

All possible debugging has been turned off

### Re-Examine DNS information on PC1.

Using the ipconfig /all command on PC1, re-examine the DNS information for IPv6. Notice that PC1 now has the valid IPv6 address 2001:db8:acad:99::9999 as a DNS server.

C:\Users\Student> ipconfig /all

Ethernet adapter Ethernet0:

Connection-specific DNS Suffix  . : example.netacad.com

Description . . . . . . . . . . . : Intel(R) 82574L Gigabit Network Connection

Physical Address. . . . . . . . . : 00-50-56-A8-68-59

DHCP Enabled. . . . . . . . . . . : Yes

Autoconfiguration Enabled . . . . : Yes

IPv6 Address. . . . . . . . . . . : 2001:db8:acad:23:61a4:5032:564f:f329(Preferred)

Temporary IPv6 Address. . . . . . : 2001:db8:acad:23:3cf5:595a:81a4:9ac8(Preferred)

Link-local IPv6 Address . . . . . : fe80::6915:c0d:6c6d:2377%14(Preferred)

IPv4 Address. . . . . . . . . . . : 10.23.0.3(Preferred)

Subnet Mask . . . . . . . . . . . : 255.255.0.0

Lease Obtained. . . . . . . . . . : Thursday, January 22, 2026 7:13:35 AM

Lease Expires . . . . . . . . . . : Friday, January 23, 2026 7:13:34 AM

Default Gateway . . . . . . . . . : fe80::23:1%14

10.23.0.1

DHCP Server . . . . . . . . . . . : 10.23.0.1

DHCPv6 IAID . . . . . . . . . . . : 50334761

DHCPv6 Client DUID. . . . . . . . : 00-01-00-01-31-03-ED-25-00-50-56-A8-68-59

DNS Servers . . . . . . . . . . . : 2001:db8:acad:99::9999

NetBIOS over Tcpip. . . . . . . . : Enabled

<output omitted>

## Implement Stateless DHCPv6 Server on a Router

In this part, you will configure the router to act as a stateless DHCPv6 server, which builds on SLAAC rather than replacing it. Hosts will continue to use SLAAC to generate their IPv6 global unicast addresses and learn the default gateway from RAs, but DHCPv6 is introduced to provide additional, generic configuration information, such as DNS servers and domain names, that is not tied to a specific interface prefix and is often shared across multiple networks. This approach combines the simplicity of SLAAC for addressing with the flexibility of DHCPv6 for distributing common network-wide parameters.

### Configure D1 as a stateless DHCPv6 server.

Remove the IPv6 DNS server information from Router Advertisements on GigabitEthernet1/0/23, ensuring that DNS configuration will be provided by stateless DHCPv6 instead of being advertised directly in the RA.

D1(config)# interface g 1/0/23

D1(config-if)# no ipv6 nd ra dns server 2001:db8:acad:99::9999

D1(config-if)# exit

Create a stateless DHCPv6 pool that supplies non-address configuration information, such as DNS server and domain name, to IPv6 hosts. A different DNS server address is intentionally used so you can clearly verify that the end device is receiving its DNS configuration from the stateless DHCPv6 server rather than from RAs.

D1(config)# ipv6 dhcp pool IPV6-STATELESS-POOL

D1(config-dhcpv6)# dns-server 2001:db8:acad:88::8888

D1(config-dhcpv6)# domain-name www.example.com

D1(config-dhcpv6)# exit

This configuration enables stateless DHCPv6 on GigabitEthernet1/0/23 by setting the O (Other Configuration) flag to 1 in Router Advertisements, instructing hosts to use DHCPv6 to obtain additional configuration information. At the same time, the A (Autonomous) flag remains set to 1 by default, allowing hosts to continue using SLAAC to generate their IPv6 global unicast addresses while receiving DNS and other network-wide parameters from the stateless DHCPv6 serve

D1(config)# interface GigabitEthernet 1/0/23

D1(config-if)# ipv6 nd other-config-flag

D1(config-if)# ipv6 dhcp server IPV6-STATELESS-POOL

D1(config-if)# exit

### Examine the Router Advertisement using debug.

Use debug ipv6 nd to display the RAs being sent after stateless DHCPv6 has been enabled on GigabitEthernet1/0/23. This may take up to 200 seconds to appear because RAs are sent periodically at that interval unless triggered by a solicitation. Be sure to disable debug after the RA message is displayed.

D1# debug ipv6 nd

ICMP Neighbor Discovery events debugging is on

ICMP ND HA events debugging is ON

D1#

<output omitted>

*Jan 22 18:13:05.269: ICMPv6-ND: (GigabitEthernet1/0/23,FE80::23:1) send RA to FF02::1

*Jan 22 18:13:05.269: ICMPv6-ND:   Other stateful

*Jan 22 18:13:05.269: ICMPv6-ND: (GigabitEthernet1/0/23,FE80::23:1) Sending RA (1800/O) to FF02::1

*Jan 22 18:13:05.270: ICMPv6-ND:   MTU = 1500

*Jan 22 18:13:05.270: ICMPv6-ND:   prefix 2001:DB8:ACAD:23::/64 [LA] 2592000/604800

D1# undebug all

All possible debugging has been turned off

D1#

The line “Other stateful” indicates that the O (Other Configuration) flag is set to 1, suggests to the hosts to use DHCPv6 for additional configuration information.

The "O" in the notation (1800/O) shows indicates that O flag is being advertised as set to 1.

The prefix line still displays [LA], confirming that the A (Autonomous) flag remains set, allowing hosts to continue using SLAAC to generate their IPv6 global unicast addresses even while DHCPv6 is used for other configuration parameters.

### Examine IPv6 interface information on G 1/0/23.

This output re-examines GigabitEthernet1/0/23 after configuring stateless DHCPv6, confirming how IPv6 hosts are instructed to obtain their addressing and configuration information.

D1# show ipv6 interface g 1/0/23

GigabitEthernet1/0/23 is up, line protocol is up

IPv6 is enabled, link-local address is FE80::23:1

No Virtual link-local address(es):

Description: Link to PC

Global unicast address(es):

2001:DB8:ACAD:23::1, subnet is 2001:DB8:ACAD:23::/64

Joined group address(es):

FF02::1

FF02::2

FF02::1:2

FF02::1:FF00:1

FF02::1:FF23:1

FF05::1:3

MTU is 1500 bytes

ICMP error messages limited to one every 100 milliseconds

ICMP redirects are enabled

ICMP unreachables are sent

ND DAD is enabled, number of DAD attempts: 1

ND reachable time is 30000 milliseconds (using 30000)

ND advertised reachable time is 0 (unspecified)

ND advertised retransmit interval is 0 (unspecified)

ND router advertisements are sent every 200 seconds

ND router advertisements live for 1800 seconds

ND advertised default router preference is Medium

Hosts use stateless autoconfig for addresses.

Hosts use DHCP to obtain other configuration.

FF05::1:3 - This multicast address indicates that the interface has joined the site-local multicast group for all DHCPv6 servers, which is used for DHCPv6 communication beyond the local link within the same site.

Hosts use stateless autoconfig for addresses - This indicates that the A (Autonomous) flag is set in Router Advertisements, allowing hosts to continue using SLAAC to generate their IPv6 global unicast addresses.

Hosts use DHCP to obtain other configuration - This indicates that the O (Other Configuration) flag is set, instructing hosts to use DHCPv6 to obtain additional, non-address configuration (generic) information such as DNS servers and domain names.

### Verify IPv6 addressing on PC1.

This output verifies the IPv6 configuration on the PC after stateless DHCPv6 has been enabled, showing that the host continues to use SLAAC for IPv6 addressing while receiving DNS and domain information from the DHCPv6 server.

C:\Users\Student> ipconfig /all

Windows IP Configuration

Host Name . . . . . . . . . . . . : DESKTOP-3FR7RKA

Primary Dns Suffix  . . . . . . . :

Node Type . . . . . . . . . . . . : Hybrid

IP Routing Enabled. . . . . . . . : No

WINS Proxy Enabled. . . . . . . . : No

DNS Suffix Search List. . . . . . : www.example.com

Ethernet adapter Ethernet0:

Connection-specific DNS Suffix  . : example.netacad.com

Description . . . . . . . . . . . : Intel(R) 82574L Gigabit Network Connection

Physical Address. . . . . . . . . : 00-50-56-A8-68-59

DHCP Enabled. . . . . . . . . . . : Yes

Autoconfiguration Enabled . . . . : Yes

IPv6 Address. . . . . . . . . . . : 2001:db8:acad:23:61a4:5032:564f:f329(Preferred)

Temporary IPv6 Address. . . . . . : 2001:db8:acad:23:4cce:e25b:425b:4d09(Preferred)

Link-local IPv6 Address . . . . . : fe80::6915:c0d:6c6d:2377%14(Preferred)

IPv4 Address. . . . . . . . . . . : 10.23.0.3(Preferred)

Subnet Mask . . . . . . . . . . . : 255.255.0.0

Lease Obtained. . . . . . . . . . : Thursday, January 22, 2026 7:13:35 AM

Lease Expires . . . . . . . . . . : Friday, January 23, 2026 7:13:34 AM

Default Gateway . . . . . . . . . : fe80::23:1%14

10.23.0.1

DHCP Server . . . . . . . . . . . : 10.23.0.1

DHCPv6 IAID . . . . . . . . . . . : 50334761

DHCPv6 Client DUID. . . . . . . . : 00-01-00-01-31-03-ED-25-00-50-56-A8-68-59

DNS Servers . . . . . . . . . . . : 2001:db8:acad:88::8888

NetBIOS over Tcpip. . . . . . . . : Enabled

<output omitted>

The following is an explanation of the highlighted output.

DNS Suffix Search List - The appearance of www.example.com confirms that the DNS suffix search list is now being learned from the stateless DHCPv6 server, which was not available when using SLAAC alone.

IPv6 Address (stable GUA - The stable IPv6 global unicast address remains unchanged, confirming that SLAAC is still responsible for IPv6 address assignment because the A flag is still set.

Temporary IPv6 address: - temporary IPv6 address is generated using RFC 4941 privacy extensions and may change if the system is rebooted, the interface is reset, or when the operating system performs a periodic address rotation to enhance privacy. If none of these events occur, the temporary address may remain the same for some time, which is why it can appear unchanged during short lab intervals.

Link-local IPv6 Address - The link-local IPv6 address may change if the interface is reset or reinitialized. However, it typically remains the same because it is locally generated by the host and is not affected by SLAAC or DHCPv6. Although modern operating systems use a non-MAC-based interface identifier, the link-local address is designed to be stable. On modern operating systems, the link-local IPv6 address typically persists across reboots and only changes if the interface or network configuration is reset.

DNS Serverv The DNS server address 2001:db8:acad:88::8888 confirms that the host is now receiving DNS server information from the stateless DHCPv6 server rather than from Router Advertisements.

### Verify DHCPv6 pool on D1.

This output verifies the configuration of the stateless DHCPv6 pool on D1 and shows the parameters being advertised to IPv6 hosts.

D1# show ipv6 dhcp pool

DHCPv6 pool: IPV6-STATELESS-POOL

DNS server: 2001:DB8:ACAD:88::8888

Domain name: www.example.com

Active clients: 0

DNS server - This command displays the IPv6 DNS server address that the stateless DHCPv6 server provides to clients.

Domain name - This command shows the DNS domain name distributed to hosts via stateless DHCPv6.

Active clients: 0 - This command indicates that no DHCPv6 clients currently have an active lease or binding in this pool, which is expected for stateless DHCPv6 because the router does not track address assignments.

## Implement stateful DHCPv6 server on a Router

In this part, you will configure stateful DHCPv6, which operates in a manner like DHCP for IPv4 by centrally assigning IPv6 addresses and maintaining state information for each client. One important difference is that DHCPv6, whether stateless or stateful, does not provide the default gateway. Instead, IPv6 hosts always learn the default gateway from RAs, using the router’s link-local address (LLA) as the next hop. Depending on the configuration, SLAAC may still be present (via the A flag in the RA), but in stateful DHCPv6 the IPv6 address itself is assigned by the DHCPv6 server rather than generated by the host.

Many enterprises choose stateful DHCPv6 because it allows centralized control and tracking of IPv6 address assignments, making it easier to audit which device is using a specific address and to integrate IPv6 addressing with security, logging, and compliance requirements. In these environments, SLAAC is often disabled, along with specific security features, so hosts cannot self-generate IPv6 addresses, ensuring that all routable IPv6 addresses are assigned, recorded, and managed by the DHCPv6 server in a manner like traditional IPv4 enterprise networks.

### Return D1 to a non-DHCPv6 (SLAAC-only) baseline configuration.

This step removes the stateless DHCPv6 configuration from D1 so the interface is returned to a clean IPv6 baseline before configuring stateful DHCPv6.

Remove the stateless DHCPv6 pool from D1.

D1(config)# no ipv6 dhcp pool IPV6-STATELESS-POOL

Remove stateless DHCPv6 settings from the interface.

D1(config)# interface GigabitEthernet 1/0/23

D1(config-if)# no ipv6 nd other-config-flag

D1(config-if)# no ipv6 dhcp server IPV6-STATELESS-POOL

D1(config-if)# exit

Verify the interface has returned to a non-DHCPv6 configuration.

D1# show running-config | section interface GigabitEthernet1/0/23

interface GigabitEthernet1/0/23

description Link to PC

no switchport

ip address 10.23.0.1 255.255.0.0

ipv6 address FE80::23:1 link-local

ipv6 address 2001:DB8:ACAD:23::1/64

You can enter ipconfig /renew to reset PC1 ensuring a clean starting point for the stateful DHCPv6 configuration. However, resetting the host is not required in most real-world scenarios, as IPv6 hosts will automatically update their configuration when new RAs or DHCPv6 information are received.

### Configure D1 as a stateful DHCPv6 server.

Next, configure a stateful DHCPv6 pool that assigns IPv6 addresses directly to clients and maintains state information for each host, similar to DHCP for IPv4. Once again, a different DNS server address is intentionally used so you can clearly verify that the end device is receiving its IPv6 address and DNS information from the stateful DHCPv6 server, not from SLAAC or a previous configuration.

D1(config)# ipv6 dhcp pool IPV6-STATEFUL-POOL

D1(config-dhcpv6)# address prefix 2001:db8:acad:23:deed::/80

D1(config-dhcpv6)# dns-server 2001:db8:cafe:77::7777

D1(config-dhcpv6)# domain-name www.example.com

D1(config-dhcpv6)# exit

- address prefix 2001:db8:acad:23:deed::/80 - This command specifies the IPv6 prefix that the DHCPv6 server will use to assign addresses to clients. While a /64 prefix (for example, 2001:db8:acad:23::/64) is the most common and typical choice in production networks, using a /80 prefix here allows the first 16 bits of the interface identifier (deed) to be easily recognized. This makes it simple to verify that the IPv6 address was assigned by the stateful DHCPv6 server rather than generated via SLAAC.

- dns-server 2001:db8:cafe:77::7777 - This command defines the IPv6 DNS server address provided to clients, using a new value to clearly confirm that DNS information is now being supplied by the stateful DHCPv6 server.

- domain-name www.example.com - This command s pecifies the DNS domain name that will be distributed to clients via DHCPv6, providing additional network-wide configuration information.

This configuration enables stateful DHCPv6 on GigabitEthernet1/0/23 by setting the M (Managed Configuration) flag in RAs, instructing hosts to obtain their IPv6 addresses from a DHCPv6 server. The DHCPv6 server is then associated with the interface so clients can receive IPv6 addresses and other configuration information from the IPV6-STATEFUL-POOL; by default, the A (Autonomous) flag remains set to 1, even though address assignment is now managed by DHCPv6 rather than SLAAC.

D1(config)# interface GigabitEthernet 1/0/23

D1(config-if)# ipv6 nd managed-config-flag

D1(config-if)# ipv6 dhcp server IPV6-STATEFUL-POOL

D1(config-if)# end

### Examine the Router Advertisement using debug.

This debug output shows RAs being sent after enabling stateful DHCPv6 on the interface, allowing you to verify how hosts are instructed to obtain their IPv6 addressing information. Be sure to disable debug after receiving the RA.

D1# debug ipv6 nd

ICMP Neighbor Discovery events debugging is on

ICMP ND HA events debugging is ON

D1#

<output omitted>

*Jan 22 21:37:25.246: ICMPv6-ND: (GigabitEthernet1/0/23,FE80::23:1) send RA to FF02::1

*Jan 22 21:37:25.246: ICMPv6-ND:   Managed address

*Jan 22 21:37:25.246: ICMPv6-ND: (GigabitEthernet1/0/23,FE80::23:1) Sending RA (1800/M) to FF02::1

*Jan 22 21:37:25.246: ICMPv6-ND:   MTU = 1500

*Jan 22 21:37:25.246: ICMPv6-ND:   prefix 2001:DB8:ACAD:23::/64 [LA] 2592000/604800

D1# undebug all

All possible debugging has been turned off

D1#

Managed address - This line indicates that the M (Managed Configuration) flag is set in the RA, suggesting to hosts to obtain their IPv6 addresses from a stateful DHCPv6 server.

The M in (1800/M - The M confirms that the Managed flag is advertised in the RA, signaling stateful DHCPv6 address assignment (the 1800 value represents the router lifetime and can be disregarded here).

The A in [LA - The A (Autonomous) flag remains set, indicating that SLAAC is still permitted by default, even though address assignment is now managed by DHCPv6 (the L simply indicates the prefix is on-link and can be disregarded for this discussion).

### Examine IPv6 interface information on G 1/0/23.

This output verifies the IPv6 behavior of GigabitEthernet1/0/23 after configuring stateful DHCPv6, showing how hosts are instructed to obtain their IPv6 addresses.

D1# show ipv6 interface g 1/0/23

GigabitEthernet1/0/23 is up, line protocol is up

IPv6 is enabled, link-local address is FE80::23:1

No Virtual link-local address(es):

Description: Link to PC

Global unicast address(es):

2001:DB8:ACAD:23::1, subnet is 2001:DB8:ACAD:23::/64

Joined group address(es):

FF02::1

FF02::2

FF02::1:2

FF02::1:FF00:1

FF02::1:FF23:1

FF05::1:3

MTU is 1500 bytes

ICMP error messages limited to one every 100 milliseconds

ICMP redirects are enabled

ICMP unreachables are sent

ND DAD is enabled, number of DAD attempts: 1

ND reachable time is 30000 milliseconds (using 30000)

ND advertised reachable time is 0 (unspecified)

ND advertised retransmit interval is 0 (unspecified)

ND router advertisements are sent every 200 seconds

ND router advertisements live for 1800 seconds

ND advertised default router preference is Medium

Hosts use DHCP to obtain routable addresses.

FF05::1:3 - This multicast address indicates that the interface has joined the site-local multicast group for all DHCPv6 servers and relay agents, which is used for DHCPv6 communication beyond the local link.

Hosts use DHCP to obtain routable addresses - This line confirms that the M (Managed Configuration) flag is set in Router Advertisements, suggesting to hosts to obtain their global (routable) IPv6 addresses from the stateful DHCPv6 server rather than using SLAAC.

### Verifying IPv6 addressing on PC1.

This command displays the IPv6 addressing information currently configured on the PC, allowing you to verify how IPv6 addresses were obtained. The output shows two stable IPv6 global unicast addresses: one assigned by stateful DHCPv6, identified by the first 16 bits of the interface identifier set to deed, and a second address generated using SLAAC. Using a /80 prefix in the DHCPv6 address pool makes this distinction easy to see, clearly confirming which address was assigned by the stateful DHCPv6 server. SLAAC has also generated a temporary IPv6 address.

C:\Users\Student> ipconfig

Windows IP Configuration

Ethernet adapter Ethernet0:

Connection-specific DNS Suffix  . : example.netacad.com

IPv6 Address. . . . . . . . . . . : 2001:db8:acad:23:9f8f:9d75:439:928b

IPv6 Address. . . . . . . . . . . : 2001:db8:acad:23:deed:6a9c:45f:6b4a

Temporary IPv6 Address. . . . . . : 2001:db8:acad:23:1081:3ff1:9de2:e2c5

Link-local IPv6 Address . . . . . : fe80::532f:367f:dad2:abc2%14

IPv4 Address. . . . . . . . . . . : 10.23.0.2

Subnet Mask . . . . . . . . . . . : 255.255.0.0

Default Gateway . . . . . . . . . : fe80::23:1%14

10.23.0.1

### Disable SLAAC on G 1/0/23.

This step disables SLAAC on GigabitEthernet1/0/23 so that hosts can no longer generate IPv6 addresses autonomously from RAs. By preventing SLAAC, this configuration ensures that stateful DHCPv6 is the sole method used for IPv6 address assignment, making it clear that all IPv6 global unicast addresses are being obtained from the DHCPv6 server rather than being self-generated by the host.

D1(config)# interface GigabitEthernet 1/0/23

D1(config-if)# ipv6 nd prefix default no-autoconfig

Note: Instead of disabling SLAAC for all advertised prefixes, you could alternatively disable SLAAC for a specific prefix using the following command:

D1(config-if)# ipv6 nd prefix 2001:db8:acad:23::/64 no-autoconfig

This variation allows more granular control by preventing hosts from using SLAAC for a particular prefix while leaving other advertised prefixes unaffected.

### Re-Examine the Router Advertisement using Debug.

In this step, you use debug ipv6 nd to examine the Router Advertisements (RAs) sent on GigabitEthernet1/0/23 and verify that SLAAC has been disabled. The debug output confirms that the A (Autonomous) flag is no longer set, as indicated by the prefix being advertised with [L] instead of [LA], while the M (Managed Configuration) flag remains set, ensuring that hosts must obtain their IPv6 addresses exclusively through stateful DHCPv6.

D1# debug ipv6 nd

ICMP Neighbor Discovery events debugging is on

ICMP ND HA events debugging is ON

D1#

<output omitted>

*Jan 22 22:00:34.438: ICMPv6-ND: (GigabitEthernet1/0/23,FE80::23:1) send RA to FF02::1

*Jan 22 22:00:34.438: ICMPv6-ND:   Managed address

*Jan 22 22:00:34.438: ICMPv6-ND: (GigabitEthernet1/0/23,FE80::23:1) Sending RA (1800/M) to FF02::1

*Jan 22 22:00:34.438: ICMPv6-ND:   MTU = 1500

*Jan 22 22:00:34.438: ICMPv6-ND:   prefix 2001:DB8:ACAD:23::/64 [L] 2592000/604800

D1# undebug all

All possible debugging has been turned off

D1#

### Re-initialize PC1 and verify IPv6 addressing.

This output verifies that the host is now using stateful DHCPv6 only for IPv6 addressing. It has a single IPv6 global unicast address assigned from the DHCPv6 pool (identified by deed), no temporary IPv6 addresses are present because SLAAC is disabled, and the default gateway is still learned via Router Advertisements using the router’s link-local address.

C:\Users\Student> ipconfig

Windows IP Configuration

Ethernet adapter Ethernet0:

Connection-specific DNS Suffix  . : example.netacad.com

IPv6 Address. . . . . . . . . . . : 2001:db8:acad:23:deed:6a9c:45f:6b4a

Link-local IPv6 Address . . . . . : fe80::532f:367f:dad2:abc2%14

IPv4 Address. . . . . . . . . . . : 10.23.0.2

Subnet Mask . . . . . . . . . . . : 255.255.0.0

Default Gateway . . . . . . . . . : fe80::23:1%14

10.23.0.

### Check the IPv6 addressing on PC1.

This output confirms that the host is now receiving DNS server information and the DNS domain name from the stateful DHCPv6 server, as shown by the DNS server address (2001:db8:cafe:77::7777) and the DNS suffix search list (www.example.com). Because SLAAC has been disabled, this information could only have been provided by the stateful DHCPv6 configuration, verifying that the DHCPv6 server is functioning as intended.

C:\Users\Student> ipconfig /all

Windows IP Configuration

Host Name . . . . . . . . . . . . : DESKTOP-3FR7RKA

Primary Dns Suffix  . . . . . . . :

Node Type . . . . . . . . . . . . : Hybrid

IP Routing Enabled. . . . . . . . : No

WINS Proxy Enabled. . . . . . . . : No

DNS Suffix Search List. . . . . . : www.example.com

Ethernet adapter Ethernet0:

Connection-specific DNS Suffix  . : example.netacad.com

Description . . . . . . . . . . . : Intel(R) 82574L Gigabit Network Connection

Physical Address. . . . . . . . . : 00-50-56-A8-68-59

DHCP Enabled. . . . . . . . . . . : Yes

Autoconfiguration Enabled . . . . : Yes

IPv6 Address. . . . . . . . . . . : 2001:db8:acad:23:deed:6a9c:45f:6b4a (Preferred)

Lease Obtained. . . . . . . . . . : Thursday, January 22, 2026 1:16:19 PM

Lease Expires . . . . . . . . . . : Saturday, January 24, 2026 12:31:37 PM

Link-local IPv6 Address . . . . . : fe80::532f:367f:dad2:abc2%14 (Preferred)

IPv4 Address. . . . . . . . . . . : 10.23.0.2 (Preferred)

Subnet Mask . . . . . . . . . . . : 255.255.0.0

Lease Obtained. . . . . . . . . . : Thursday, January 22, 2026 1:16:18 PM

Lease Expires . . . . . . . . . . : Friday, January 23, 2026 1:16:18 PM

Default Gateway . . . . . . . . . : fe80::23:1%14

10.23.0.1

DHCP Server . . . . . . . . . . . : 10.23.0.1

DHCPv6 IAID . . . . . . . . . . . : 50334761

DHCPv6 Client DUID. . . . . . . . : 00-01-00-01-31-03-ED-25-00-50-56-A8-68-59

DNS Servers . . . . . . . . . . . : 2001:db8:cafe:77::7777

NetBIOS over Tcpip. . . . . . . . : Enabled

Connection-specific DNS Suffix Search List :

www.example.com

<output omitted>

### Verify stateful DHCPv6 activity on D1.

This command displays the current status of the stateful DHCPv6 pool, allowing you to verify the configured address prefix, DNS information, and client usage. The output confirms that the IPV6-STATEFUL-POOL is using the specified /80 address prefix, shows that one IPv6 address is currently allocated and in use, and indicates one active DHCPv6 client, verifying that stateful DHCPv6 address assignment is operating correctly.

D1# show ipv6 dhcp pool

DHCPv6 pool: IPV6-STATEFUL-POOL

Address allocation prefix: 2001:DB8:ACAD:23:DEED::/80 valid 172800 preferred 86400 (1 in use, 0 conflicts)

DNS server: 2001:DB8:CAFE:77::7777

Domain name: www.example.com

Active clients: 1

This command displays the stateful DHCPv6 bindings, allowing you to verify which IPv6 address has been assigned to a specific client and how long that address remains valid. The output shows the client’s link-local address (FE80::532F:367F:DAD2:ABC2) used for DHCPv6 communication, the IPv6 address assigned from the DHCPv6 pool (2001:DB8:ACAD:23:DEED:6A9C:45F:6B4A), and the associated preferred and valid lifetimes, confirming that the address is actively leased to the client and will expire at the indicated time if not renewed.

D1# show ipv6 dhcp binding

Client: FE80::532F:367F:DAD2:ABC2

DUID: 000100013103ED25005056A86859

Username : unassigned

VRF : default

IA NA: IA ID 0x03000C29, T1 0, T2 0

Address: 2001:DB8:ACAD:23:DEED:6A9C:45F:6B4A

preferred lifetime 86400, valid lifetime 172800

expires at Jan 24 2026 09:19 PM (169097 seconds)

On Cisco IOS XE, the default stateful DHCPv6 lease timers are:

Preferred lifetime - 86,400 seconds (24 hours)

Valid lifetime -  172,800 seconds (48 hours)

The remaining lifetime of the DHCPv6 lease is the Valid lifetime of 48 hours expressed in two ways:

expires at Jan 24 2026 09:19 PM is the wall-clock time when the valid lifetime will end if the lease is not renewed

(169097 seconds) is the remaining number of seconds until that expiration moment, counted from when the command was issued.

This output provides a summary of DHCPv6 message activity on D1, confirming that stateful DHCPv6 exchanges are occurring successfully.

Messages received / sent – Shows that DHCPv6 communication is active, with no messages discarded or failed.

SOLICIT / ADVERTISE – Indicates the client discovered the DHCPv6 server and the server responded, starting the address assignment process.

REQUEST / REPLY – Confirms the client requested an IPv6 address and the server successfully assigned it.

CONFIRM – Shows the client verified that its assigned IPv6 address is still valid on the link, which commonly occurs after a restart or interface reset.

Overall, this output confirms normal and successful stateful DHCPv6 operation with no errors

D1# show ipv6 dhcp statistics

Messages received                4

Messages sent                    4

Messages discarded               0

Messages could not be sent       0

Messages                         Received

SOLICIT                          1

REQUEST                          1

CONFIRM                          2

Messages                         Sent

ADVERTISE                        1

REPLY                            3

of document

# Device Configs - Final

# Switch D1

D1# show run brief

Building configuration...

Current configuration : 5530 bytes

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform punt-keepalive disable-kernel-core

hostname D1

vrf definition Mgmt-vrf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

switch 1 provision c9300l-24t-4g

no ip domain lookup

ip dhcp pool LAN

network 10.23.0.0 255.255.0.0

default-router 10.23.0.1

domain-name example.netacad.com

login on-success log

ipv6 nd cache expire refresh

ipv6 unicast-routing

ipv6 dhcp pool IPV6-STATEFUL-POOL

address prefix 2001:DB8:ACAD:23:DEED::/80

dns-server 2001:DB8:CAFE:77::7777

domain-name www.example.com

crypto pki trustpoint TP-self-signed-1929813547

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-1929813547

revocation-check none

rsakeypair TP-self-signed-1929813547

hash sha512

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

hash sha512

crypto pki certificate chain TP-self-signed-1929813547

certificate self-signed 01

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

license boot level network-advantage addon dna-advantage

memory free low-watermark processor 130178

diagnostic bootup level minimal

spanning-tree mode rapid-pvst

spanning-tree extend system-id

enable secret 9 $9$e26G7zmh8eJZkE$sqhsQF1TpyytrYD2f74J9egNg6f6kIKHDVW/9ls4AiU

redundancy

mode sso

crypto engine compliance shield disable

transceiver type all

monitoring

class-map match-any system-cpp-police-ewlc-control

description EWLC Control

class-map match-any system-cpp-police-topology-control

description Topology control

class-map match-any system-cpp-police-sw-forward

description Sw forwarding, L2 LVX data packets, LOGGING, Transit Traffic

class-map match-any system-cpp-default

description EWLC Data, Inter FED Traffic

class-map match-any system-cpp-police-sys-data

description Openflow, Exception, EGR Exception, NFL Sampled Data, RPF Failed

class-map match-any system-cpp-police-punt-webauth

description Punt Webauth

class-map match-any system-cpp-police-l2lvx-control

description L2 LVX control packets

class-map match-any system-cpp-police-forus

description Forus Address resolution and Forus traffic

class-map match-any system-cpp-police-multicast-end-station

description MCAST END STATION

class-map match-any system-cpp-police-high-rate-app

description High Rate Applications

class-map match-any system-cpp-police-multicast

description MCAST Data

class-map match-any system-cpp-police-l2-control

description L2 control

class-map match-any system-cpp-police-dot1x-auth

description DOT1X Auth

class-map match-any system-cpp-police-data

description ICMP redirect, ICMP_GEN and BROADCAST

class-map match-any system-cpp-police-stackwise-virt-control

description Stackwise Virtual OOB

class-map match-any non-client-nrt-class

class-map match-any system-cpp-police-routing-control

description Routing control and Low Latency

class-map match-any system-cpp-police-protocol-snooping

description Protocol snooping

class-map match-any system-cpp-police-dhcp-snooping

description DHCP snooping

class-map match-any system-cpp-police-ios-routing

description L2 control, Topology control, Routing control, Low Latency

class-map match-any system-cpp-police-system-critical

description System Critical and Gold Pkt

class-map match-any system-cpp-police-ios-feature

description ICMPGEN,BROADCAST,ICMP,L2LVXCntrl,ProtoSnoop,PuntWebauth,MCASTData,Transit,DOT1XAuth,Swfwd,LOGGING,L2LVXData,ForusTraffic,ForusARP,McastEndStn,Openflow,Exception,EGRExcption,NflSampled,RpfFailed

policy-map system-cpp-policy

interface GigabitEthernet0/0

vrf forwarding Mgmt-vrf

no ip address

shutdown

negotiation auto

interface GigabitEthernet1/0/1

interface GigabitEthernet1/0/2

interface GigabitEthernet1/0/3

interface GigabitEthernet1/0/4

interface GigabitEthernet1/0/5

interface GigabitEthernet1/0/6

interface GigabitEthernet1/0/7

interface GigabitEthernet1/0/8

interface GigabitEthernet1/0/9

interface GigabitEthernet1/0/10

interface GigabitEthernet1/0/11

interface GigabitEthernet1/0/12

interface GigabitEthernet1/0/13

interface GigabitEthernet1/0/14

interface GigabitEthernet1/0/15

interface GigabitEthernet1/0/16

interface GigabitEthernet1/0/17

interface GigabitEthernet1/0/18

interface GigabitEthernet1/0/19

interface GigabitEthernet1/0/20

interface GigabitEthernet1/0/21

interface GigabitEthernet1/0/22

interface GigabitEthernet1/0/23

description Link to PC

no switchport

ip address 10.23.0.1 255.255.0.0

ipv6 address FE80::23:1 link-local

ipv6 address 2001:DB8:ACAD:23::1/64

ipv6 nd prefix default 2592000 604800 no-autoconfig

ipv6 nd managed-config-flag

ipv6 dhcp server IPV6-STATEFUL-POOL

interface GigabitEthernet1/0/24

interface GigabitEthernet1/1/1

interface GigabitEthernet1/1/2

interface GigabitEthernet1/1/3

interface GigabitEthernet1/1/4

interface AppGigabitEthernet1/0/1

interface Vlan1

no ip address

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip ssh bulk-mode 131072

control-plane

service-policy input system-cpp-policy

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line vty 0 4

login

transport input ssh

line vty 5 31

login

transport input ssh

end

D1#