# Lab - Troubleshoot IPv4 ACLs (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

# Topology

# Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| R1 | G0/0/1 | 192.0.0.1 | 255.255.255.0 |
| R1 | S0/1/0 | 209.165.200.1 | 255.255.255.0 |
| R1 | S0/1/1 | 209.165.201.1 | 255.255.255.0 |
| R1 | Loopback0 | 209.165.226.1 | 255.255.255.0 |
| R3 | G0/0/1.16 | 10.0.16.1 | 255.255.255.0 |
| R3 | G0/0/1.27 | 10.0.27.1 | 255.255.255.0 |
| R3 | S0/1/0 | 209.165.200.2 | 255.255.255.0 |
| R3 | S0/1/1 | 209.165.201.2 | 255.255.255.0 |
| R3 | Loopback0 | 209.165.227.1 | 255.255.255.0 |
| R3 | Loopback1 | 209.165.228.1 | 255.255.255.0 |
| D1 | G1/0/11 | 192.0.0.2 | 255.255.255.0 |
| D1 | VLAN 11 | 209.165.224.1 | 255.255.255.0 |
| D1 | VLAN 12 | 209.165.225.1 | 255.255.255.0 |
| PC1 | NIC | DHCP |  |
| PC2 | NIC | DHCP |  |
| PC3 | NIC | DHCP |  |
| PC4 | NIC | DHCP |  |

# Objectives

- Troubleshoot network issues related to the configuration and operation of ACLs for IPv4.

# Background / Scenario

- In this topology, R1 and D1 are OSPF neighbors, while R1 and R3 are BGP neighbors. Switch D1 provides inter-VLAN routing for two subnets. R3 provides inter-VLAN routing for two subnets, and switch D2 provides connectivity for the two VLANs supporting those subnets. The BGP relationship between R1 and R3 is established using EBGP multihop between the router’s respective Loopback 0 interfaces. You will be loading configurations with intentional errors onto the network. Your tasks are to FIND the error(s), document your findings and the command(s) or method(s) used to fix them, FIX the issue(s) presented here and then test the network to ensure both of the following conditions are met:

the complaint received in the ticket is resolved

full reachability is restored

Note: The routers used with CCNP hands-on labs are Cisco 8200L with Cisco IOS XE Release 17.12.05c (universalk9 image). The switches used in the labs are Cisco Catalyst 9300L with Cisco IOS XE Release 17.12.05 (universalk9 image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs.

Note: Make sure that the devices have been erased and have no startup configurations. If you are unsure, contact your instructor.

Instructor Note: Refer to the Instructor Lab Manual for the procedures to initialize and reload devices.

# Required Resources

2 Routers (Cisco 8200L with Cisco IOS XE Release 17.12.05c universal image or comparable)

2 Switches (Cisco 9300L with Cisco IOS XE Release 17.12.05 universal image or comparable)

4 PCs (Choice of operating system with terminal emulation program installed)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet and serial cables as shown in the topology

# Instructions

## Trouble Ticket 32.1.1.1

Scenario:

A Sunday night change window was used by an external security consultant to bring R1 and R3 into alignment with RFC 1918 policies. The work included updates to ACLs and NAT policy on the edge path between the campus and the WAN. By 06:45 Monday morning, Operations reports that a business-critical workflow between PC1 in VLAN 11 and PC3 in VLAN 27 is failing. Pings time out and the application no longer connects. OSPF adjacencies look stable and the BGP session between R1 and R3 is established, but users are blocked from processing orders. Your task is to investigate and restore the PC1-PC3 communication while preserving the intended security posture from the change.

- Be sure you are using devices that have been connected and prepared by your instructor. Use the commands listed below to load the configuration files for this trouble ticket:

Instructor Note: Commands for uploading the configuration are provided at the end of this document.

| Device | Command |
| --- | --- |
| R1 | copy flash:/enarsi/32.1.1.1-r1-config.txt running-config |
| R3 | copy flash:/enarsi/32.1.1.1-r3-config.txt running-config |
| D1 | copy flash:/enarsi/32.1.1.1-d1-config.txt running-config |
| D2 | copy flash:/enarsi/32.1.1.1-d2-config.txt running-config |

- PCs 1, 2, 3, and 4 receive their addressing via DHCP for IPv4.

- PC3 should be able to ping PC1. However, PC1 should not be able to ping PC3.

- Passwords on all devices are cisco12345. If a username is required, use admin.

- When you have fixed the ticket, change the MOTD on EACH DEVICE using the following command:

banner motd # This is $(hostname) FIXED from ticket <ticket number> #

- Then save the configuration by issuing the wri command (on each device).

- Inform your instructor that you are ready for the next ticket.

- After the instructor approves your solution for this ticket, issue the reset.now privileged EXEC command. This script will clear your configurations and reload the devices.

Instructor Notes:

Although the trouble ticket does not mention this, the student may also notice that PC2 and PC4 cannot communicate. The ticket came in from the users of the connection between PC1 and PC3. Therefore, that is the focus. Fixing the issue between PC1 and PC3 will also restore full connectivity between all PCs. This trouble ticket contains 1 intentional error. The NAT ACL on R3, named NAT-ELIGIBLE, is configured with the wrong wildcard mask, causing hosts in VLAN 16 and VLAN 27 not to qualify for NAT. Because those networks are not advertised in BGP and blocked at R1, PC3 cannot communicate with PC1.

The student may choose one of two methods to correct the error. They may either completely remove and replace the NAT-ELIGIBLE ACL, or they may edit the NAT-ELIGIBLE ACL and replace the errored statement.

The commands the use the latter method to fix these errors should be:

R3(config)# ip access-list standard NAT-ELIGIBLE

R3(config-std-nacl)# no 10

R3(config-std-nacl)# 10 permit 10.0.0.0 0.0.255.255

R3(config-std-nacl)# end

## Trouble Ticket 32.1.1.2

Scenario:

Late Friday evening, a junior network administrator “tightened” ACLs to reduce unnecessary exposure and lateral movement. Early Monday, Finance users on VLAN 12 (PC2 via DHCP) report they cannot reach two internal services at 209.165.227.1 and 209.165.228.1, which host reporting and licensing endpoints on R3. Users on VLAN 11 can still reach these addresses, suggesting a policy impact specific to VLAN 12. You need to restore PC2’s reachability to those loopback addresses without undermining the hardening intent.

- Use the commands listed below to load the configuration files for this trouble ticket:

Instructor Note: Commands for uploading the configuration are provided at the end of this document.

| Device | Command |
| --- | --- |
| R1 | copy flash:/enarsi/32.1.1.2-r1-config.txt running-config |
| R3 | copy flash:/enarsi/32.1.1.2-r3-config.txt running-config |
| D1 | copy flash:/enarsi/32.1.1.2-d1-config.txt running-config |
| D2 | copy flash:/enarsi/32.1.1.2-d2-config.txt running-config |

- PCs 1, 2, 3, and 4 receive their addressing via DHCP for IPv4.

- PC2 should be able to ping 209.165.226.1 and 209.165.227.1.

- Passwords on all devices are cisco12345. If a username is required, use admin.

- When you have fixed the ticket, change the MOTD on EACH DEVICE using the following command:

banner motd # This is $(hostname) FIXED from ticket <ticket number> #

- Then save the configuration by issuing the wri command (on each device).

- Inform your instructor that you are ready for the next ticket.

- After the instructor approves your solution for this ticket, issue the reset.now privileged EXEC command. This script will clear your configurations and reload the devices.

Instructor Notes:

This trouble ticket contains 1 intentional error. An ACL called IPCHECK has been added inbound on R1. The wildcard mask does not include the VLAN 12 network, so PC2 and VLAN 12 are blocked.

The student may choose one of two methods to correct the error. They may either completely remove and replace the IPCHECK ACL, or they may edit the IPCHECK ACL and replace the errored statement, or add an additional statement to permit the VLAN 12 network.

The commands to replace the errored statement should be:

R1(config)# ip access-list extended IPCHECK

R1(config-std-nacl)# no 10

R1(config-std-nacl)# 10 permit ip 209.165.224.0 0.0.1.255 any

R1(config-std-nacl)# exit

The commands to add permission for VLAN 12 should be:

R1(config)# ip access-list extended IPCHECK

R1(config-std-nacl)# 15 permit ip 209.165.225.0 0.0.0.255 any

R1(config-std-nacl)# end

## Trouble Ticket 32.1.1.3

Scenario:

Over the weekend, a junior administrator attempted to enforce time-based restrictions on remote access as part of security improvements related to the external BGP peer in AS 181035. At 08:00 Monday, NOC staff report they cannot Telnet to R1 or D1 to perform routine checks and incident response, impacting morning operations. Console access is limited across sites, so daytime remote management must be restored quickly while keeping the intent of off-hours restrictions. Your job is to identify the access control misconfiguration and re-establish appropriate remote access during business hours.

- Use the commands listed below to load the configuration files for this trouble ticket:

Instructor Note: Commands for uploading the configuration are provided at the end of this document.

| Device | Command |
| --- | --- |
| R1 | copy flash:/enarsi/32.1.1.3-r1-config.txt running-config |
| R3 | copy flash:/enarsi/32.1.1.3-r3-config.txt running-config |
| D1 | copy flash:/enarsi/32.1.1.3-d1-config.txt running-config |
| D2 | copy flash:/enarsi/32.1.1.3-d2-config.txt running-config |

- PCs 1, 2, 3, and 4 receive their addressing via DHCP for IPv4.

- PC1 should be able to Telnet into R1 and D1.

- Passwords on all devices are cisco12345. If a username is required, use admin.

- When you have fixed the ticket, change the MOTD on EACH DEVICE using the following command:

banner motd # This is $(hostname) FIXED from ticket <ticket number> #

- Then save the configuration by issuing the wri command (on each device).

- Inform your instructor that you are ready for the next ticket.

- After the instructor approves your solution for this ticket, issue the reset.now privileged EXEC command. This script will clear your configurations and reload the devices.

Instructor Notes:

This trouble ticket contains 1 intentional error that is repeated in two places. The junior administrator did not understand the commands to use for adding a time-range to an ACL, and subsequently made the time-ranges extremely restrictive. He thought he was denying access between midnight and five a.m., when the result of the configuration was to allow access between midnight and five a.m. only.

To correct the error, the student must completely remove and replace the time range REMOTE-CONTROL and then reassign the VTY-CONTROL access-list to the vty lines.

The commands to replace the errored time-range should be:

R1(config)# no time-range REMOTE-CONTROL

R1(config)# time-range REMOTE-CONTROL

R1(config-time-range)# periodic weekdays 05:00 to 23:59

R1(config-time-range)# periodic weekend 05:00 to 23:59

R1(config-time-range)# exit

R1(config)# line vty 0 4

R1(config-line)# access-class VTY-CONTROL in

R1(config-line)# end

D1(config)# no time-range REMOTE-CONTROL

D1(config)# time-range REMOTE-CONTROL

D1(config-time-range)# periodic weekdays 05:00 to 23:59

D1(config-time-range)# periodic weekend 05:00 to 23:59

D1(config-time-range)# exit

D1(config)# line vty 0 4

D1(config-line)# access-class VTY-CONTROL in

D1(config-line)# end

End of document

Uploading Configuration Files

Use the commands below to create the configuration files on the lab devices for each trouble ticket in this lab. The TCL script commands help create and copy the configurations. However, the configuration commands could also be copied and pasted directly into global config mode on each device. Simply remove the TCL script commands, enter the enable and configure t commands on the device, and copy and paste the configuration commands.

Important: The device requires a folder in flash named enarsi. Use the dir command to verify. If the folder is missing, then create it using the mkdir flash:/enarsi privileged exec command. For all switches, make sure the vlan.dat file is set to the default. Use the delete vlan.dat privileged exec command, if necessary.

Reset scripts

These TCL scripts will completely clear and reload the device in preparation for the next ticket. Copy and paste the appropriate script to the appropriate device.

Router Reset Script

tclsh

puts [ open "flash:/enarsi/reset.tcl" w+ ] {

typeahead "\n"

copy running-config startup-config

typeahead "\n"

erase startup-config

puts "Reloading the router"

typeahead "\n"

reload

}

tclquit

D1/D2 (Cisco 3650) Reset Script - The default 3650 SDM template supports IPv6, so it is not set by this script.

tclsh

puts [ open "flash:/enarsi/reset.tcl" w+ ] {

typeahead "\n"

copy running-config startup-config

typeahead "\n"

erase startup-config

delete /force vlan.dat

puts "Reloading the switch"

typeahead "\n"

reload

}

tclquit

R1 Configuration File Scripts

! R1 - Trouble Ticket # 1

tclsh

puts [ open "flash:/enarsi/32.1.1.1-r1-config.txt" w+ ] {

hostname R1

no ip domain lookup

banner motd # This is R1, Trouble Ticket 32.1.1.1 #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

ip access-list extended IPCHECK

permit ip 209.165.224.0 0.0.1.255 any

permit ip 192.0.0.0 0.0.0.255 any

exit

ip access-list standard BOGON-MARTIAN

deny 10.0.0.0 0.255.255.255

deny 172.16.0.0 0.31.255.255

deny 192.168.0.0 0.0.255.255

deny 127.0.0.0 0.255.255.255

deny 192.0.0.0 0.0.0.255

deny 209.165.224.0 0.0.0.255

deny 209.165.225.0 0.0.0.255

deny host 209.165.226.1

permit any

exit

interface g0/0/1

ip address 192.0.0.1 255.255.255.0

ip access-group IPCHECK in

no shutdown

exit

interface s0/1/0

ip address 209.165.200.1 255.255.255.0

ip access-group BOGON-MARTIAN in

no shutdown

exit

interface s0/1/1

ip address 209.165.201.1 255.255.255.0

ip access-group BOGON-MARTIAN in

no shutdown

exit

interface loopback 0

ip address 209.165.226.1 255.255.255.255

no shutdown

exit

ip route 209.165.227.1 255.255.255.255 s0/1/0

ip route 209.165.227.1 255.255.255.255 s0/1/1

router ospf 1

router-id 1.1.1.1

network 192.0.0.0 0.0.0.255 area 0

default-information originate always

exit

router bgp 181035

bgp router-id 209.165.226.1

neighbor 209.165.227.1 remote-as 39457

neighbor 209.165.227.1 update-source loopback 0

neighbor 209.165.227.1 ebgp-multihop 3

network 209.165.224.0 mask 255.255.255.0

network 209.165.225.0 mask 255.255.255.0

network 192.0.0.0 mask 255.255.255.0

exit

time-range REMOTE-CONTROL

periodic weekdays 05:00 to 23:59

periodic weekend 05:00 to 23:59

exit

ip access-list extended VTY-CONTROL

permit ip 209.165.224.0 0.0.0.255 any time-range REMOTE-CONTROL

deny ip any any log

exit

line con 0

logging synchronous

exec-timeout 0 0

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

access-class VTY-CONTROL in

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

! R1 - Trouble Ticket # 2

tclsh

puts [ open "flash:/enarsi/32.1.1.2-r1-config.txt" w+ ] {

hostname R1

no ip domain lookup

banner motd # This is R1,  Trouble Ticket 32.1.1.2  #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

ip access-list extended IPCHECK

permit ip 209.165.224.0 0.0.0.255 any

permit ip 192.0.0.0 0.0.0.255 any

exit

ip access-list standard BOGON-MARTIAN

deny 10.0.0.0 0.255.255.255

deny 172.16.0.0 0.31.255.255

deny 192.168.0.0 0.0.255.255

deny 127.0.0.0 0.255.255.255

deny 192.0.0.0 0.0.0.255

deny 209.165.224.0 0.0.0.255

deny 209.165.225.0 0.0.0.255

deny host 209.165.226.1

permit any

exit

interface g0/0/1

ip address 192.0.0.1 255.255.255.0

ip access-group IPCHECK in

no shutdown

exit

interface s0/1/0

ip address 209.165.200.1 255.255.255.0

ip access-group BOGON-MARTIAN in

no shutdown

exit

interface s0/1/1

ip address 209.165.201.1 255.255.255.0

ip access-group BOGON-MARTIAN in

no shutdown

exit

interface loopback 0

ip address 209.165.226.1 255.255.255.255

no shutdown

exit

ip route 209.165.227.1 255.255.255.255 s0/1/0

ip route 209.165.227.1 255.255.255.255 s0/1/1

router ospf 1

router-id 1.1.1.1

network 192.0.0.0 0.0.0.255 area 0

default-information originate always

exit

router bgp 181035

bgp router-id 209.165.226.1

neighbor 209.165.227.1 remote-as 39457

neighbor 209.165.227.1 update-source loopback 0

neighbor 209.165.227.1 ebgp-multihop 3

network 209.165.224.0 mask 255.255.255.0

network 209.165.225.0 mask 255.255.255.0

network 192.0.0.0 mask 255.255.255.0

exit

time-range REMOTE-CONTROL

periodic weekdays 05:00 to 23:59

periodic weekend 05:00 to 23:59

exit

ip access-list extended VTY-CONTROL

permit ip 209.165.224.0 0.0.0.255 any time-range REMOTE-CONTROL

deny ip any any log

exit

line con 0

logging synchronous

exec-timeout 0 0

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

access-class VTY-CONTROL in

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

! R1 - Trouble Ticket # 3

tclsh

puts [ open "flash:/enarsi/32.1.1.3-r1-config.txt" w+ ] {

hostname R1

no ip domain lookup

banner motd # This is R1, Trouble Ticket 32.1.1.3 #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

ip access-list extended IPCHECK

permit ip 209.165.224.0 0.0.1.255 any

permit ip 192.0.0.0 0.0.0.255 any

exit

ip access-list standard BOGON-MARTIAN

deny 10.0.0.0 0.255.255.255

deny 172.16.0.0 0.31.255.255

deny 192.168.0.0 0.0.255.255

deny 127.0.0.0 0.255.255.255

deny 192.0.0.0 0.0.0.255

deny 209.165.224.0 0.0.0.255

deny 209.165.225.0 0.0.0.255

deny host 209.165.226.1

permit any

exit

interface g0/0/1

ip address 192.0.0.1 255.255.255.0

ip access-group IPCHECK in

no shutdown

exit

interface s0/1/0

ip address 209.165.200.1 255.255.255.0

ip access-group BOGON-MARTIAN in

no shutdown

exit

interface s0/1/1

ip address 209.165.201.1 255.255.255.0

ip access-group BOGON-MARTIAN in

no shutdown

exit

interface loopback 0

ip address 209.165.226.1 255.255.255.255

no shutdown

exit

ip route 209.165.227.1 255.255.255.255 s0/1/0

ip route 209.165.227.1 255.255.255.255 s0/1/1

router ospf 1

router-id 1.1.1.1

network 192.0.0.0 0.0.0.255 area 0

default-information originate always

exit

router bgp 181035

bgp router-id 209.165.226.1

neighbor 209.165.227.1 remote-as 39457

neighbor 209.165.227.1 update-source loopback 0

neighbor 209.165.227.1 ebgp-multihop 3

network 209.165.224.0 mask 255.255.255.0

network 209.165.225.0 mask 255.255.255.0

network 192.0.0.0 mask 255.255.255.0

exit

time-range REMOTE-CONTROL

periodic weekdays 00:00 to 05:00

periodic weekend 00:00 to 05:00

exit

ip access-list extended VTY-CONTROL

permit ip 209.165.224.0 0.0.0.255 any time-range REMOTE-CONTROL

deny ip any any log

exit

line con 0

logging synchronous

exec-timeout 0 0

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

access-class VTY-CONTROL in

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

R3 Configuration File Scripts

! R3 - Trouble Ticket # 1

tclsh

puts [ open "flash:/enarsi/32.1.1.1-r3-config.txt" w+ ] {

hostname R3

no ip domain lookup

banner motd # This is R3, Trouble Ticket 32.1.1.1  #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

ip access-list standard BOGON-MARTIAN

deny 10.0.0.0 0.255.255.255

deny 172.16.0.0 0.31.255.255

deny 192.168.0.0 0.0.255.255

deny 127.0.0.0 0.255.255.255

deny host 209.165.227.1

deny host 209.165.228.1

permit any

exit

interface g0/0/1

no ip address

no shutdown

interface g0/0/1.16

encapsulation dot1q 16

ip address 10.0.16.1 255.255.255.0

no shutdown

exit

interface g0/0/1.27

encapsulation dot1q 27

ip address 10.0.27.1 255.255.255.0

no shutdown

exit

interface s0/1/0

ip address 209.165.200.2 255.255.255.0

ip access-group BOGON-MARTIAN in

no shutdown

interface s0/1/1

ip address 209.165.201.2 255.255.255.0

ip access-group BOGON-MARTIAN in

no shutdown

interface loopback 0

ip address 209.165.227.1 255.255.255.255

no shutdown

exit

interface loopback 1

ip address 209.165.228.1 255.255.255.255

no shutdown

exit

ip route 209.165.226.1 255.255.255.255 s0/1/0

ip route 209.165.226.1 255.255.255.255 s0/1/1

router bgp 39457

bgp router-id 209.165.227.1

neighbor 209.165.226.1 remote-as 181035

neighbor 209.165.226.1 update-source loopback 0

neighbor 209.165.226.1 ebgp-multihop 3

network 209.165.228.1 mask 255.255.255.255

exit

ip access-list standard NAT-ELIGIBLE

permit 10.0.0.0 0.0.0.255

deny any log

exit

ip nat inside source list NAT-ELIGIBLE interface loopback 1 overload

ip dhcp excluded-address 10.0.16.1 10.0.16.5

ip dhcp excluded-address 10.0.27.1 10.0.27.5

ip dhcp pool NAT16

network 10.0.16.0 255.255.255.0

default-router 10.0.16.1

exit

ip dhcp pool NAT27

network 10.0.27.0 255.255.255.0

default-router 10.0.27.1

exit

interface s0/1/0

ip nat outside

exit

interface s0/1/1

ip nat outside

exit

interface g0/0/1.16

ip nat inside

exit

interface g0/0/1.27

ip nat inside

exit

line con 0

logging synchronous

exec-timeout 0 0

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

! R3 - Trouble Ticket # 2

tclsh

puts [ open "flash:/enarsi/32.1.1.2-r3-config.txt" w+ ] {

hostname R3

no ip domain lookup

banner motd # This is R3, Trouble Ticket 32.1.1.2 #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

ip access-list standard BOGON-MARTIAN

deny 10.0.0.0 0.255.255.255

deny 172.16.0.0 0.31.255.255

deny 192.168.0.0 0.0.255.255

deny 127.0.0.0 0.255.255.255

deny host 209.165.227.1

deny host 209.165.228.1

permit any

exit

interface g0/0/1

no ip address

no shutdown

interface g0/0/1.16

encapsulation dot1q 16

ip address 10.0.16.1 255.255.255.0

no shutdown

exit

interface g0/0/1.27

encapsulation dot1q 27

ip address 10.0.27.1 255.255.255.0

no shutdown

exit

interface s0/1/0

ip address 209.165.200.2 255.255.255.0

ip access-group BOGON-MARTIAN in

no shutdown

interface s0/1/1

ip address 209.165.201.2 255.255.255.0

ip access-group BOGON-MARTIAN in

no shutdown

interface loopback 0

ip address 209.165.227.1 255.255.255.255

no shutdown

exit

interface loopback 1

ip address 209.165.228.1 255.255.255.255

no shutdown

exit

ip route 209.165.226.1 255.255.255.255 s0/1/0

ip route 209.165.226.1 255.255.255.255 s0/1/1

router bgp 39457

bgp router-id 209.165.227.1

neighbor 209.165.226.1 remote-as 181035

neighbor 209.165.226.1 update-source loopback 0

neighbor 209.165.226.1 ebgp-multihop 3

network 209.165.228.1 mask 255.255.255.255

exit

ip access-list standard NAT-ELIGIBLE

permit 10.0.0.0 0.255.255.255

deny any log

exit

ip nat inside source list NAT-ELIGIBLE interface loopback 1 overload

ip dhcp excluded-address 10.0.16.1 10.0.16.5

ip dhcp excluded-address 10.0.27.1 10.0.27.5

ip dhcp pool NAT16

network 10.0.16.0 255.255.255.0

default-router 10.0.16.1

exit

ip dhcp pool NAT27

network 10.0.27.0 255.255.255.0

default-router 10.0.27.1

exit

interface s0/1/0

ip nat outside

exit

interface s0/1/1

ip nat outside

exit

interface g0/0/1.16

ip nat inside

exit

interface g0/0/1.27

ip nat inside

exit

line con 0

logging synchronous

exec-timeout 0 0

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

! R3 - Trouble Ticket # 3

tclsh

puts [ open "flash:/enarsi/32.1.1.3-r3-config.txt" w+ ] {

hostname R3

no ip domain lookup

banner motd # This is R3, Trouble Ticket 32.1.1.3 #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

ip access-list standard BOGON-MARTIAN

deny 10.0.0.0 0.255.255.255

deny 172.16.0.0 0.31.255.255

deny 192.168.0.0 0.0.255.255

deny 127.0.0.0 0.255.255.255

deny host 209.165.227.1

deny host 209.165.228.1

permit any

exit

interface g0/0/1

no ip address

no shutdown

interface g0/0/1.16

encapsulation dot1q 16

ip address 10.0.16.1 255.255.255.0

no shutdown

exit

interface g0/0/1.27

encapsulation dot1q 27

ip address 10.0.27.1 255.255.255.0

no shutdown

exit

interface s0/1/0

ip address 209.165.200.2 255.255.255.0

ip access-group BOGON-MARTIAN in

no shutdown

interface s0/1/1

ip address 209.165.201.2 255.255.255.0

ip access-group BOGON-MARTIAN in

no shutdown

interface loopback 0

ip address 209.165.227.1 255.255.255.255

no shutdown

exit

interface loopback 1

ip address 209.165.228.1 255.255.255.255

no shutdown

exit

ip route 209.165.226.1 255.255.255.255 s0/1/0

ip route 209.165.226.1 255.255.255.255 s0/1/1

router bgp 39457

bgp router-id 209.165.227.1

neighbor 209.165.226.1 remote-as 181035

neighbor 209.165.226.1 update-source loopback 0

neighbor 209.165.226.1 ebgp-multihop 3

network 209.165.228.1 mask 255.255.255.255

exit

ip access-list standard NAT-ELIGIBLE

permit 10.0.0.0 0.255.255.255

deny any log

exit

ip nat inside source list NAT-ELIGIBLE interface loopback 1 overload

ip dhcp excluded-address 10.0.16.1 10.0.16.5

ip dhcp excluded-address 10.0.27.1 10.0.27.5

ip dhcp pool NAT16

network 10.0.16.0 255.255.255.0

default-router 10.0.16.1

exit

ip dhcp pool NAT27

network 10.0.27.0 255.255.255.0

default-router 10.0.27.1

exit

interface s0/1/0

ip nat outside

exit

interface s0/1/1

ip nat outside

exit

interface g0/0/1.16

ip nat inside

exit

interface g0/0/1.27

ip nat inside

exit

line con 0

logging synchronous

exec-timeout 0 0

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

D1 Configuration File Scripts

! D1 - Trouble Ticket # 1

tclsh

puts [ open "flash:/enarsi/32.1.1.1-d1-config.txt" w+ ] {

hostname D1

no ip domain lookup

ip routing

banner motd # This is D1, Trouble Ticket 32.1.1.1  #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

interface range g1/0/1-24

switchport mode access

shutdown

exit

interface g1/0/11

no switchport

ip address 192.0.0.2 255.255.255.0

no shutdown

exit

interface vlan 11

ip address 209.165.224.1 255.255.255.0

no shutdown

exit

interface vlan 12

ip address 209.165.225.1 255.255.255.0

no shutdown

exit

interface g1/0/23

switchport mode access

switchport access vlan 11

spanning-tree portfast

no shutdown

exit

interface g1/0/24

switchport mode access

switchport access vlan 12

spanning-tree portfast

no shutdown

exit

router ospf 1

router-id 131.131.131.131

network 192.0.0.0 0.0.0.255 area 0

network 209.165.224.0 0.0.1.255 area 0

passive-interface vlan 11

passive-interface vlan 12

exit

ip dhcp excluded-address 209.165.224.1 209.165.224.5

ip dhcp excluded-address 209.165.225.1 209.165.225.5

ip dhcp pool VLAN11

network 209.165.224.0 255.255.255.0

default-router 209.165.224.1

exit

ip dhcp pool VLAN12

network 209.165.225.0 255.255.255.0

default-router 209.165.225.1

exit

line con 0

logging synchronous

exec-timeout 0 0

exit

time-range REMOTE-CONTROL

periodic weekdays 05:00 to 23:59

periodic weekend 05:00 to 23:59

exit

ip access-list extended VTY-CONTROL

permit ip 209.165.224.0 0.0.0.255 any time-range REMOTE-CONTROL

deny ip any any log

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

access-class VTY-CONTROL in

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

! D1 - Trouble Ticket # 2

tclsh

puts [ open "flash:/enarsi/32.1.1.2-d1-config.txt" w+ ] {

hostname D1

no ip domain lookup

ip routing

banner motd # This is D1, Trouble Ticket 32.1.1.2 #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

interface range g1/0/1-24

switchport mode access

shutdown

exit

interface g1/0/11

no switchport

ip address 192.0.0.2 255.255.255.0

no shutdown

exit

interface vlan 11

ip address 209.165.224.1 255.255.255.0

no shutdown

exit

interface vlan 12

ip address 209.165.225.1 255.255.255.0

no shutdown

exit

interface g1/0/23

switchport mode access

switchport access vlan 11

spanning-tree portfast

no shutdown

exit

interface g1/0/24

switchport mode access

switchport access vlan 12

spanning-tree portfast

no shutdown

exit

router ospf 1

router-id 131.131.131.131

network 192.0.0.0 0.0.0.255 area 0

network 209.165.224.0 0.0.1.255 area 0

passive-interface vlan 11

passive-interface vlan 12

exit

ip dhcp excluded-address 209.165.224.1 209.165.224.5

ip dhcp excluded-address 209.165.225.1 209.165.225.5

ip dhcp pool VLAN11

network 209.165.224.0 255.255.255.0

default-router 209.165.224.1

exit

ip dhcp pool VLAN12

network 209.165.225.0 255.255.255.0

default-router 209.165.225.1

exit

line con 0

logging synchronous

exec-timeout 0 0

exit

time-range REMOTE-CONTROL

periodic weekdays 05:00 to 23:59

periodic weekend 05:00 to 23:59

exit

ip access-list extended VTY-CONTROL

permit ip 209.165.224.0 0.0.0.255 any time-range REMOTE-CONTROL

deny ip any any log

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

access-class VTY-CONTROL in

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

! D1 - Trouble Ticket # 3

tclsh

puts [ open "flash:/enarsi/32.1.1.3-d1-config.txt" w+ ] {

hostname D1

no ip domain lookup

ip routing

banner motd # This is D1, Trouble Ticket 32.1.1.3 #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

interface range g1/0/1-24

switchport mode access

shutdown

exit

interface g1/0/11

no switchport

ip address 192.0.0.2 255.255.255.0

no shutdown

exit

interface vlan 11

ip address 209.165.224.1 255.255.255.0

no shutdown

exit

interface vlan 12

ip address 209.165.225.1 255.255.255.0

no shutdown

exit

interface g1/0/23

switchport mode access

switchport access vlan 11

spanning-tree portfast

no shutdown

exit

interface g1/0/24

switchport mode access

switchport access vlan 12

spanning-tree portfast

no shutdown

exit

router ospf 1

router-id 131.131.131.131

network 192.0.0.0 0.0.0.255 area 0

network 209.165.224.0 0.0.1.255 area 0

passive-interface vlan 11

passive-interface vlan 12

exit

ip dhcp excluded-address 209.165.224.1 209.165.224.5

ip dhcp excluded-address 209.165.225.1 209.165.225.5

ip dhcp pool VLAN11

network 209.165.224.0 255.255.255.0

default-router 209.165.224.1

exit

ip dhcp pool VLAN12

network 209.165.225.0 255.255.255.0

default-router 209.165.225.1

exit

line con 0

logging synchronous

exec-timeout 0 0

exit

time-range REMOTE-CONTROL

periodic weekdays 00:00 to 01:00

periodic weekend 00:00 to 02:00

exit

ip access-list extended VTY-CONTROL

permit ip 209.165.224.0 0.0.0.255 any time-range REMOTE-CONTROL

deny ip any any log

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

access-class VTY-CONTROL in

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

D2 Configuration File Scripts

! D2- Trouble Ticket # 1

tclsh

puts [ open "flash:/enarsi/32.1.1.1-d2-config.txt" w+ ] {

hostname D2

no ip domain lookup

ip routing

banner motd # This is D2, Trouble Ticket 32.1.1.1  #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

interface range g1/0/1-24

switchport mode access

shutdown

exit

interface g1/0/11

switchport mode trunk

switchport nonegotiate

no shutdown

exit

interface g1/0/23

switchport mode access

switchport access vlan 16

spanning-tree portfast

no shutdown

exit

interface g1/0/24

switchport mode access

switchport access vlan 27

spanning-tree portfast

no shutdown

exit

interface vlan 16

ip address 10.0.16.2 255.255.255.0

no shutdown

exit

interface vlan 27

ip address 10.0.27.2 255.255.255.0

no shutdown

exit

ip default-gateway 10.0.27.1

line con 0

logging synchronous

exec-timeout 0 0

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

! D2- Trouble Ticket # 2

tclsh

puts [ open "flash:/enarsi/32.1.1.2-d2-config.txt" w+ ] {

hostname D2

no ip domain lookup

ip routing

banner motd # This is D2, Trouble Ticket 32.1.1.2 #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

interface range g1/0/1-24

switchport mode access

shutdown

exit

interface g1/0/11

switchport mode trunk

switchport nonegotiate

no shutdown

exit

interface g1/0/23

switchport mode access

switchport access vlan 16

spanning-tree portfast

no shutdown

exit

interface g1/0/24

switchport mode access

switchport access vlan 27

spanning-tree portfast

no shutdown

exit

interface vlan 27

ip address 10.0.27.2 255.255.255.0

no shutdown

exit

ip default-gateway 10.0.27.1

line con 0

logging synchronous

exec-timeout 0 0

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

! D2- Trouble Ticket # 3

tclsh

puts [ open "flash:/enarsi/32.1.1.3-d2-config.txt" w+ ] {

hostname D2

no ip domain lookup

ip routing

banner motd # This is D2, Trouble Ticket 32.1.1.3 #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

interface range g1/0/1-24

switchport mode access

shutdown

exit

interface g1/0/11

switchport mode trunk

switchport nonegotiate

no shutdown

exit

interface g1/0/23

switchport mode access

switchport access vlan 16

spanning-tree portfast

no shutdown

exit

interface g1/0/24

switchport mode access

switchport access vlan 27

spanning-tree portfast

no shutdown

exit

interface vlan 27

ip address 10.0.27.2 255.255.255.0

no shutdown

exit

ip default-gateway 10.0.27.1

line con 0

logging synchronous

exec-timeout 0 0

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit