# Lab - Troubleshoot IPv6 ACLs (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

# Topology

# Addressing Table

| Device | Interface | IPv6 Address/Prefix Length | Link-Local Address |
| --- | --- | --- | --- |
| R1 | G0/0/1 | 2001:db8:acad:192::1/64 | fe80::1:1 |
| R1 | S0/1/0 | 2001:db8:acad:2000::1/64 | fe80::1:2 |
| R1 | S0/1/1 | 2001:db8:acad:2001::/1/64 | fe80::1:3 |
| R1 | Loopback 0 | 2001:db8:acad:226::1/64 | fe80::1:4 |
| R3 | G0/0/1.16 | 2001:db8:acad:16::1/64 | fe80::3:1 |
| R3 | G0/0/1.27 | 2001:db8:acad:27::1/64 | fe80::3:2 |
| R3 | Loopback 1 | 2001:db8:acad:227::1/64 | fe80::3:3 |
| D1 | G1/0/11 | 2001:db8:acad:192::2/64 | fe80::d1:1 |
| D1 | VLAN 11 | 2001:db8:acad:224::1/64 | fe80::d1:2 |
| D1 | VLAN 12 | 2001:db8:acad:225::1/64 | fe80::d1:3 |
| D2 | VLAN 27 | 2001:db8:acad:27::2/64 | fe80::d2:1 |
| D2 | G1/0/11 | 2001:db8:1d1::2/64 | fe80::d1:1 |
| D2 | Loopback 0 | 2001:db8:acad:1000::1/64 | fe80::d1:2 |
| D2 | Loopback 1 | 2001:db8:acad:1001::1/64 | fe80::d1:3 |
| PC1 | NIC | SLAAC | EUI-64/CGA |
| PC2 | NIC | SLAAC | EUI-64/CGA |
| PC3 | NIC | DHCPv6 | EUI-64/CGA |
| PC4 | NIC | DHCPv6 | EUI-64/CGA |

# Objectives

- Troubleshoot network issues related to the configuration and operation of IPv6 ACLs.

# Background / Scenario

Your team operates a small enterprise lab that mirrors a regional campus network. R1 (ASN 15) and R3 (ASN 41) are peered via BGP using their loopback interfaces with multihop across the serial links between them. R1 maintains an OSPFv3 adjacency with distribution switch D1 and advertises a default route into the campus. R3 provides router-on-a-stick routing for user VLANs 16 and 27. Client segments behind D1 rely on SLAAC for IPv6 addressing, while client segments behind D2  are required to use DHCPv6 sourced from R3. Devices on VLANs 11 and 12 contain a large number of battery-powered Wi-Fi and IoT devices. To decrease energy consumption and channel contention, a policy was implemented to increase the periodic ICMPv6 Router Advertisement messages to 20 minutes. The clients will now rely on sending their ICMPv6 Router Solicitation message immediately after joining the network.

During a planned overnight change window, the night shift implemented IPv6 “hardening” controls, including several ACL updates at different enforcement points. Shortly after, multiple IPv6-related issues were reported by end users coming in for the morning shift.

You will be loading configurations with intentional errors onto the network. Your tasks are to FIND the error(s), document your findings and the command(s) or method(s) used to fix them, FIX the issue(s) presented here, and then test the network to ensure both of the following conditions are met:

the complaint received in the ticket is resolved

full reachability is restored

Note: The routers used with CCNP hands-on labs are Cisco 8200L with Cisco IOS XE Release 17.12.05c (universalk9 image). The switches used in the labs are Cisco Catalyst 9300L with Cisco IOS XE Release 17.12.05 (universalk9 image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs.

Note: Make sure that the devices have been erased and have no startup configurations. If you are unsure, contact your instructor.

Instructor Note: Refer to the Instructor Lab Manual for the procedures to initialize and reload devices.

# Required Resources

2 Routers (Cisco 8200L with Cisco IOS XE Release 17.12.05c universal image or comparable)

2 Switches (Cisco 9300L with Cisco IOS XE Release 17.12.05 universal image or comparable)

4 PCs (Choice of operating system with terminal emulation program installed)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet and serial cables as shown in the topology

## Trouble Ticket 32.1.2.1

Scenario:

Following last night’s “client VLAN lockdown” change, the help desk received reports that new or rebooted devices on VLAN 11 and VLAN 12 had long delays in obtaining IPv6 addresses via SLAAC, upwards of 20 minutes. Existing devices that did not renew may still appear online, but new joins and address refreshes fail. Your task is to assess the impact of the ACL changes on client autoconfiguration behavior and restore normal SLAAC operation without weakening the intended client access restrictions.

- Be sure you are using devices that have been connected and prepared by your instructor. Use the commands listed below to load the configuration files for this trouble ticket:

Instructor Note: Commands for uploading the configuration are provided at the end of this document.

| Device | Command |
| --- | --- |
| R1 | copy flash:/enarsi/32.1.2.1-r1-config.txt running-config |
| R3 | copy flash:/enarsi/32.1.2.1-r3-config.txt running-config |
| D1 | copy flash:/enarsi/32.1.2.1-d1-config.txt running-config |
| D2 | copy flash:/enarsi/32.1.2.1-d2-config.txt running-config |

- PCs 1, 2, 3, and 4 should be configured for dynamic acquisition of an IPv6 address.

- Passwords on all devices are cisco12345. If a username is required, use admin.

- After you have corrected the ticket, change the MOTD on EACH DEVICE using the following command:

banner motd # This is $(hostname) FIXED from ticket <ticket number> #

- Save the configuration by issuing the wri command (on each device).

- Inform your instructor that you are ready for the next ticket.

- After the instructor approves your solution for this ticket, issue the privileged EXEC command reset.now. This script will clear your configurations and reload the devices.

Instructor Notes:

This trouble ticket contains 1 intentional error. The ACL applied to D1 interface VLAN 11 and VLAN 12 does not allow for ICMPv6 Router Solicitation (RS) messages. The administrator that added the ACLs did not take RSmessages into account when adding the ‘deny ipv6 any any; ACE. The ‘deny ipv6 any any’ ACE is also implied, so even if it were not added by the administrator, this statement would still be in effect.

The commands used to fix this error should be:

D1(config)# ipv6 access-list CLIENT-CONTROL-VLAN11

D1(config-ipv6-acl)#  sequence 31 permit icmp any any router-solicitation

D1(config-ipv6-acl)# exit

D1(config)# ipv6 access-list CLIENT-CONTROL-VLAN12

D1(config-ipv6-acl)#  sequence 31 permit icmp any any router-solicitation

D1(config-ipv6-acl)#  exit

Note: The permit icmp any any eq echo-request and permit icmp any any eq echo-reply ACE are necessary for a later ticket. They were added in this ticket to normalize their presence.

Note: Host behavior may be different amongst operating systems. For example, while Windows 10 honors the /no-autoconfig flag, it seems that Ubuntu 18.0.4 LTS ignores the lack of an A flag in the RA creates the GUA anyway.

## Trouble Ticket 32.1.2.2

Scenario:

Users on VLANs 16 and 27 behind R3 report they cannot reach internal gateway interfaces on D1 using IPv6 ping this morning, although routing adjacencies between R1 and R3 appear stable. The night shift notes indicate a “bogon/martian” IPv6 filter was introduced inbound on R1’s serial links to prevent unrouted or inappropriate source addresses from entering ASN 15. It appears the new filter is too broad and is blocking legitimate campus prefixes. Your task is to adjust the control so that valid traffic from VLANs 16 and 27 can reach internal resources while preserving the intent of the filter.

- Use the commands listed below to load the configuration files for this trouble ticket:

Instructor Note: Commands for uploading the configuration are provided at the end of this document.

| Device | Command |
| --- | --- |
| R1 | copy flash:/enarsi/32.1.2.2-r1-config.txt running-config |
| R3 | copy flash:/enarsi/32.1.2.2-r3-config.txt running-config |
| D1 | copy flash:/enarsi/32.1.2.2-d1-config.txt running-config |
| D2 | copy flash:/enarsi/32.1.2.2-d2-config.txt running-config |

- PCs 1, 2, 3, and 4 should be configured for dynamic acquisition of an IPv6 address.

- Passwords on all devices are cisco12345. If a username is required, use admin.

- Once you have fixed the ticket, change the MOTD on EACH DEVICE using the following command:

banner motd # This is $(hostname) FIXED from ticket <ticket number> #

- Then save the configuration by issuing the wri command (on each device).

- Inform your instructor that you are ready for the next ticket.

- After the instructor approves your solution for this ticket, issue the privileged EXEC command reset.now. This script will clear your configurations and reload the devices.

Instructor Notes:

This trouble ticket contains 1 intentional error repeated in two places. The ACL applied to the R1 serial interfaces has a broad deny statement in place blocking hosts from the 2001:db8:acad:16::/64 and 2001:db8:acad:27::/64 networks from communicating with the interior of ASN 15.

There are several possible ways to fix this error and allow traffic from the 2001:db8:acad:16::/64 and 2001:db8:acad:27::/64 networks to pass. The commands used to fix this error in the most specific manner are:

R1(config)# ipv6 access-list BOGON-MARTIAN

R1(config-ipv6-acl)#  sequence 11 permit ipv6 2001:db8:acad:16::/64 any

R1(config-ipv6-acl)#  sequence 12 permit ipv6 2001:db8:acad:27::/64 any

R1(config-ipv6-acl)#  exit

## Trouble Ticket 32.1.2.3

Scenario:

Shortly after another “UDP hardening” change on R3’s subinterfaces on VLAN 16, Users on VLAN 16 reported to the help desk that the point-of-sale (POS) systems on VLAN 16 are not appearing in the network’s DHCPv6 lease logs. These Windows-based POS terminals are part of a monitored group of devices whose network activity must be tracked for compliance and auditing purposes. While it does not matter which IPv6 address each POS system receives, it is critical that the address be assigned by the DHCPv6 server so that the lease information is recorded. The DHCPv6 server assigns addresses with an interface ID beginning with “feed:”. Currently, these devices are not obtaining any IPv6 global unicast addresses. Your task is to troubleshoot the issue and ensure that the POS systems receive their IPv6 addresses from the DHCPv6 server.

- Use the commands listed below to load the configuration files for this trouble ticket:

Instructor Note: Commands for uploading the configuration are provided at the end of this document.

| Device | Command |
| --- | --- |
| R1 | copy flash:/enarsi/32.1.2.3-r1-config.txt running-config |
| R3 | copy flash:/enarsi/32.1.2.3-r3-config.txt running-config |
| D1 | copy flash:/enarsi/32.1.2.3-d1-config.txt running-config |
| D2 | copy flash:/enarsi/32.1.2.3-d2-config.txt running-config |

- PCs 1, 2, 3, and 4 should be configured for dynamic acquisition of an IPv6 address.

- Passwords on all devices are cisco12345. If a username is required, use admin.

- Once you have fixed the ticket, change the MOTD on EACH DEVICE using the following command:

banner motd # This is $(hostname) FIXED from ticket <ticket number> #

- Then save the configuration by issuing the wri command (on each device).

- Inform your instructor that you are ready for the next ticket.

- After the instructor approves your solution for this ticket, issue the privileged EXEC command reset.now. This script will clear your configurations and reload the devices.

Instructor Notes:

This trouble ticket contains 1 intentional error repeated in two places. The ACL applied to the R3 sub-interfaces is allowing the wrong UDP ports for DHCPv6. The ACL is allowing UDP ports 67 and 68, while DHCPv6 uses UDP ports 546 and 547.

There are several possible ways to fix this error and allow DHCPv6 traffic to pass. The commands used to fix this error in the most specific manner are:

R3(config)# ipv6 access-list CONTROL-UDP-TRAFFIC

R3(config-ipv6-acl)# no sequence 10

R3(config-ipv6-acl)# sequence 10 permit udp any any range 546 547

R3(config-ipv6-acl)# exit

Note: Host behavior may be different amongst operating systems. For example, while Windows 10 honors the /no-autoconfig flag, it seems that Ubuntu 18.0.4 LTS ignores the lack of an A flag in the RA creates the GUA anyway.

End of document

Uploading Configuration Files

Use the commands below to create the configuration files for each trouble ticket in this lab on the lab devices. The TCL script commands help create and copy the configurations. However, the configuration commands could also be copied and pasted directly into global config mode on each device. Simply remove the TCL script commands, enter the enable and configure t commands on the device, and copy and paste the configuration commands.

Important: The device requires a folder in flash named enarsi. Use the dir command to verify. If the folder is missing, then create it using the mkdir flash:/enarsi privileged EXEC command. For all switches, make sure the vlan.dat file is set to the default. Use the delete vlan.dat privileged EXEC command, if necessary.

Reset scripts

These TCL scripts will completely clear and reload the device in preparation for the next ticket. Copy and paste the appropriate script to the appropriate device.

Router Reset Script

tclsh

puts [ open "flash:/enarsi/reset.tcl" w+ ] {

typeahead "\n"

copy running-config startup-config

typeahead "\n"

erase startup-config

puts "Reloading the router"

typeahead "\n"

reload

}

tclquit

D1/D2 (Cisco 3650) Reset Script - The default 3650 SDM template supports IPv6 by default, so it is not set by this script.

tclsh

puts [ open "flash:/enarsi/reset.tcl" w+ ] {

typeahead "\n"

copy running-config startup-config

typeahead "\n"

erase startup-config

delete /force vlan.dat

puts "Reloading the switch"

typeahead "\n"

reload

}

tclquit

A1 (Cisco 2960 Script) - The default 2960 SDM template does not support IPv6, so this script includes that setting.

tclsh

puts [ open "flash:reset.tcl" w+ ] {

typeahead "\n"

copy running-config startup-config

typeahead "\n"

erase startup-config

delete /force vlan.dat

delete /force multiple-fs

ios_config "sdm prefer lanbase-routing"

typeahead "\n"

puts "Reloading the switch in 1 minute, type reload cancel to halt"

typeahead "\n"

reload

}

tclquit

R1 Configuration File Scripts

! R1 - Trouble Ticket # 1

tclsh

puts [ open "flash:/enarsi/32.1.2.1-r1-config.txt" w+ ] {

hostname R1

no ip domain lookup

ipv6 unicast-routing

banner motd # This is R1, Trouble Ticket 32.1.2.1 #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

interface g0/0/1

ipv6 address fe80::1:1 link-local

ipv6 address 2001:db8:acad:192::1/64

no shutdown

exit

interface s0/1/0

ipv6 address fe80::1:2 link-local

ipv6 address 2001:db8:acad:2000::1/64

no shutdown

exit

interface s0/1/1

ipv6 address fe80::1:3 link-local

ipv6 address 2001:db8:acad:2001::1/64

no shutdown

exit

interface loopback 0

ipv6 address fe80::1:4 link-local

ipv6 address 2001:db8:acad:226::1/64

no shutdown

exit

ipv6 route 2001:db8:acad:227::1/64 s0/1/0 2001:db8:acad:2000::2

ipv6 route 2001:db8:acad:227::1/64 s0/1/1 2001:db8:acad:2001::2

router ospfv3 1

router-id 1.1.1.1

address-family ipv6 unicast

default-information originate always

exit

exit

router bgp 15

bgp router-id 1.1.1.1

neighbor 2001:db8:acad:227::1 remote-as 41

neighbor 2001:db8:acad:227::1 update-source loopback 0

neighbor 2001:db8:acad:227::1 ebgp-multihop 3

address-family ipv6 unicast

neighbor 2001:db8:acad:227::1 activate

network 2001:db8:acad:224::/64

network 2001:db8:acad:225::/64

network 2001:db8:acad:192::/64

exit

exit

interface g0/0/1

ospfv3 1 ipv6 area 0

exit

line con 0

logging synchronous

exec-timeout 0 0

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

! R1 - Trouble Ticket # 2

tclsh

puts [ open "flash:/enarsi/32.1.2.2-r1-config.txt" w+ ] {

hostname R1

no ip domain lookup

ipv6 unicast-routing

banner motd # This is R1, Trouble Ticket 32.1.2.2 #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

interface g0/0/1

ipv6 address fe80::1:1 link-local

ipv6 address 2001:db8:acad:192::1/64

no shutdown

exit

interface s0/1/0

ipv6 address fe80::1:2 link-local

ipv6 address 2001:db8:acad:2000::1/64

no shutdown

exit

interface s0/1/1

ipv6 address fe80::1:3 link-local

ipv6 address 2001:db8:acad:2001::1/64

no shutdown

exit

interface loopback 0

ipv6 address fe80::1:4 link-local

ipv6 address 2001:db8:acad:226::1/64

no shutdown

exit

ipv6 route 2001:db8:acad:227::1/64 s0/1/0 2001:db8:acad:2000::2

ipv6 route 2001:db8:acad:227::1/64 s0/1/1 2001:db8:acad:2001::2

router ospfv3 1

router-id 1.1.1.1

address-family ipv6 unicast

default-information originate always

exit

exit

router bgp 15

bgp router-id 1.1.1.1

neighbor 2001:db8:acad:227::1 remote-as 41

neighbor 2001:db8:acad:227::1 update-source loopback 0

neighbor 2001:db8:acad:227::1 ebgp-multihop 3

address-family ipv6 unicast

neighbor 2001:db8:acad:227::1 activate

network 2001:db8:acad:224::/64

network 2001:db8:acad:225::/64

network 2001:db8:acad:192::/64

exit

exit

interface g0/0/1

ospfv3 1 ipv6 area 0

exit

ipv6 access-list BOGON-MARTIAN

permit ipv6 2001:db8:acad:227::/64 any

deny ipv6 2001:db8:acad::/48 any

permit ipv6 any any

exit

interface s0/1/0

ipv6 traffic-filter BOGON-MARTIAN in

exit

interface s0/1/1

ipv6 traffic-filter BOGON-MARTIAN in

exit

line con 0

logging synchronous

exec-timeout 0 0

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

! R1 - Trouble Ticket # 3

tclsh

puts [ open "flash:/enarsi/32.1.2.3-r1-config.txt" w+ ] {

hostname R1

no ip domain lookup

ipv6 unicast-routing

banner motd # This is R1, Trouble Ticket 32.1.2.3 #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

interface g0/0/1

ipv6 address fe80::1:1 link-local

ipv6 address 2001:db8:acad:192::1/64

no shutdown

exit

interface s0/1/0

ipv6 address fe80::1:2 link-local

ipv6 address 2001:db8:acad:2000::1/64

no shutdown

exit

interface s0/1/1

ipv6 address fe80::1:3 link-local

ipv6 address 2001:db8:acad:2001::1/64

no shutdown

exit

interface loopback 0

ipv6 address fe80::1:4 link-local

ipv6 address 2001:db8:acad:226::1/64

no shutdown

exit

ipv6 route 2001:db8:acad:227::1/64 s0/1/0 2001:db8:acad:2000::2

ipv6 route 2001:db8:acad:227::1/64 s0/1/1 2001:db8:acad:2001::2

router ospfv3 1

router-id 1.1.1.1

address-family ipv6 unicast

default-information originate always

exit

exit

router bgp 15

bgp router-id 1.1.1.1

neighbor 2001:db8:acad:227::1 remote-as 41

neighbor 2001:db8:acad:227::1 update-source loopback 0

neighbor 2001:db8:acad:227::1 ebgp-multihop 3

address-family ipv6 unicast

neighbor 2001:db8:acad:227::1 activate

network 2001:db8:acad:224::/64

network 2001:db8:acad:225::/64

network 2001:db8:acad:192::/64

exit

exit

interface g0/0/1

ospfv3 1 ipv6 area 0

exit

ipv6 access-list BOGON-MARTIAN

permit ipv6 2001:db8:acad:227::/64 any

permit ipv6 2001:db8:acad:16::/64 any

permit ipv6 2001:db8:acad:27::/64 any

deny ipv6 2001:db8:acad::/48 any

permit ipv6 any any

exit

interface s0/1/0

ipv6 traffic-filter BOGON-MARTIAN in

exit

interface s0/1/1

ipv6 traffic-filter BOGON-MARTIAN in

exit

line con 0

logging synchronous

exec-timeout 0 0

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

R3 Configuration File Scripts

! R3 - Trouble Ticket # 1

tclsh

puts [ open "flash:/enarsi/32.1.2.1-r3-config.txt" w+ ] {

hostname R3

no ip domain lookup

ipv6 unicast-routing

banner motd # This is R3, Trouble Ticket 32.1.2.1  #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

interface g0/0/1

no ipv6 address

no shutdown

exit

interface g0/0/1.16

encapsulation dot1q 16

ipv6 address fe80::3:1 link-local

ipv6 address 2001:db8:acad:16::1/64

ipv6 nd prefix 2001:db8:acad:16::/64 no-autoconfig

no shutdown

exit

interface g0/0/1.27

encapsulation dot1q 27

ipv6 address fe80::3:2 link-local

ipv6 address 2001:db8:acad:27::1/64

ipv6 nd prefix 2001:db8:acad:27::/64 no-autoconfig

no shutdown

exit

interface s0/1/0

ipv6 address fe80::3:3 link-local

ipv6 address 2001:db8:acad:2000::2/64

no shutdown

exit

interface s0/1/1

ipv6 address fe80::3:4 link-local

ipv6 address 2001:db8:acad:2001::2/64

no shutdown

exit

interface loopback 0

ipv6 address fe80::3:5 link-local

ipv6 address 2001:db8:acad:227::1/64

no shutdown

exit

ipv6 route 2001:db8:acad:226::1/64 s0/1/0 2001:db8:acad:2000::1

ipv6 route 2001:db8:acad:226::1/64 s0/1/1 2001:db8:acad:2001::1

router bgp 41

bgp router-id 3.3.3.3

neighbor 2001:db8:acad:226::1 remote-as 15

neighbor 2001:db8:acad:226::1 update-source loopback 0

neighbor 2001:db8:acad:226::1 ebgp-multihop 3

address-family ipv6 unicast

neighbor 2001:db8:acad:226::1 activate

network 2001:db8:acad:16::/64

network 2001:db8:acad:27::/64

exit

exit

ipv6 dhcp pool LAN16

address prefix 2001:db8:acad:16::/64

exit

ipv6 dhcp pool LAN27

address prefix 2001:db8:acad:27::/64

exit

interface g0/0/1.16

ipv6 nd managed-config-flag

ipv6 dhcp server LAN16

exit

interface g0/0/1.27

ipv6 nd managed-config-flag

ipv6 dhcp server LAN27

exit

line con 0

logging synchronous

exec-timeout 0 0

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

! R3 - Trouble Ticket # 2

tclsh

puts [ open "flash:/enarsi/32.1.2.2-r3-config.txt" w+ ] {

hostname R3

no ip domain lookup

ipv6 unicast-routing

banner motd # This is R3, Trouble Ticket 32.1.2.2  #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

interface g0/0/1

no ipv6 address

no shutdown

exit

interface g0/0/1.16

encapsulation dot1q 16

ipv6 address fe80::3:1 link-local

ipv6 address 2001:db8:acad:16::1/64

ipv6 nd prefix 2001:db8:acad:16::/64 no-autoconfig

no shutdown

exit

interface g0/0/1.27

encapsulation dot1q 27

ipv6 address fe80::3:2 link-local

ipv6 address 2001:db8:acad:27::1/64

ipv6 nd prefix 2001:db8:acad:27::/64 no-autoconfig

no shutdown

exit

interface s0/1/0

ipv6 address fe80::3:3 link-local

ipv6 address 2001:db8:acad:2000::2/64

no shutdown

exit

interface s0/1/1

ipv6 address fe80::3:4 link-local

ipv6 address 2001:db8:acad:2001::2/64

no shutdown

exit

interface loopback 0

ipv6 address fe80::3:5 link-local

ipv6 address 2001:db8:acad:227::1/64

no shutdown

exit

ipv6 route 2001:db8:acad:226::1/64 s0/1/0 2001:db8:acad:2000::1

ipv6 route 2001:db8:acad:226::1/64 s0/1/1 2001:db8:acad:2001::1

router bgp 41

bgp router-id 3.3.3.3

neighbor 2001:db8:acad:226::1 remote-as 15

neighbor 2001:db8:acad:226::1 update-source loopback 0

neighbor 2001:db8:acad:226::1 ebgp-multihop 3

address-family ipv6 unicast

neighbor 2001:db8:acad:226::1 activate

network 2001:db8:acad:16::/64

network 2001:db8:acad:27::/64

exit

exit

ipv6 dhcp pool LAN16

address prefix 2001:db8:acad:16::/64

exit

ipv6 dhcp pool LAN27

address prefix 2001:db8:acad:27::/64

exit

interface g0/0/1.16

ipv6 nd managed-config-flag

ipv6 dhcp server LAN16

exit

interface g0/0/1.27

ipv6 nd managed-config-flag

ipv6 dhcp server LAN27

exit

line con 0

logging synchronous

exec-timeout 0 0

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

! R3 - Trouble Ticket # 3

tclsh

puts [ open "flash:/enarsi/32.1.2.3-r3-config.txt" w+ ] {

hostname R3

no ip domain lookup

ipv6 unicast-routing

banner motd # This is R3, Trouble Ticket 32.1.2.3  #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

interface g0/0/1

no ipv6 address

no shutdown

exit

interface g0/0/1.16

encapsulation dot1q 16

ipv6 address fe80::3:1 link-local

ipv6 address 2001:db8:acad:16::1/64

ipv6 nd prefix 2001:db8:acad:16::/64 no-autoconfig

no shutdown

exit

interface g0/0/1.27

encapsulation dot1q 16

ipv6 address fe80::3:2 link-local

ipv6 address 2001:db8:acad:27::1/64

ipv6 nd prefix 2001:db8:acad:27::/64

no shutdown

exit

interface s0/1/0

ipv6 address fe80::3:3 link-local

ipv6 address 2001:db8:acad:2000::2/64

no shutdown

exit

interface s0/1/1

ipv6 address fe80::3:4 link-local

ipv6 address 2001:db8:acad:2001::2/64

no shutdown

exit

interface loopback 0

ipv6 address fe80::3:5 link-local

ipv6 address 2001:db8:acad:227::1/64

no shutdown

exit

ipv6 route 2001:db8:acad:226::1/64 s0/1/0 2001:db8:acad:2000::1

ipv6 route 2001:db8:acad:226::1/64 s0/1/1 2001:db8:acad:2001::1

router bgp 41

bgp router-id 3.3.3.3

neighbor 2001:db8:acad:226::1 remote-as 15

neighbor 2001:db8:acad:226::1 update-source loopback 0

neighbor 2001:db8:acad:226::1 ebgp-multihop 3

address-family ipv6 unicast

neighbor 2001:db8:acad:226::1 activate

network 2001:db8:acad:16::/64

network 2001:db8:acad:27::/64

exit

exit

ipv6 dhcp pool LAN16

address prefix 2001:db8:acad:16:feed::/80

exit

interface g0/0/1.16

ipv6 nd managed-config-flag

ipv6 dhcp server LAN16

exit

interface g0/0/1.27

ipv6 nd managed-config-flag

ipv6 dhcp server LAN27

exit

ipv6 access-list CONTROL-UDP-TRAFFIC

permit udp any any range 67 68

permit udp any any gt 1024

deny udp any any

permit ipv6 any any

exit

interface g0/0/1.16

ipv6 traffic-filter CONTROL-UDP-TRAFFIC in

exit

interface g0/0/1.27

ipv6 traffic-filter CONTROL-UDP-TRAFFIC in

exit

line con 0

logging synchronous

exec-timeout 0 0

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

D1 Configuration File Scripts

! D1 - Trouble Ticket # 1

tclsh

puts [ open "flash:/enarsi/32.1.2.1-d1-config.txt" w+ ] {

hostname D1

no ip domain lookup

ip routing

ipv6 unicast-routing

banner motd # This is D1, Trouble Ticket 32.1.2.1  #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

interface range g1/0/1-24

switchport mode access

shutdown

exit

interface g1/0/11

no switchport

ipv6 address fe80::d1:1 link-local

ipv6 address 2001:db8:acad:192::2/64

no shutdown

exit

interface vlan 11

ipv6 address fe80::d1:2 link-local

ipv6 address 2001:db8:acad:224::1/64

no shutdown

exit

interface vlan 12

ipv6 address fe80::d1:3 link-local

ipv6 address 2001:db8:acad:225::1/64

no shutdown

exit

interface g1/0/23

switchport mode access

switchport access vlan 11

spanning-tree portfast

no shutdown

exit

interface g1/0/24

switchport mode access

switchport access vlan 12

spanning-tree portfast

no shutdown

exit

router ospfv3 1

router-id 0.0.13.1

address-family ipv6 unicast

passive-interface vlan 11

passive-interface vlan 12

exit

exit

interface g1/0/11

ospfv3 1 ipv6 area 0

exit

interface vlan 11

ospfv3 1 ipv6 area 0

exit

interface vlan 12

ospfv3 1 ipv6 area 0

exit

ipv6 access-list CLIENT-CONTROL-VLAN11

permit ipv6 2001:db8:acad:224::/64 any

permit icmp any any echo-request

permit icmp any any echo-reply

deny ipv6 any any

exit

interface vlan11

ipv6 traffic-filter CLIENT-CONTROL-VLAN11 in

ipv6 nd ra interval 1200

ipv6 nd ra lifetime 3600

exit

ipv6 access-list CLIENT-CONTROL-VLAN12

permit ipv6 2001:db8:acad:225::/64 any

permit icmp any any echo-request

permit icmp any any echo-reply

deny ipv6 any any

exit

interface vlan12

ipv6 traffic-filter CLIENT-CONTROL-VLAN12 in

ipv6 nd ra interval 1200

ipv6 nd ra lifetime 3600

exit

line con 0

logging synchronous

exec-timeout 0 0

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

exit

interface range gig 1/0/1-6

shutdown

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

! D1 - Trouble Ticket # 2

tclsh

puts [ open "flash:/enarsi/32.1.2.2-d1-config.txt" w+ ] {

hostname D1

no ip domain lookup

ip routing

ipv6 unicast-routing

banner motd # This is D1, Trouble Ticket 32.1.2.2  #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

interface range g1/0/1-24

switchport mode access

shutdown

exit

interface g1/0/11

no switchport

ipv6 address fe80::d1:1 link-local

ipv6 address 2001:db8:acad:192::2/64

no shutdown

exit

interface vlan 11

ipv6 address fe80::d1:2 link-local

ipv6 address 2001:db8:acad:224::1/64

no shutdown

exit

interface vlan 12

ipv6 address fe80::d1:3 link-local

ipv6 address 2001:db8:acad:225::1/64

no shutdown

exit

interface g1/0/23

switchport mode access

switchport access vlan 11

spanning-tree portfast

no shutdown

exit

interface g1/0/24

switchport mode access

switchport access vlan 12

spanning-tree portfast

no shutdown

exit

router ospfv3 1

router-id 0.0.13.1

address-family ipv6 unicast

passive-interface vlan 11

passive-interface vlan 12

exit

exit

interface g1/0/11

ospfv3 1 ipv6 area 0

exit

interface vlan 11

ospfv3 1 ipv6 area 0

exit

interface vlan 12

ospfv3 1 ipv6 area 0

exit

ipv6 access-list CLIENT-CONTROL-VLAN11

permit ipv6 2001:db8:acad:224::/64 any

permit icmp any any echo-request

permit icmp any any echo-reply

permit icmp any any nd-na

permit icmp any any nd-ns

deny ipv6 any any

exit

interface vlan11

ipv6 traffic-filter CLIENT-CONTROL-VLAN11 in

exit

ipv6 access-list CLIENT-CONTROL-VLAN12

permit ipv6 2001:db8:acad:225::/64 any

permit icmp any any echo-request

permit icmp any any echo-reply

permit icmp any any nd-na

permit icmp any any nd-ns

deny ipv6 any any

exit

interface vlan12

ipv6 traffic-filter CLIENT-CONTROL-VLAN12 in

exit

line con 0

logging synchronous

exec-timeout 0 0

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

exit

interface range gig 1/0/1-6

shutdown

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

! D1 - Trouble Ticket # 3

tclsh

puts [ open "flash:/enarsi/32.1.2.3-d1-config.txt" w+ ] {

hostname D1

no ip domain lookup

ip routing

ipv6 unicast-routing

banner motd # This is D1, Trouble Ticket 32.1.2.3  #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

interface range g1/0/1-24

switchport mode access

shutdown

exit

interface g1/0/11

no switchport

ipv6 address fe80::d1:1 link-local

ipv6 address 2001:db8:acad:192::2/64

no shutdown

exit

interface vlan 11

ipv6 address fe80::d1:2 link-local

ipv6 address 2001:db8:acad:224::1/64

no shutdown

exit

interface vlan 12

ipv6 address fe80::d1:3 link-local

ipv6 address 2001:db8:acad:225::1/64

no shutdown

exit

interface g1/0/23

switchport mode access

switchport access vlan 11

spanning-tree portfast

no shutdown

exit

interface g1/0/24

switchport mode access

switchport access vlan 12

spanning-tree portfast

no shutdown

exit

router ospfv3 1

router-id 0.0.13.1

address-family ipv6 unicast

passive-interface vlan 11

passive-interface vlan 12

exit

exit

interface g1/0/11

ospfv3 1 ipv6 area 0

exit

interface vlan 11

ospfv3 1 ipv6 area 0

exit

interface vlan 12

ospfv3 1 ipv6 area 0

exit

ipv6 access-list CLIENT-CONTROL-VLAN11

permit ipv6 2001:db8:acad:224::/64 any

permit icmp any any echo-request

permit icmp any any echo-reply

permit icmp any any nd-na

permit icmp any any nd-ns

deny ipv6 any any

exit

interface vlan11

ipv6 traffic-filter CLIENT-CONTROL-VLAN11 in

exit

ipv6 access-list CLIENT-CONTROL-VLAN12

permit ipv6 2001:db8:acad:225::/64 any

permit icmp any any echo-request

permit icmp any any echo-reply

permit icmp any any nd-na

permit icmp any any nd-ns

deny ipv6 any any

exit

interface vlan12

ipv6 traffic-filter CLIENT-CONTROL-VLAN12 in

exit

line con 0

logging synchronous

exec-timeout 0 0

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

exit

interface range gig 1/0/1-6

shutdown

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

D2 Configuration File Scripts

! D2- Trouble Ticket # 1

tclsh

puts [ open "flash:/enarsi/32.1.2.1-d2-config.txt" w+ ] {

hostname D2

no ip domain lookup

ip routing

ipv6 unicast-routing

banner motd # This is D2, Trouble Ticket 32.1.2.1  #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

interface range g1/0/1-24

switchport mode access

shutdown

exit

interface g1/0/11

switchport mode trunk

switchport nonegotiate

no shutdown

exit

interface g1/0/23

switchport mode access

switchport access vlan 16

spanning-tree portfast

no shutdown

exit

interface g1/0/24

switchport mode access

switchport access vlan 27

spanning-tree portfast

no shutdown

exit

interface vlan 27

ipv6 address fe80::d2:1 link-local

ipv6 address 2001:db8:acad:27::2/64

no shutdown

exit

ipv6 route ::/0 2001:db8:acad:27::1

line con 0

logging synchronous

exec-timeout 0 0

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

exit

interface range gig 1/0/1-6

shutdown

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

! D2- Trouble Ticket # 2

tclsh

puts [ open "flash:/enarsi/32.1.2.2-d2-config.txt" w+ ] {

hostname D2

no ip domain lookup

ip routing

ipv6 unicast-routing

banner motd # This is D2, Trouble Ticket 32.1.2.2  #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

interface range g1/0/1-24

switchport mode access

shutdown

exit

interface g1/0/11

switchport mode trunk

switchport nonegotiate

no shutdown

exit

interface g1/0/23

switchport mode access

switchport access vlan 16

spanning-tree portfast

no shutdown

exit

interface g1/0/24

switchport mode access

switchport access vlan 27

spanning-tree portfast

no shutdown

exit

interface vlan 27

ipv6 address fe80::d2:1 link-local

ipv6 address 2001:db8:acad:27::2/64

no shutdown

exit

ipv6 route ::/0 2001:db8:acad:27::1

line con 0

logging synchronous

exec-timeout 0 0

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

! D2- Trouble Ticket # 3

tclsh

puts [ open "flash:/enarsi/32.1.2.3-d2-config.txt" w+ ] {

hostname D2

no ip domain lookup

ip routing

ipv6 unicast-routing

banner motd # This is D2, Trouble Ticket 32.1.2.3  #

enable secret cisco12345

username admin privilege 15 algorithm-type scrypt secret cisco12345

interface range g1/0/1-24

switchport mode access

shutdown

exit

interface g1/0/11

switchport mode trunk

switchport nonegotiate

no shutdown

exit

interface g1/0/23

switchport mode access

switchport access vlan 16

spanning-tree portfast

no shutdown

exit

interface g1/0/24

switchport mode access

switchport access vlan 27

spanning-tree portfast

no shutdown

exit

interface vlan 27

ipv6 address fe80::d2:1 link-local

ipv6 address 2001:db8:acad:27::2/64

no shutdown

exit

ipv6 route ::/0 2001:db8:acad:27::1

line con 0

logging synchronous

exec-timeout 0 0

exit

line vty 0 4

login local

transport input telnet

exec-timeout 5 0

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit