# Lab - Troubleshoot uRPF (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

# Topology

# Addressing Table

| Device | Interface | IP Address |  | Subnet Mask |
| --- | --- | --- | --- | --- |
| R1 | G0/0/0 | 10.10.1.1 |  | 255.255.255.0 |
| R1 | S0/1/0 | 10.10.3.2 |  | 255.255.255.0 |
| R1 | Lo1 | 192.168.10.10 |  | 255.255.255.0 |
| R2 | G0/0/0 | 10.10.1.2 |  | 255.255.255.0 |
| R2 | G0/0/1 | 10.10.2.1 |  | 255.255.255.0 |
| R3 | G0/0/0 | 10.10.2.2 | 255.255.255.0 |  |
| R3 | S0/1/0 | 10.10.3.1 | 255.255.255.0 |  |
| R3 | Lo1 | 192.168.20.20 | 255.255.255.0 |  |

# Objectives

- Troubleshoot issues related to the configuration and operation of uRPF.

# Background / Scenario

- At your company, an internal security review identified an increased risk of spoofed-source traffic traversing the WAN edge. To strengthen the network’s security posture without impacting legitimate operations, the network team is rolling out unicast Reverse Path Forwarding (uRPF) on selected interfaces in a staged manner. This lab mirrors a small enterprise environment with three routers, EIGRP-based routing, and loopback interfaces representing internal services hosted on distinct subnets. Your objective is to validate that the uRPF rollout preserves expected reachability between all networks while maintaining the intended security controls.

- In this lab, you will be loading configurations with intentional errors onto the network. Your tasks are to FIND the error(s), document your findings and the command(s) or method(s) used to fix them, FIX the issue(s) presented here, and then test the network to ensure both of the following conditions are met:

        - The trouble ticket has been resolved

        - The network is fully functioning

Note: The routers used with CCNP hands-on labs are Cisco 8200L with Cisco IOS XE Release 17.12.05c (universalk9 image). Other routers and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs. Refer to the Router Interface Summary Table at the end of the lab for the correct interface identifiers.

Note: Make sure that the devices have been erased and have no startup configurations. If you are unsure, contact your instructor.

Instructor Note: Refer to the Instructor Lab Manual for the procedures to initialize and reload devices.

# Required Resources

3 Routers (Cisco 8200L with Cisco IOS XE Release 17.12.05c universal image or comparable)

Console cables to configure the Cisco IOS devices via the console ports

1 PC (Choice of operating system with a terminal emulation program installed)

Ethernet and serial cables as shown in the topology

# Instructions

  - Trouble Ticket 32.1.3.1

Scenario:

During an approved maintenance window, uRPF was applied on R1 as part of the phased security hardening plan. Shortly after the change, operations began receiving reports that systems within the 192.168.20.0/24 segment could no longer reach resources in the 192.168.10.0/24 segment. The change advisory board has asked the network team to investigate and restore full connectivity while keeping the new security control in place. You have console access to R1, R2, and R3. Validate the issue, identify the misconfiguration causing the reachability problem, correct it, and confirm that all routed interfaces and loopback networks can communicate as expected. Document your findings and the steps you took to resolve the incident.

### Cable the network as shown in the topology.

Attach the devices as shown in the topology diagram, and cable as necessary.

Use the commands listed below to load the configuration files for this trouble ticket:

Instructor Note: Commands for uploading the configuration are provided at the end of this document.

| Device | Command |
| --- | --- |
| R1 | copy flash:/enarsi/32.1.3.1-r1-config.txt running-config |
| R2 | copy flash:/enarsi/32.1.3.1-r2-config.txt running-config |
| R3 | copy flash:/enarsi/32.1.3.1-r3-config.txt running-config |

- Note: Passwords on all devices are cisco12345.

### Troubleshoot Ticket.

- Troubleshoot and repair the issue. All devices, including loopback addresses, should be able to ping each other.

### Complete the Ticket.

- After you have fixed the ticket, change the MOTD on Router R1 using the following command:

banner motd # This is $(hostname) FIXED from ticket <ticket number> #

- Verify that uRPF is enabled, configured correctly and all devices, including loopback addresses, can ping each other. Then save the configuration by issuing the wri command.

- Inform your instructor that you are have completed the ticket.

- After the instructor approves your solution for this ticket, issue the reset.now privileged EXEC command on each device. This script will clear your configurations and reload the devices.

Instructor Notes:

This trouble ticket contains one intentional error:

Static route on R3 is routing traffic for 192.168.10.0/24 from its Lo1 via S0/1/0 instead of G0/0/0. This is triggering uRPF to drop packets because the packets are being received on an unexpected interface.

- The command to fix these errors should be:

R3(config)# no ip route 192.168.10.0 255.255.255.0 s0/1/0

End of document

Uploading Configuration Files

Use the commands below to create the configuration files on the lab devices for each trouble ticket in this lab. The TCL script commands help create and copy the configurations. However, the configuration commands could also be copied and pasted directly into global config mode on each device. Simply remove the TCL script commands, enter the enable and configure t commands on the device, and copy and paste the configuration commands.

Important: The device requires a folder in flash named enarsi. Use the dir command to verify. If the folder is missing, then create it using the mkdir flash:/enarsi privileged EXEC command. For all switches, make sure the vlan.dat file is set to the default. Use the delete vlan.dat privileged EXEC command, if necessary.

- Reset scripts

These TCL scripts will completely clear and reload the device in preparation for the next ticket. Copy and paste the appropriate script to the appropriate device.

- Router Reset Script

tclsh

puts [ open "flash:/enarsi/reset.tcl" w+ ] {

typeahead "\n"

copy running-config startup-config

typeahead "\n"

erase startup-config

puts "Reloading the router"

typeahead "\n"

reload

}

tclquit

R1 Configuration File Scripts

!R1 - Trouble Ticket #1

tclsh

puts [ open "flash:/enarsi/32.1.3.1-r1-config.txt" w+ ] {

hostname R1

enable secret cisco12345

no ip domain lookup

banner motd # R1 Trouble ticket 32.1.3.1 #

line con 0

exec-timeout 0 0

logging synchronous

exit

interface g0/0/0

ip address 10.10.1.1 255.255.255.0

no shutdown

ip verify unicast source reachable-via rx

exit

interface s0/1/0

ip address 10.10.3.2 255.255.255.0

ip verify unicast source reachable-via rx

exit

interface lo1

ip address 192.168.10.10 255.255.255.0

exit

router eigrp 1

network 10.10.1.0 255.255.255.0

network 10.10.3.0 255.255.255.0

network 192.168.10.0 255.255.255.0

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

R2 Configuration File Scripts

!R2 - Trouble Ticket #1

tclsh

puts [ open "flash:/enarsi/32.1.3.1-r2-config.txt" w+ ] {

hostname R2

enable secret cisco12345

no ip domain lookup

banner motd # R2 Trouble ticket 32.1.3.1 #

line con 0

exec-timeout 0 0

logging synchronous

exit

interface g0/0/0

ip address 10.10.1.2 255.255.255.0

no shutdown

exit

interface g0/0/1

ip address 10.10.2.1 255.255.255.0

no shutdown

exit

router eigrp 1

network 10.10.1.0 255.255.255.0

network 10.10.2.0 255.255.255.0

exit

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit

R3 Configuration File Scripts

!R3 - Trouble Ticket #1

tclsh

puts [ open "flash:/enarsi/32.1.3.1-r3-config.txt" w+ ] {

hostname R3

enable secret cisco12345

no ip domain lookup

banner motd # R3 Trouble ticket 32.1.3.1 #

line con 0

exec-timeout 0 0

logging synchronous

exit

interface g0/0/0

ip address 10.10.2.2 255.255.255.0

no shutdown

exit

interface s0/1/0

ip address 10.10.3.1 255.255.255.0

exit

interface lo1

ip address 192.168.20.20 255.255.255.0

exit

router eigrp 1

network 10.10.2.0 255.255.255.0

network 10.10.3.0 255.255.255.0

network 192.168.20.0 255.255.255.0

exit

ip route 192.168.10.0 255.255.255.0 s0/1/0

alias exec reset.now tclsh flash:/enarsi/reset.tcl

end

}

tclquit