# Lab - Implement VRF-Lite (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

# Topology

# Addressing Table

| Device | Interface | IPv4 Address | IPv6 Address | IPv6 Link-Local |
| --- | --- | --- | --- | --- |
| R1 | G0/0/0 | 10.1.2.1/24 | 2001:db8:acad:1012::1/64 | fe80::1:1 |
| R1 | G0/0/1.5 | 10.1.2.1/24 | 2001:db8:acad:1012::1/64 | fe80::1:2 |
| R1 | G0/0/1.8 | 10.1.3.1/24 | 2001:db8:acad:1013::1/64 | fe80::1:4 |
| R1 | S0/1/0 | 10.1.3.1/25 | 2001:db8:acad:1013::1/64 | fe80::1:2 |
| R2 | G0/0/0 | 10.2.3.2/24 | 2001:db8:acad:1023::2/64 | fe80::2:1 |
| R2 | Loopback0 | 192.168.2.1/24 | 2001:db8:acad:2000::1/64 | fe80::2:2 |
| R3 | S0/1/0 | 10.1.3.3/25 | 2001:db8:acad:1013::3/64 | fe80::3:1 |
| R3 | Loopback0 | 192.168.3.1/27 | 2001:db8:acad:3000::1/64 | fe80::3:2 |
| D1 | G1/0/5 | 10.1.2.2/24 | 2001:db8:acad:1012::2/64 | fe80::d1:1 |
| D1 | VLAN 11 | 192.168.2.1/24 | 2001:db8:acad:2000::2/64 | fe80::d1:2 |
| D2 | G1/0/5 | 10.1.3.2/24 | 2001:db8:acad:1013::2/64 | fe80::d2:1 |
| D2 | VLAN 11 | 192.168.3.1/24 | 2001:db8:acad:3000::1/64 | fe80::d2:2 |

# Objectives

Part 1: Build the Network and Configure Basic Device Settings

Part 2: Configure and Verify VRF and Interface Addressing

Part 3: Configure and Verify Dynamic Routing for Reachability Inside Each VRF

# Background / Scenario

On a traditional router, all interfaces participate in a single, global routing table. That means every route learned or configured is visible to every interface, and any overlapping IP address space would immediately conflict. Service providers and large enterprises often need to virtualize a router so that it can act like multiple independent routers at once.

Virtual Routing and Forwarding (VRF) solves this by creating separate routing tables on the same device. Each table is an isolated routing instance with its own interfaces and routes. VRF-Lite delivers this capability without Multiprotocol Label Switching (MPLS) and is intended for local segmentation on a single router or a small network.

With VRF-Lite, you place interfaces into specific VRFs. Each VRF maintains its own routes, so identical prefixes can exist at the same time without interference. The router forwards based on the VRF tied to the incoming interface, which keeps customer or department traffic isolated. Traffic does not cross between VRFs by default, which prevents leakage. Reachability inside each VRF uses familiar methods such as static routes or dynamic routing protocols configured per VRF. Inter-VRF communication, if needed, must be configured explicitly.

In this lab, you will work on R1, acting as a service provider router that supports two customers using the same addressing scheme. Your task is to deploy VRF-Lite and static routing to ensure full reachability within each customer’s network.

Note: This lab is an exercise in developing, deploying, and verifying VRF-Lite, and does not reflect networking best practices.

Note: The routers and switches used with CCNP hands-on labs are Cisco 8200L, Cisco 9300L, and Cisco 9200L with Cisco IOS XE Release 17.12.05 (universalk9 image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs

Note: Ensure that the routers and switches have been erased and have no startup configurations. If you are unsure contact your instructor.

Note: The PCs used in this lab do not require addressing. They are needed to enable interface VLAN 11.

Instructor Note: Refer to the Instructor Lab Manual for the procedures to initialize and reload devices.

# Required Resources

3 Routers with a compatible serial interface (Cisco 8200L with Cisco IOS XE Release 17.12.05c universal image or comparable)

2 Switches (Cisco 9300L with Cisco IOS XE release 17.12.05 universal image or comparable)

1 Switch (Cisco 9200L with Cisco IOS XE release 17.12.05 universal image or comparable)

2 PCs (Windows with a terminal emulation program, such as Tera Term)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet and serial cables as shown in the topology

## Build the Network and Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings on all devices.

### Cable the network as shown in the topology.

Attach the devices as shown in the topology diagram, and cable as necessary.

### Configure basic settings for each device.

Console into each device, enter global configuration mode, and apply the basic settings. A command list for each device using the following startup configurations.

Open configuration window

Router R1

enable

configure terminal

hostname R1

no ip domain lookup

ipv6 unicast-routing

banner motd # R1, Implement VRF-Lite #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

exit

Router R2

enable

configure terminal

hostname R2

no ip domain lookup

ipv6 unicast-routing

banner motd # R2, Implement VRF-Lite #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

exit

interface g0/0/0

ip address 10.1.2.2 255.255.255.0

ipv6 address fe80::2:1 link-local

ipv6 address 2001:db8:acad:1012::2/64

ospfv3 100 ipv6 area 0

ospfv3 100 ipv4 area 0

no shutdown

exit

interface loopback 0

ip address 192.168.2.1 255.255.255.0

ipv6 address fe80::2:2 link-local

ipv6 address 2001:db8:acad:2000::1/64

ospfv3 network point-to-point

ospfv3 100 ipv4 area 0

ospfv3 100 ipv6 area 0

no shutdown

exit

router ospfv3 100

router-id 2.2.2.2

address-family ipv4 unicast

exit-address-family

address-family ipv6 unicast

exit-address-family

Router R3

enable

configure terminal

hostname R3

no ip domain lookup

ipv6 unicast-routing

banner motd # R3, Implement VRF-Lite #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

exit

interface loopback 0

ip address 192.168.3.1 255.255.255.0

ipv6 address fe80::3:2 link-local

ipv6 address 2001:db8:acad:3000::1/64

ospfv3 network point-to-point

ospfv3 100 ipv4 area 0

ospfv3 100 ipv6 area 0

no shutdown

exit

interface s0/1/0

ip address 10.1.3.2 255.255.255.0

ipv6 address fe80::3:1 link-local

ipv6 address 2001:db8:acad:1013::2/64

ospfv3 100 ipv4 area 0

ospfv3 100 ipv6 area 0

no shutdown

exit

router ospfv3 100

router-id 3.3.3.3

address-family ipv4 unicast

exit-address-family

address-family ipv6 unicast

exit-address-family

Switch D1

enable

configure terminal

hostname D1

no ip domain lookup

ip routing

ipv6 unicast-routing

banner motd # D1, Implement VRF-Lite #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

exit

interface range g1/0/1-24, g1/1/1-4, g0/0

shutdown

exit

interface g1/0/5

no switchport

ip address 10.1.2.2 255.255.255.0

ipv6 address fe80::d1:1 link-local

ipv6 address 2001:db8:acad:1012::2/64

ospfv3 200 ipv4 area 0

ospfv3 200 ipv6 area 0

no shutdown

exit

vlan 11

name LOCAL_VLAN

exit

interface vlan 11

ip address 192.168.2.1 255.255.255.0

ipv6 address fe80::d1:2 link-local

ipv6 address 2001:db8:acad:2000::1/64

ospfv3 200 ipv4 area 0

ospfv3 200 ipv6 area 0

no shutdown

exit

interface g1/0/23

switchport mode access

switchport access vlan 11

no shutdown

exit

router ospfv3 200

router-id 4.4.4.4

address-family ipv4 unicast

exit-address-family

address-family ipv6 unicast

exit-address-family

Switch D2

enable

configure terminal

hostname D2

no ip domain lookup

ip routing

ipv6 unicast-routing

banner motd # D2, Implement VRF-Lite #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

exit

interface range g1/0/1-24, g1/1/1-4, g0/0

shutdown

exit

interface g1/0/5

no switchport

ip address 10.1.3.2 255.255.255.0

ipv6 address fe80::d2:1 link-local

ipv6 address 2001:db8:acad:1013::2/64

ospfv3 200 ipv4 area 0

ospfv3 200 ipv6 area 0

no shutdown

exit

vlan 11

name LOCAL_VLAN

exit

interface vlan 11

ip address 192.168.3.1 255.255.255.0

ipv6 address fe80::d2:2 link-local

ipv6 address 2001:db8:acad:3000::1/64

ospfv3 200 ipv4 area 0

ospfv3 200 ipv6 area 0

no shutdown

exit

interface g1/0/23

switchport mode access

switchport access vlan 11

no shutdown

exit

router ospfv3 200

router-id 5.5.5.5

address-family ipv4 unicast

exit-address-family

address-family ipv6 unicast

exit-address-family

Switch A1

enable

configure terminal

hostname A1

no ip domain lookup

banner motd # A1, Implement VRF-Lite #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

exit

interface range g1/0/1-24, g1/1/1-4, g0/0

shutdown

exit

vlan 5

name D1

exit

vlan 8

name D2

exit

interface g1/0/11

switchport mode trunk

switchport nonegotiate

no shutdown

exit

interface g1/0/1

switchport mode access

switchport access vlan 5

no shutdown

exit

interface g1/0/3

switchport mode access

switchport access vlan 8

no shutdown

Set the clock on each router to UTC time.

Save the running configuration to startup-config.

Close configuration window

## Configure and Verify VRF and Interface Addressing

In Part 2, you will configure and verify VRF-Lite on R1. The other devices, R2, R3, D1, D2, and A1 require no additional configuration. Once again, the configuration being used here is not meant to represent best practice, but to assess your ability to complete the required configurations.

### On R1, create the required VRFs.

Create the CustomerA and CustomerB VRFs, and initialize them for both IPv4 and IPv6. The VRF names are case sensitive.

Open configuration window

R1(config)# vrf definition CustomerA

R1(config-vrf)# address-family ipv4

R1(config-vrf-af)# address-family ipv6

R1(config-vrf-af)# exit

R1(config-vrf)# vrf definition CustomerB

R1(config-vrf)# address-family ipv4

R1(config-vrf-af)# address-family ipv6

R1(config-vrf-af)# exit

Configure interfaces G0/0/0 and S0/1/0 for the CustomerA network.

R1(config)# interface g0/0/0

R1(config-if)# vrf forwarding CustomerA

R1(config-if)# ip address 10.1.2.1 255.255.255.0

R1(config-if)# ipv6 address fe80::1:1 link-local

R1(config-if)# ipv6 address 2001:db8:acad:1012::1/64

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# interface s0/1/0

R1(config-if)# vrf forwarding CustomerA

R1(config-if)# ip address 10.1.3.1 255.255.255.0

R1(config-if)# ipv6 address fe80::1:4 link-local

R1(config-if)# ipv6 address 2001:db8:acad:1013::1/64

R1(config-if)# no shutdown

R1(config-if)# exit

Configure R1 interface G0/0/1 to support the CustomerB networks. G0/0/1 will be performing inter-VLAN routing between VLANs 5 and 8.

R1(config)# interface g0/0/1

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# interface g0/0/1.5

R1(config-subif)# encapsulation dot1q 5

R1(config-subif)# vrf forwarding CustomerB

R1(config-subif)# ip address 10.1.2.1 255.255.255.0

R1(config-subif)# ipv6 address fe80::1:2 link-local

R1(config-subif)# ipv6 address 2001:db8:acad:1012::1/64

R1(config-subif)# exit

R1(config)# interface g0/0/1.8

R1(config-subif)# encapsulation dot1q 8

R1(config-subif)# vrf forwarding CustomerB

R1(config-subif)# ip address 10.1.3.1 255.255.255.0

R1(config-subif)# ipv6 address fe80::1:3 link-local

R1(config-subif)# ipv6 address 2001:db8:acad:1013::1/64

R1(config-subif)# exit

Configure loopback interfaces on R1 to simulate access to external networks.

R1(config)# interface Loopback1

R1(config-if)# description simulated access to external networks

R1(config-if)# vrf forwarding CustomerA

R1(config-if)# ip address 64.100.1.1 255.255.255.252

R1(config-if)# ipv6 address 2001:db8:acad:64:100:1::1/64

R1(config-if)# interface Loopback2

R1(config-if)# description simulated access to external networks

R1(config-if)# vrf forwarding CustomerB

R1(config-if)# ip address 64.100.1.1 255.255.255.252

R1(config-if)# ipv6 address 2001:db8:acad:64:100:1::1/64

### Verify the VRF-Lite configuration.

Verify the routing and forwarding information.

R1# show ip vrf

Name                             Default RD            Interfaces

CustomerA                        <not set>             Gi0/0/0

Se0/1/0

Lo1

CustomerB                        <not set>             Gi0/0/1.5

Gi0/0/1.8

Lo2

Verify the interface assignments using the show ip vrf interfaces command.

R1# show ip vrf interfaces

Interface              IP-Address      VRF                              Protocol

Gi0/0/0                10.1.2.1        CustomerA                        up

Se0/1/0                10.1.3.1        CustomerA                        up

Lo1                    64.100.1.1      CustomerA                        up

Gi0/0/1.5              10.1.2.1        CustomerB                        up

Gi0/0/1.8              10.1.3.1        CustomerB                        up

Lo2                    64.100.1.1      CustomerB                        up

Verify the VRF routing tables with the show ip route vrf vrf_name and show ipv6 route vrf vrf_name command.

R1# show ip route vrf CustomerA | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 4 subnets, 2 masks

C        10.1.2.0/24 is directly connected, GigabitEthernet0/0/0

L        10.1.2.1/32 is directly connected, GigabitEthernet0/0/0

C        10.1.3.0/24 is directly connected, Serial0/1/0

L        10.1.3.1/32 is directly connected, Serial0/1/0

64.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C        64.100.1.0/30 is directly connected, Loopback1

L        64.100.1.1/32 is directly connected, Loopback1

R1# show ipv6 route vrf CustomerB

IPv6 Routing Table - CustomerB - 5 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

<output omitted>

C   2001:DB8:ACAD:64::/64 [0/0]

via Loopback2, directly connected

L   2001:DB8:ACAD:64:100:1:0:1/128 [0/0]

via Loopback2, receive

C   2001:DB8:ACAD:1012::/64 [0/0]

via GigabitEthernet0/0/1.5, directly connected

L   2001:DB8:ACAD:1012::1/128 [0/0]

via GigabitEthernet0/0/1.5, receive

C   2001:DB8:ACAD:1013::/64 [0/0]

via GigabitEthernet0/0/1.8, directly connected

L   2001:DB8:ACAD:1013::1/128 [0/0]

via GigabitEthernet0/0/1.8, receive

L   FF00::/8 [0/0]

via Null0, receive

Verify next-hop reachability within each vrf with the ping vrf vrf_name address command.

R1# ping vrf CustomerA 10.1.2.2

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.1.2.2, timeout is 2 seconds:

.!!!!

Success rate is 80 percent (4/5), round-trip min/avg/max = 1/1/1 ms

R1# ping vrf CustomerA 2001:db8:acad:1012::2

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 2001:DB8:ACAD:1012::2, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/2/10 ms

R1#  ping vrf CustomerA 10.1.3.2

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.1.3.2, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 2/2/3 ms

R1# ping vrf CustomerA 2001:db8:acad:1013::2

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 2001:DB8:ACAD:1013::2, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 2/2/3 ms

Close configuration window

## Configure and Verify Dynamic Routing for Reachability Inside Each VRF

In Part 3, you will configure OSFP so that all networks are reachable within their respective VRFs. At the end of this part, R1 should be able to successfully source a ping from interface loopback0 to R3 interface loopback0, and D1 should be able to successfully source a ping from interface VLAN 11 to D2 interface VLAN 11. Furthermore, you will configure a default route to access an external network simulated the loopback interfaces. Once again, the way these networks are being implemented is not meant to represent best practice, but to assess your ability to complete the required configurations.

### Verify that distant networks are not reachable within each VRF.

In this step, you will check to make sure that distant networks are not reachable from R1 within each VRF.

On R1, issue the commands ping vrf CustomerA 192.168.2.1 and ping vrf CustomerA 192.168.3.1. Neither should succeed.

Open configuration window

R1# ping vrf CustomerA 192.168.2.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.2.1, timeout is 2 seconds:

.....

Success rate is 0 percent (0/5)

R1# ping vrf CustomerA 192.168.3.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.3.1, timeout is 2 seconds:

.....

Success rate is 0 percent (0/5)

On R1, issue the commands ping vrf CustomerA 2001:db8:acad:2000::1 and ping vrf CustomerA 2001:db8:acad:3000::1. Neither should succeed.

R1# ping vrf CustomerA 2001:db8:acad:2000::1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 2001:DB8:ACAD:2000::1, timeout is 2 seconds:

% No valid route for destination

Success rate is 0 percent (0/1)

R1# ping vrf CustomerA 2001:db8:acad:3000::1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 2001:DB8:ACAD:3000::1, timeout is 2 seconds:

% No valid route for destination

Success rate is 0 percent (0/1)

On R1, issue the commands ping vrf CustomerB 192.168.2.1 and ping vrf CustomerB 192.168.3.1. Neither should succeed.

R1# ping vrf CustomerB 192.168.2.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.2.1, timeout is 2 seconds:

.....

Success rate is 0 percent (0/5)

R1# ping vrf CustomerB 192.168.3.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.3.1, timeout is 2 seconds:

.....

Success rate is 0 percent (0/5)

On R1, issue the commands ping vrf CustomerB 2001:db8:acad:2000::1 and ping vrf CustomerB 2001:db8:acad:3000::1. Neither should succeed.

R1# ping vrf CustomerB 2001:db8:acad:2000::1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 2001:DB8:ACAD:2000::1, timeout is 2 seconds:

% No valid route for destination

Success rate is 0 percent (0/1)

R1# ping vrf CustomerB 2001:db8:acad:3000::1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 2001:DB8:ACAD:3000::1, timeout is 2 seconds:

% No valid route for destination

Success rate is 0 percent (0/1)

### Configure OSFP at R1 for each VRF.

In this step, you will configure R1 so that it can reach distant networks in each VRF. The neighbor systems (D1, D2, R2, and R3) have OSPF routes already configured, so as soon as you correctly install these dynamic routes, there will be full reachability within each VRF.

On R1, configure OSPF routes for the distant networks in the CustomerA VRF using OSPFv3.

R1(config)# interface GigabitEthernet0/0/0

R1(config-if)# ospfv3 100 ipv4 area 0

R1(config-if)# ospfv3 100 ipv6 area 0

R1(config-if)# interface Serial0/1/0

R1(config-if)# ospfv3 100 ipv4 area 0

R1(config-if)# ospfv3 100 ipv6 area 0

R1(config-if)# router ospfv3 100

R1(config-router)# router-id 1.1.1.1

R1(config-router)# address-family ipv4 unicast vrf CustomerA

R1(config-router-af)# capability vrf-lite

R1(config-router-af)# exit-address-family

R1(config-router)# address-family ipv6 unicast vrf CustomerA

R1(config-router-af)# capability vrf-lite

R1(config-router-af)# exit-address-family

R1(config-router)# exit

Use the example above to correctly configure the OSPF routes for the CustomerB network.

R1(config)# interface GigabitEthernet0/0/1.5

R1(config-subif)# ospfv3 200 ipv4 area 0

R1(config-subif)# ospfv3 200 ipv6 area 0

R1(config-subif)# interface GigabitEthernet0/0/1.8

R1(config-subif)# ospfv3 200 ipv4 area 0

R1(config-subif)# ospfv3 200 ipv6 area 0

R1(config-subif)# router ospfv3 200

R1(config-router)# router-id 1.1.1.1

R1(config-router)# address-family ipv4 unicast vrf CustomerB

R1(config-router-af)# capability vrf-lite

R1(config-router-af)# exit-address-family

R1(config-router)# address-family ipv6 unicast vrf CustomerB

R1(config-router-af)# capability vrf-lite

R1(config-router-af)# exit-address-family

R1(config-router)# exit

### Verify OSPF routes.

On R1, verify the OSPF routes in routing table for CustomerA.

R1# show ip route vrf CustomerA ospfv3 | begin Gateway

Gateway of last resort is not set

O     192.168.2.0/24 [110/2] via 10.1.2.2, 00:23:07, GigabitEthernet0/0/0

O     192.168.3.0/24 [110/51] via 10.1.3.2, 00:23:07, Serial0/1/0

On R2 and R3, display and verify the OSPF routes in the routing table.

R2# show ip route ospfv3 | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 3 subnets, 2 masks

O        10.1.3.0/24 [110/51] via 10.1.2.1, 00:25:47, GigabitEthernet0/0/0

O     192.168.3.0/24 [110/52] via 10.1.2.1, 00:25:43, GigabitEthernet0/0/0

R3# show ip route ospfv3 | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 3 subnets, 2 masks

O        10.1.2.0/24 [110/50] via 10.1.3.1, 00:26:16, Serial0/1/0

O     192.168.2.0/24 [110/51] via 10.1.3.1, 00:26:16, Serial0/1/0

Verify the OSPF routes for CustomerB.

R1# show ip route vrf CustomerB ospfv3 | begin Gateway

Gateway of last resort is not set

O     192.168.2.0/24 [110/2] via 10.1.2.2, 00:27:42, GigabitEthernet0/0/1.5

O     192.168.3.0/24 [110/2] via 10.1.3.2, 00:27:42, GigabitEthernet0/0/1.8

D1# show ip route ospfv3 | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 3 subnets, 2 masks

O        10.1.3.0/24 [110/2] via 10.1.2.1, 00:28:33, GigabitEthernet1/0/5

O     192.168.3.0/24 [110/3] via 10.1.2.1, 00:28:28, GigabitEthernet1/0/5

D2# show ip route ospfv3 | begin Gateway

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 3 subnets, 2 masks

O        10.1.2.0/24 [110/2] via 10.1.3.1, 00:29:08, GigabitEthernet1/0/5

O     192.168.2.0/24 [110/3] via 10.1.3.1, 00:29:08, GigabitEthernet1/0/5

Repeat the above steps for IPv6 OSPF routes.

R1# show ipv6 route vrf CustomerA ospf

<output omitted>

O   2001:DB8:ACAD:2000::/64 [110/2]

via FE80::2:1, GigabitEthernet0/0/0

O   2001:DB8:ACAD:3000::/64 [110/51]

via FE80::3:1, Serial0/1/0

R2# show ipv6 route ospf

<output omitted>

O   2001:DB8:ACAD:1013::/64 [110/51]

via FE80::1:1, GigabitEthernet0/0/0

O   2001:DB8:ACAD:3000::/64 [110/52]

via FE80::1:1, GigabitEthernet0/0/0

R3# show ipv6 route ospf

<output omitted>

O   2001:DB8:ACAD:1012::/64 [110/50]

via FE80::1:4, Serial0/1/0

O   2001:DB8:ACAD:2000::/64 [110/51]

via FE80::1:4, Serial0/1/0

Because all the interfaces are participating in the VRF, there are no routes in global routing table.

R1# show ip route | begin Gateway

Gateway of last resort is not set

R1# show ipv6 route

IPv6 Routing Table - default - 1 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, H - NHRP, HG - NHRP registered

Hg - NHRP registration summary, HE - NHRP External, I1 - ISIS L1

I2 - ISIS L2, IA - ISIS interarea, IS - ISIS summary, D - EIGRP

EX - EIGRP external, ND - ND Default, NDp - ND Prefix, DCE - Destination

NDr - Redirect, O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1

OE2 - OSPF ext 2, ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2

a - Application, m - OMP

L   FF00::/8 [0/0]

via Null0, receive

### Verify full reachability within each VRF.

On R2, ping the IPv4 and IPv6 addresses of R3 interface Loopback0 using a source address of R2 interface Loopback0. All pings should be successful.

R2# ping 192.168.3.1 source loopback0

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.3.1, timeout is 2 seconds:

Packet sent with a source address of 192.168.2.1

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 2/2/3 ms

R2# ping 2001:db8:acad:3000::1 source loopback0

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 2001:DB8:ACAD:3000::1, timeout is 2 seconds:

Packet sent with a source address of 2001:DB8:ACAD:2000::1

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 2/2/2 ms

On D1, ping the IPv4 and IPv6 addresses of D2 interface VLAN 11 using a source address of D1 interface VLAN 11. All pings should be successful.

D1# ping 192.168.3.1 source vlan11

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.3.1, timeout is 2 seconds:

Packet sent with a source address of 192.168.2.1

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/4/9 ms

D1# ping 2001:db8:acad:3000::1 source vlan11

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 2001:DB8:ACAD:3000::1, timeout is 2 seconds:

Packet sent with a source address of 2001:DB8:ACAD:2000::1

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 0/5/17 ms

### Configure and distribute default route for each VRF.

In this step, you will configure default routes on R1 and advertise the default routes into each VRF so that the devices in each VRF can reach distant networks simulated by the loopback interfaces on R1.

On R1, configure the default routes for each VRF.

R1(config)# ip route vrf CustomerA 0.0.0.0 0.0.0.0 lo 1 64.100.1.2

R1(config)# ipv6 route vrf CustomerA ::/0 lo 1 2001:db8:acad:64:100:1::2

R1(config)# ip route vrf CustomerB 0.0.0.0 0.0.0.0 lo 2 64.100.1.2

R1(config)# ipv6 route vrf CustomerB ::/0 lo 2 2001:db8:acad:64:100:1::2

Advertise the default routes into the OSPF process 100 and 200 so each VRF member learns the default routes.

R1(config)# router ospfv3 100

R1(config-router)# address-family ipv4 unicast vrf CustomerA

R1(config-router-af)# default-information originate

R1(config-router-af)# exit-address-family

R1(config-router)# address-family ipv6 unicast vrf CustomerA

R1(config-router-af)# default-information originate

R1(config-router-af)# exit-address-family

R1(config-router)# router ospfv3 200

R1(config-router)# address-family ipv4 unicast vrf CustomerB

R1(config-router-af)# default-information originate

R1(config-router-af)# exit-address-family

R1(config-router)# address-family ipv6 unicast vrf CustomerB

R1(config-router-af)# default-information originate

R1(config-router-af)# exit-address-family

Verify that default routes have been advertised with each VRF.

R1# show ip route vrf CustomerA | begin Gateway

Gateway of last resort is 64.100.1.2 to network 0.0.0.0

S*    0.0.0.0/0 [1/0] via 64.100.1.2, Loopback1

10.0.0.0/8 is variably subnetted, 4 subnets, 2 masks

C        10.1.2.0/24 is directly connected, GigabitEthernet0/0/0

L        10.1.2.1/32 is directly connected, GigabitEthernet0/0/0

C        10.1.3.0/24 is directly connected, Serial0/1/0

L        10.1.3.1/32 is directly connected, Serial0/1/0

64.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C        64.100.1.0/30 is directly connected, Loopback1

L        64.100.1.1/32 is directly connected, Loopback1

O     192.168.2.0/24 [110/2] via 10.1.2.2, 00:10:22, GigabitEthernet0/0/0

O     192.168.3.0/24 [110/51] via 10.1.3.2, 00:10:21, Serial0/1/0

R1# show ipv6 route vrf CustomerA

<output omitted>

S   ::/0 [1/0]

via 2001:DB8:ACAD:64:100:1:0:2, Loopback1

C   2001:DB8:ACAD:64::/64 [0/0]

via Loopback2, directly connected

L   2001:DB8:ACAD:64:100:1:0:1/128 [0/0]

via Loopback2, receive

C   2001:DB8:ACAD:1012::/64 [0/0]

via GigabitEthernet0/0/1.5, directly connected

L   2001:DB8:ACAD:1012::1/128 [0/0]

via GigabitEthernet0/0/1.5, receive

C   2001:DB8:ACAD:1013::/64 [0/0]

via GigabitEthernet0/0/1.8, directly connected

L   2001:DB8:ACAD:1013::1/128 [0/0]

via GigabitEthernet0/0/1.8, receive

O   2001:DB8:ACAD:2000::/64 [110/2]

via FE80::D1:1, GigabitEthernet0/0/1.5

O   2001:DB8:ACAD:3000::/64 [110/2]

via FE80::D2:1, GigabitEthernet0/0/1.8

L   FF00::/8 [0/0]

via Null0, receive

R2# show ip route | begin Gateway

Gateway of last resort is 10.1.2.1 to network 0.0.0.0

O*E2  0.0.0.0/0 [110/1] via 10.1.2.1, 00:16:15, GigabitEthernet0/0/0

10.0.0.0/8 is variably subnetted, 3 subnets, 2 masks

C        10.1.2.0/24 is directly connected, GigabitEthernet0/0/0

L        10.1.2.2/32 is directly connected, GigabitEthernet0/0/0

O        10.1.3.0/24 [110/51] via 10.1.2.1, 00:16:15, GigabitEthernet0/0/0

192.168.2.0/24 is variably subnetted, 2 subnets, 2 masks

C        192.168.2.0/24 is directly connected, Loopback0

L        192.168.2.1/32 is directly connected, Loopback0

O     192.168.3.0/24 [110/52] via 10.1.2.1, 00:14:44, GigabitEthernet0/0/0

R3# show ipv6 route

<output omitted>

OE2 ::/0 [110/1], tag 100

via FE80::1:4, Serial0/1/0

O   2001:DB8:ACAD:1012::/64 [110/50]

via FE80::1:4, Serial0/1/0

C   2001:DB8:ACAD:1013::/64 [0/0]

via Serial0/1/0, directly connected

L   2001:DB8:ACAD:1013::2/128 [0/0]

via Serial0/1/0, receive

O   2001:DB8:ACAD:2000::/64 [110/51]

via FE80::1:4, Serial0/1/0

C   2001:DB8:ACAD:3000::/64 [0/0]

via Loopback0, directly connected

L   2001:DB8:ACAD:3000::1/128 [0/0]

via Loopback0, receive

L   FF00::/8 [0/0]

via Null0, receive

Review the routing protocol parameters and statistics for each VRF.

R1# show ip protocols vrf CustomerA

*** IP Routing is NSF aware ***

Routing Protocol is "ospfv3 100"

Outgoing update filter list for all interfaces is not set

Incoming update filter list for all interfaces is not set

Router ID 1.1.1.1

Autonomous system boundary router

Number of areas: 1 normal, 0 stub, 0 nssa

Interfaces (Area 0):

Serial0/1/0

GigabitEthernet0/0/0

Maximum path: 4

Routing Information Sources:

Gateway         Distance      Last Update

3.3.3.3              110      00:07:48

2.2.2.2              110      00:09:16

Distance: (default is 110)

R1# show ip protocols vrf CustomerB

*** IP Routing is NSF aware ***

Routing Protocol is "ospfv3 200"

Outgoing update filter list for all interfaces is not set

Incoming update filter list for all interfaces is not set

Router ID 1.1.1.1

Autonomous system boundary router

Number of areas: 1 normal, 0 stub, 0 nssa

Interfaces (Area 0):

GigabitEthernet0/0/1.8

GigabitEthernet0/0/1.5

Maximum path: 4

Routing Information Sources:

Gateway         Distance      Last Update

5.5.5.5              110      00:02:14

4.4.4.4              110      00:04:26

Distance: (default is 110)

Verify that R2, R3, D1 and D2 can ping the simulated access to the external network.

R2# ping 64.100.1.1 source lo0

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 64.100.1.1, timeout is 2 seconds:

Packet sent with a source address of 192.168.2.1

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/12/59 ms

D2# ping 2001:db8:acad:64:100:1::1 source vlan 11

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 2001:DB8:ACAD:64:100:1:0:1, timeout is 2 seconds:

Packet sent with a source address of 2001:DB8:ACAD:3000::1

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/1 ms

R3# ping 2001:db8:acad:64:100:1::1 source lo 0

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 2001:DB8:ACAD:64:100:1:0:1, timeout is 2 seconds:

Packet sent with a source address of 2001:DB8:ACAD:3000::1

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 2/2/4 ms

D2# ping 64.100.1.1 source vlan 11

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 64.100.1.1, timeout is 2 seconds:

Packet sent with a source address of 192.168.3.1

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/1 ms

End of document

# Device Configs - Final

# Router R1

R1# show run brief

Building configuration...

Current configuration : 3854 bytes

!

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

!

hostname R1

!

boot-start-marker

boot-end-marker

!

vrf definition CustomerA

!

address-family ipv4

exit-address-family

!

address-family ipv6

exit-address-family

!

vrf definition CustomerB

!

address-family ipv4

exit-address-family

!

address-family ipv6

exit-address-family

!

no aaa new-model

!

no ip domain lookup

!

login on-success log

!

ipv6 unicast-routing

!

subscriber templating

vtp domain TSHOOT

vtp mode transparent

multilink bundle-name authenticated

!

pae

!

diagnostic bootup level minimal

!

spanning-tree extend system-id

!

redundancy

mode none

!

interface Loopback1

description simulated access to external networks

vrf forwarding CustomerA

ip address 64.100.1.1 255.255.255.252

ipv6 address 2001:DB8:ACAD:64:100:1:0:1/64

!

interface Loopback2

description simulated access to external networks

vrf forwarding CustomerB

ip address 64.100.1.1 255.255.255.252

ipv6 address 2001:DB8:ACAD:64:100:1:0:1/64

!

interface GigabitEthernet0/0/0

vrf forwarding CustomerA

ip address 10.1.2.1 255.255.255.0

negotiation auto

ipv6 address FE80::1:1 link-local

ipv6 address 2001:DB8:ACAD:1012::1/64

ospfv3 100 ipv6 area 0

ospfv3 100 ipv4 area 0

!

interface GigabitEthernet0/0/1

no ip address

negotiation auto

!

interface GigabitEthernet0/0/1.5

encapsulation dot1Q 5

vrf forwarding CustomerB

ip address 10.1.2.1 255.255.255.0

ipv6 address FE80::1:2 link-local

ipv6 address 2001:DB8:ACAD:1012::1/64

ospfv3 200 ipv6 area 0

ospfv3 200 ipv4 area 0

!

interface GigabitEthernet0/0/1.8

encapsulation dot1Q 8

vrf forwarding CustomerB

ip address 10.1.3.1 255.255.255.0

ipv6 address FE80::1:3 link-local

ipv6 address 2001:DB8:ACAD:1013::1/64

ospfv3 200 ipv6 area 0

ospfv3 200 ipv4 area 0

!

interface GigabitEthernet0/0/2

no ip address

negotiation auto

!

interface GigabitEthernet0/0/3

no ip address

negotiation auto

!

interface Serial0/1/0

vrf forwarding CustomerA

ip address 10.1.3.1 255.255.255.0

ipv6 address FE80::1:4 link-local

ipv6 address 2001:DB8:ACAD:1013::1/64

ospfv3 100 ipv6 area 0

ospfv3 100 ipv4 area 0

!

interface Serial0/1/1

no ip address

!

router ospfv3 100

router-id 1.1.1.1

!

address-family ipv4 unicast vrf CustomerA

default-information originate

capability vrf-lite

exit-address-family

!

address-family ipv6 unicast vrf CustomerA

default-information originate

capability vrf-lite

exit-address-family

!

router ospfv3 200

router-id 1.1.1.1

!

address-family ipv4 unicast vrf CustomerB

default-information originate

capability vrf-lite

exit-address-family

!

address-family ipv6 unicast vrf CustomerB

default-information originate

capability vrf-lite

exit-address-family

!

ip forward-protocol nd

no ip http server

ip http secure-server

!

ip route vrf CustomerA 0.0.0.0 0.0.0.0 Loopback1 64.100.1.2

ip route vrf CustomerB 0.0.0.0 0.0.0.0 Loopback2 64.100.1.2

ip ssh bulk-mode 131072

!

ipv6 route vrf CustomerB ::/0 Loopback2 2001:DB8:ACAD:64:100:1:0:2

ipv6 route vrf CustomerA ::/0 Loopback1 2001:DB8:ACAD:64:100:1:0:2

!

control-plane

!

banner motd ^C R1, Implement VRF-Lite ^C

!

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line aux 0

line vty 0 4

exec-timeout 0 0

privilege level 15

password cisco123

logging synchronous

login

transport input ssh

line vty 5 14

login

transport input ssh

!

end

# Router R2

R2# show run brief

Building configuration...

Current configuration : 2006 bytes

!

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

!

hostname R2

!

boot-start-marker

boot-end-marker

!

!

no aaa new-model

!

no ip domain lookup

!

login on-success log

!

ipv6 unicast-routing

!

!

subscriber templating

multilink bundle-name authenticated

!

pae

!

diagnostic bootup level minimal

!

spanning-tree extend system-id

!

redundancy

mode none

!

interface Loopback0

ip address 192.168.2.1 255.255.255.0

ipv6 address FE80::2:2 link-local

ipv6 address 2001:DB8:ACAD:2000::1/64

ospfv3 network point-to-point

ospfv3 100 ipv4 area 0

ospfv3 100 ipv6 area 0

!

interface GigabitEthernet0/0/0

ip address 10.1.2.2 255.255.255.0

negotiation auto

ipv6 address FE80::2:1 link-local

ipv6 address 2001:DB8:ACAD:1012::2/64

ospfv3 100 ipv4 area 0

ospfv3 100 ipv6 area 0

!

interface GigabitEthernet0/0/1

no ip address

negotiation auto

!

interface GigabitEthernet0/0/2

no ip address

negotiation auto

!

interface GigabitEthernet0/0/3

no ip address

negotiation auto

!

router ospfv3 100

router-id 2.2.2.2

!

address-family ipv4 unicast

exit-address-family

!

address-family ipv6 unicast

exit-address-family

!

ip forward-protocol nd

no ip http server

ip http secure-server

!

ip ssh bulk-mode 131072

!

control-plane

!

banner motd ^C R2, Implement VRF-Lite ^C

!

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line aux 0

line vty 0 4

exec-timeout 0 0

privilege level 15

password cisco123

logging synchronous

login

transport input ssh

line vty 5 14

login

transport input ssh

!

end

# Router R3

R3# show run brief

Building configuration...

Current configuration : 2239 bytes

!

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

!

hostname R3

!

boot-start-marker

boot-end-marker

!

no aaa new-model

!

no ip domain lookup

!

login on-success log

!

ipv6 unicast-routing

!

subscriber templating

multilink bundle-name authenticated

!

pae

!

diagnostic bootup level minimal

!

spanning-tree extend system-id

!

redundancy

mode none

!

interface Loopback0

ip address 192.168.3.1 255.255.255.0

ipv6 address FE80::3:2 link-local

ipv6 address 2001:DB8:ACAD:3000::1/64

ospfv3 network point-to-point

ospfv3 100 ipv6 area 0

ospfv3 100 ipv4 area 0

!

interface GigabitEthernet0/0/0

no ip address

negotiation auto

!

interface GigabitEthernet0/0/1

no ip address

negotiation auto

!

interface GigabitEthernet0/0/2

no ip address

negotiation auto

!

interface GigabitEthernet0/0/3

no ip address

negotiation auto

!

interface Serial0/1/0

ip address 10.1.3.2 255.255.255.0

ipv6 address FE80::3:1 link-local

ipv6 address 2001:DB8:ACAD:1013::2/64

ospfv3 100 ipv6 area 0

ospfv3 100 ipv4 area 0

!

interface Serial0/1/1

no ip address

!

router ospfv3 100

router-id 3.3.3.3

!

address-family ipv4 unicast

exit-address-family

!

address-family ipv6 unicast

exit-address-family

!

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

!

ip ssh bulk-mode 131072

!

control-plane

!

banner motd ^C R3, Implement VRF-Lite ^C

!

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line aux 0

line vty 0 4

exec-timeout 0 0

privilege level 15

password cisco123

logging synchronous

login

transport input ssh

line vty 5 14

login

transport input ssh

!

end

# Switch D1

D1# show run brief

Building configuration...

Current configuration : 5871 bytes

!

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform punt-keepalive disable-kernel-core

!

hostname D1

!

vrf definition Mgmt-vrf

!

address-family ipv4

exit-address-family

!

address-family ipv6

exit-address-family

!

no aaa new-model

switch 1 provision c9300l-24t-4g

!

ip routing

!

no ip domain lookup

!

login on-success log

ipv6 nd cache expire refresh

ipv6 unicast-routing

!

diagnostic bootup level minimal

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

redundancy

mode sso

crypto engine compliance shield disable

!

transceiver type all

monitoring

!

class-map match-any system-cpp-police-ewlc-control

description EWLC Control

class-map match-any system-cpp-police-topology-control

description Topology control

class-map match-any system-cpp-police-sw-forward

description Sw forwarding, L2 LVX data packets, LOGGING, Transit Traffic

class-map match-any system-cpp-default

description EWLC Data, Inter FED Traffic

class-map match-any system-cpp-police-sys-data

description Openflow, Exception, EGR Exception, NFL Sampled Data, RPF Failed

class-map match-any system-cpp-police-punt-webauth

description Punt Webauth

class-map match-any system-cpp-police-l2lvx-control

description L2 LVX control packets

class-map match-any system-cpp-police-forus

description Forus Address resolution and Forus traffic

class-map match-any system-cpp-police-multicast-end-station

description MCAST END STATION

class-map match-any system-cpp-police-high-rate-app

description High Rate Applications

class-map match-any system-cpp-police-multicast

description MCAST Data

class-map match-any system-cpp-police-l2-control

description L2 control

class-map match-any system-cpp-police-dot1x-auth

description DOT1X Auth

class-map match-any system-cpp-police-data

description ICMP redirect, ICMP_GEN and BROADCAST

class-map match-any system-cpp-police-stackwise-virt-control

description Stackwise Virtual OOB

class-map match-any non-client-nrt-class

class-map match-any system-cpp-police-routing-control

description Routing control and Low Latency

class-map match-any system-cpp-police-protocol-snooping

description Protocol snooping

class-map match-any system-cpp-police-dhcp-snooping

description DHCP snooping

class-map match-any system-cpp-police-ios-routing

description L2 control, Topology control, Routing control, Low Latency

class-map match-any system-cpp-police-system-critical

description System Critical and Gold Pkt

class-map match-any system-cpp-police-ios-feature

description ICMPGEN,BROADCAST,ICMP,L2LVXCntrl,ProtoSnoop,PuntWebauth,MCASTData,Transit,DOT1XAuth,Swfwd,LOGGING,L2LVXData,ForusTraffic,ForusARP,McastEndStn,Openflow,Exception,EGRExcption,NflSampled,RpfFailed

!

policy-map system-cpp-policy

!

interface GigabitEthernet0/0

vrf forwarding Mgmt-vrf

no ip address

shutdown

negotiation auto

!

interface GigabitEthernet1/0/1

shutdown

!

interface GigabitEthernet1/0/2

shutdown

!

interface GigabitEthernet1/0/3

shutdown

!

interface GigabitEthernet1/0/4

shutdown

!

interface GigabitEthernet1/0/5

no switchport

ip address 10.1.2.2 255.255.255.0

ipv6 address FE80::D1:1 link-local

ipv6 address 2001:DB8:ACAD:1012::2/64

ospfv3 200 ipv6 area 0

ospfv3 200 ipv4 area 0

!

interface GigabitEthernet1/0/6

shutdown

!

interface GigabitEthernet1/0/7

shutdown

!

interface GigabitEthernet1/0/8

shutdown

!

interface GigabitEthernet1/0/9

shutdown

!

interface GigabitEthernet1/0/10

shutdown

!

interface GigabitEthernet1/0/11

shutdown

!

interface GigabitEthernet1/0/12

shutdown

!

interface GigabitEthernet1/0/13

shutdown

!

interface GigabitEthernet1/0/14

shutdown

!

interface GigabitEthernet1/0/15

shutdown

!

interface GigabitEthernet1/0/16

shutdown

!

interface GigabitEthernet1/0/17

shutdown

!

interface GigabitEthernet1/0/18

shutdown

!

interface GigabitEthernet1/0/19

shutdown

!

interface GigabitEthernet1/0/20

shutdown

!

interface GigabitEthernet1/0/21

shutdown

!

interface GigabitEthernet1/0/22

shutdown

!

interface GigabitEthernet1/0/23

switchport access vlan 11

switchport mode access

!

interface GigabitEthernet1/0/24

shutdown

!

interface GigabitEthernet1/1/1

shutdown

!

interface GigabitEthernet1/1/2

shutdown

!

interface GigabitEthernet1/1/3

shutdown

!

interface GigabitEthernet1/1/4

shutdown

!

interface AppGigabitEthernet1/0/1

!

interface Vlan1

no ip address

!

interface Vlan11

ip address 192.168.2.1 255.255.255.0

ipv6 address FE80::D1:2 link-local

ipv6 address 2001:DB8:ACAD:2000::1/64

ospfv3 200 ipv6 area 0

ospfv3 200 ipv4 area 0

!

router ospfv3 200

router-id 4.4.4.4

!

address-family ipv4 unicast

exit-address-family

!

address-family ipv6 unicast

exit-address-family

!

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip ssh bulk-mode 131072

!

control-plane

service-policy input system-cpp-policy

!

!

banner motd ^C D1, Implement VRF-Lite ^C

!

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line vty 0 4

exec-timeout 0 0

privilege level 15

password cisco123

logging synchronous

login

transport input ssh

line vty 5 31

login

transport input ssh

!

end

# Switch D2

D2# show run

Building configuration...

Current configuration : 5872 bytes

!

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform punt-keepalive disable-kernel-core

!

hostname D2

!

vrf definition Mgmt-vrf

!

address-family ipv4

exit-address-family

!

address-family ipv6

exit-address-family

!

no aaa new-model

switch 1 provision c9300l-24p-4g

!

ip routing

!

no ip domain lookup

!

login on-success log

ipv6 nd cache expire refresh

ipv6 unicast-routing

!

diagnostic bootup level minimal

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

redundancy

mode sso

crypto engine compliance shield disable

!

transceiver type all

monitoring

!

class-map match-any system-cpp-police-ewlc-control

description EWLC Control

class-map match-any system-cpp-police-topology-control

description Topology control

class-map match-any system-cpp-police-sw-forward

description Sw forwarding, L2 LVX data packets, LOGGING, Transit Traffic

class-map match-any system-cpp-default

description EWLC Data, Inter FED Traffic

class-map match-any system-cpp-police-sys-data

description Openflow, Exception, EGR Exception, NFL Sampled Data, RPF Failed

class-map match-any system-cpp-police-punt-webauth

description Punt Webauth

class-map match-any system-cpp-police-l2lvx-control

description L2 LVX control packets

class-map match-any system-cpp-police-forus

description Forus Address resolution and Forus traffic

class-map match-any system-cpp-police-multicast-end-station

description MCAST END STATION

class-map match-any system-cpp-police-high-rate-app

description High Rate Applications

class-map match-any system-cpp-police-multicast

description MCAST Data

class-map match-any system-cpp-police-l2-control

description L2 control

class-map match-any system-cpp-police-dot1x-auth

description DOT1X Auth

class-map match-any system-cpp-police-data

description ICMP redirect, ICMP_GEN and BROADCAST

class-map match-any system-cpp-police-stackwise-virt-control

description Stackwise Virtual OOB

class-map match-any non-client-nrt-class

class-map match-any system-cpp-police-routing-control

description Routing control and Low Latency

class-map match-any system-cpp-police-protocol-snooping

description Protocol snooping

class-map match-any system-cpp-police-dhcp-snooping

description DHCP snooping

class-map match-any system-cpp-police-ios-routing

description L2 control, Topology control, Routing control, Low Latency

class-map match-any system-cpp-police-system-critical

description System Critical and Gold Pkt

class-map match-any system-cpp-police-ios-feature

description ICMPGEN,BROADCAST,ICMP,L2LVXCntrl,ProtoSnoop,PuntWebauth,MCASTData,Transit,DOT1XAuth,Swfwd,LOGGING,L2LVXData,ForusTraffic,ForusARP,McastEndStn,Openflow,Exception,EGRExcption,NflSampled,RpfFailed

!

policy-map system-cpp-policy

!

interface GigabitEthernet0/0

vrf forwarding Mgmt-vrf

no ip address

shutdown

negotiation auto

!

interface GigabitEthernet1/0/1

shutdown

!

interface GigabitEthernet1/0/2

shutdown

!

interface GigabitEthernet1/0/3

shutdown

!

interface GigabitEthernet1/0/4

shutdown

!

interface GigabitEthernet1/0/5

no switchport

ip address 10.1.3.2 255.255.255.0

ipv6 address FE80::D2:1 link-local

ipv6 address 2001:DB8:ACAD:1013::2/64

ospfv3 200 ipv6 area 0

ospfv3 200 ipv4 area 0

!

interface GigabitEthernet1/0/6

shutdown

!

interface GigabitEthernet1/0/7

shutdown

!

interface GigabitEthernet1/0/8

shutdown

!

interface GigabitEthernet1/0/9

shutdown

!

interface GigabitEthernet1/0/10

shutdown

!

interface GigabitEthernet1/0/11

shutdown

!

interface GigabitEthernet1/0/12

shutdown

!

interface GigabitEthernet1/0/13

shutdown

!

interface GigabitEthernet1/0/14

shutdown

!

interface GigabitEthernet1/0/15

shutdown

!

interface GigabitEthernet1/0/16

shutdown

!

interface GigabitEthernet1/0/17

shutdown

!

interface GigabitEthernet1/0/18

shutdown

!

interface GigabitEthernet1/0/19

shutdown

!

interface GigabitEthernet1/0/20

shutdown

!

interface GigabitEthernet1/0/21

shutdown

!

interface GigabitEthernet1/0/22

shutdown

!

interface GigabitEthernet1/0/23

switchport access vlan 11

switchport mode access

!

interface GigabitEthernet1/0/24

shutdown

!

interface GigabitEthernet1/1/1

shutdown

!

interface GigabitEthernet1/1/2

shutdown

!

interface GigabitEthernet1/1/3

shutdown

!

interface GigabitEthernet1/1/4

shutdown

!

interface AppGigabitEthernet1/0/1

!

interface Vlan1

no ip address

!

interface Vlan11

ip address 192.168.3.1 255.255.255.0

ipv6 address FE80::D2:2 link-local

ipv6 address 2001:DB8:ACAD:3000::1/64

ospfv3 200 ipv6 area 0

ospfv3 200 ipv4 area 0

!

router ospfv3 200

router-id 5.5.5.5

!

address-family ipv4 unicast

exit-address-family

!

address-family ipv6 unicast

exit-address-family

!

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip ssh bulk-mode 131072

!

control-plane

service-policy input system-cpp-policy

!

!

banner motd ^C D2, Implement VRF-Lite ^C

!

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line vty 0 4

exec-timeout 0 0

privilege level 15

password cisco123

logging synchronous

login

transport input ssh

line vty 5 31

login

transport input ssh

!

end

# Switch A1

A1# show run brief

Building configuration...

Current configuration : 4857 bytes

!

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform punt-keepalive disable-kernel-core

!

hostname A1

!

!

vrf definition Mgmt-vrf

!

address-family ipv4

exit-address-family

!

address-family ipv6

exit-address-family

!

no aaa new-model

!

switch 1 provision c9200l-24t-4g

!

no ip domain lookup

!

login on-success log

!

diagnostic bootup level minimal

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

redundancy

mode sso

crypto engine compliance shield disable

!

transceiver type all

monitoring

!

!

class-map match-any system-cpp-police-ewlc-control

description EWLC Control

class-map match-any system-cpp-police-topology-control

description Topology control

class-map match-any system-cpp-police-sw-forward

description Sw forwarding, L2 LVX data packets, LOGGING, Transit Traffic

class-map match-any system-cpp-default

description EWLC data, Inter FED Traffic

class-map match-any system-cpp-police-sys-data

description Openflow, Exception, EGR Exception, NFL Sampled Data, RPF Failed

class-map match-any system-cpp-police-punt-webauth

description Punt Webauth

class-map match-any system-cpp-police-l2lvx-control

description L2 LVX control packets

class-map match-any system-cpp-police-forus

description Forus Address resolution and Forus traffic

class-map match-any system-cpp-police-multicast-end-station

description MCAST END STATION

class-map match-any system-cpp-police-high-rate-app

description High Rate Applications

class-map match-any system-cpp-police-multicast

description MCAST Data

class-map match-any system-cpp-police-l2-control

description L2 control

class-map match-any system-cpp-police-dot1x-auth

description DOT1X Auth

class-map match-any system-cpp-police-data

description ICMP redirect, ICMP_GEN and BROADCAST

class-map match-any system-cpp-police-stackwise-virt-control

description Stackwise Virtual OOB

class-map match-any non-client-nrt-class

class-map match-any system-cpp-police-routing-control

description Routing control and Low Latency

class-map match-any system-cpp-police-protocol-snooping

description Protocol snooping

class-map match-any system-cpp-police-dhcp-snooping

description DHCP snooping

class-map match-any system-cpp-police-ios-routing

description L2 control, Topology control, Routing control, Low Latency

class-map match-any system-cpp-police-system-critical

description System Critical and Gold Pkt

class-map match-any system-cpp-police-ios-feature

description ICMPGEN,BROADCAST,ICMP,L2LVXCntrl,ProtoSnoop,PuntWebauth,MCASTData,Transit,DOT1XAuth,Swfwd,LOGGING,L2LVXData,ForusTraffic,ForusARP,McastEndStn,Openflow,Exception,EGRExcption,NflSampled,RpfFailed

!

policy-map system-cpp-policy

!

interface GigabitEthernet0/0

vrf forwarding Mgmt-vrf

no ip address

shutdown

!

interface GigabitEthernet1/0/1

switchport access vlan 5

switchport mode access

!

interface GigabitEthernet1/0/2

shutdown

!

interface GigabitEthernet1/0/3

switchport access vlan 8

switchport mode access

!

interface GigabitEthernet1/0/4

shutdown

!

interface GigabitEthernet1/0/5

shutdown

!

interface GigabitEthernet1/0/6

shutdown

!

interface GigabitEthernet1/0/7

shutdown

!

interface GigabitEthernet1/0/8

shutdown

!

interface GigabitEthernet1/0/9

shutdown

!

interface GigabitEthernet1/0/10

shutdown

!

interface GigabitEthernet1/0/11

switchport mode trunk

switchport nonegotiate

!

interface GigabitEthernet1/0/12

shutdown

!

interface GigabitEthernet1/0/13

shutdown

!

interface GigabitEthernet1/0/14

shutdown

!

interface GigabitEthernet1/0/15

shutdown

!

interface GigabitEthernet1/0/16

shutdown

!

interface GigabitEthernet1/0/17

shutdown

!

interface GigabitEthernet1/0/18

shutdown

!

interface GigabitEthernet1/0/19

shutdown

!

interface GigabitEthernet1/0/20

shutdown

!

interface GigabitEthernet1/0/21

shutdown

!

interface GigabitEthernet1/0/22

shutdown

!

interface GigabitEthernet1/0/23

shutdown

!

interface GigabitEthernet1/0/24

shutdown

!

interface GigabitEthernet1/1/1

shutdown

!

interface GigabitEthernet1/1/2

shutdown

!

interface GigabitEthernet1/1/3

shutdown

!

interface GigabitEthernet1/1/4

shutdown

!

interface Vlan1

no ip address

!

ip http server

ip http secure-server

ip forward-protocol nd

ip ssh bulk-mode 131072

!

control-plane

service-policy input system-cpp-policy

!

banner motd ^C A1, Implement VRF-Lite ^C

!

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line vty 0 4

exec-timeout 0 0

privilege level 15

password cisco123

logging synchronous

login

transport input ssh

line vty 5 15

login

transport input ssh

!

end