# Lab - Implement a DMVPN Phase 3 Spoke-to-Spoke Topology (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

# Topology

# Addressing Table

| Device | Interface | IPv4 Address |
| --- | --- | --- |
| R1 | G0/0/1 | 192.0.2.1/30 |
| R1 | Tunnel 1 | 100.100.100.1/29 |
| R2 | G0/0/1 | 198.51.100.2/30 |
| R2 | Loopback 0 | 192.168.2.1/24 |
| R2 | Loopback 1 | 172.16.2.1/24 |
| R2 | Tunnel 1 | 100.100.100.2/29 |
| R3 | G0/0/1 | 203.0.113.2/30 |
| R3 | Loopback 0 | 192.168.3.1/24 |
| R3 | Loopback 1 | 172.16.3.1/24 |
| R3 | Tunnel 1 | 100.100.100.3/29 |

# Objectives

In this lab, you will create a Dynamic Multipoint Virtual Private Network (DMVPN) that consists of a hub router with two spokes. You will implement a DMVPN Phase 3 spoke-to-spoke topology.

Part 1: Build the Network and Configure Basic Device Settings

Part 2: Configure DMVPN Phase 3

Part 3: Verify DMVPN Phase 3

# Background / Scenario

In this lab, you will modify a DMVPN Phase 1 network to create Phase 3 spoke-to-spoke network. The configuration from the previous Phase 1 lab are required as the starting point for this lab. They are provided under Initial Configurations if you no longer have them.

Phase 3 improved upon Phase 2 by using a different mechanism for routing between destinations in spoke-to-spoke tunnels. Because of this, Phase 3 can be seen as superseding Phase 2. For that reason, there is no Phase 2 lab.

In DMVPN Phase 3, the spoke routers create routing table entries in which NHRP manipulates the routing information for tunnel destinations by overriding the next hop that was determined by the routing protocol. NHRP provides the actual destination network as the next hop. This enables the distribution of route summaries from the hub router to the spokes. Finally, Phase 3 enables a hierarchical tree-based VPN architecture in which central hub routers connect other, regional hubs and their spokes. This allows for the establishment of tunnels between routers that are not connected to the same regional hub.

In Phase 1 DMVPN, all spoke router traffic must pass through the hub router, as shown in the topology diagram.

In a Phase 3 spoke-spoke DMVPN, traffic initially travels to the hub. The traffic then travels directly between the spoke routers over the spoke-to-spoke tunnel.

Note: This lab does not include the configuration of IPSec to secure the tunnels. This essential procedure will be covered in a later lab.

Note: This lab is an exercise in configuring and verifying various implementations of DMVPN topologies and does not reflect networking best practices.

Note: The routers used with CCNP hands-on labs are the Cisco 8200L with Cisco IOS XE Release 17.12.05c (universalk9 image). The Layer 3 switch is a Cisco Catalyst 9300L with Cisco IOS XE Release 17.12.05 (universalk9 image). Other routers, Layer 3 switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs.

Note: Make sure that the devices have been erased and have no startup configurations. If you are unsure, please contact your instructor.

Instructor Note: Refer to the Instructor Lab Manual for the procedures to initialize and reload devices.

# Required Resources

3 Routers (Cisco 8200L with Cisco IOS XE Release 17.12.05c universal image or comparable)

1 Layer 3 switch (Cisco 9300L with Cisco IOS Release 17.12.05 universal image or comparable)

1 PC (Choice of operating system with a terminal emulation program installed)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

# Initial Configurations

These are the final configurations from the “Lab - Implement a DMVPN Phase 1 Hub-to-Spoke Topology”. If you do not have preconfigured devices from that lab, paste in these initial configurations into the devices before proceeding to Part 1.

R1 hub router

hostname R1

no ip domain lookup

banner motd # R1, Implement DMVPN Hub #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

exit

interface g0/0/1

ip address 192.0.2.1 255.255.255.252

no shutdown

exit

interface tunnel 1

tunnel mode gre multipoint

tunnel source g0/0/1

tunnel key 999

ip address 100.100.100.1 255.255.255.248

ip nhrp network-id 1

ip nhrp authentication NHRPauth

ip nhrp map multicast dynamic

bandwidth 4000

ip mtu 1400

ip tcp adjust-mss 1360

exit

router eigrp DMVPN_TUNNEL_NET

address-family ipv4 unicast autonomous-system 68

eigrp router-id 1.1.1.1

network 100.100.100.0 255.255.255.248

af-interface tunnel 1

no split-horizon

router eigrp DMVPN_TRANS_NET

address-family ipv4 unicast autonomous-system 168

eigrp router-id 10.1.1.1

network 192.0.2.0 255.255.255.252

end

R2 spoke router 1

hostname R2

no ip domain lookup

banner motd # R2, Implement DMVPN Spoke 1 #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

exit

interface g0/0/1

ip address 198.51.100.2 255.255.255.252

no shutdown

exit

interface loopback 0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface loopback 1

ip address 172.16.2.1 255.255.255.0

no shutdown

exit

interface tunnel 1

tunnel mode gre ip

tunnel source loopback 0

tunnel destination 192.0.2.1

tunnel key 999

ip address 100.100.100.2 255.255.255.248

ip nhrp network-id 1

ip nhrp authentication NHRPauth

ip nhrp nhs 100.100.100.1

ip nhrp map multicast 192.0.2.1

ip nhrp map 100.100.100.1 192.0.2.1

ip mtu 1400

ip tcp adjust-mss 1360

router eigrp DMVPN_TUNNEL_NET

address-family ipv4 unicast autonomous-system 68

eigrp router-id 2.2.2.2

network 100.100.100.0 255.255.255.248

network 172.16.2.0 255.255.255.0

eigrp stub connected

router eigrp DMVPN_TRANS_NET

address-family ipv4 unicast autonomous-system 168

eigrp router-id 20.2.2.2

network 198.51.100.0 255.255.255.252

network 192.168.2.0 255.255.255.0

end

R3 spoke router 2

hostname R3

no ip domain lookup

banner motd # R3, Implement DMVPN Spoke 2 #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

exit

interface g0/0/1

ip address 203.0.113.2 255.255.255.252

no shutdown

exit

interface loopback 0

ip address 192.168.3.1 255.255.255.0

no shutdown

exit

interface loopback 1

ip address 172.16.3.1 255.255.255.0

no shutdown

exit

interface tunnel 1

tunnel mode gre ip

tunnel source loopback 0

tunnel destination 192.0.2.1

tunnel key 999

ip address 100.100.100.3 255.255.255.248

ip nhrp network-id 1

ip nhrp authentication NHRPauth

ip nhrp nhs 100.100.100.1

ip nhrp map multicast 192.0.2.1

ip nhrp map 100.100.100.1 192.0.2.1

ip mtu 1400

ip tcp adjust-mss 1360

router eigrp DMVPN_TUNNEL_NET

address-family ipv4 unicast autonomous-system 68

eigrp router-id 3.3.3.3

network 100.100.100.0 255.255.255.248

network 172.16.3.0 255.255.255.0

eigrp stub connected

router eigrp DMVPN_TRANS_NET

address-family ipv4 unicast autonomous-system 168

eigrp router-id 30.3.3.3

network 203.0.113.0 255.255.255.252

network 192.168.3.0 255.255.255.0

eigrp stub connected

end

DMVPN Layer 3 Switch

hostname DMVPN

no ip domain lookup

ip routing

banner motd # DMVPN, DMVPN cloud switch #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

interface g1/0/11

no switchport

ip address 192.0.2.2 255.255.255.252

no shutdown

exit

interface g1/0/12

no switchport

ip address 198.51.100.1 255.255.255.252

no shutdown

exit

interface g1/0/13

no switchport

ip address 203.0.113.1 255.255.255.252

no shutdown

exit

router eigrp DMVPN_TRANS_NET

address-family ipv4 unicast autonomous-system 168

eigrp router-id 40.4.4.4

network 192.0.2.0 255.255.255.252

network 198.51.100.0 255.255.255.252

network 203.0.113.0 255.255.255.252

end

# Instructions

## Build the Network and Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings if the network is not already configured. This lab uses the same topology and final configurations from the Lab - Implement a DMVPN Phase 1 Hub-to-Spoke Topology.

### Cable the network as shown in the topology.

Attach the devices as shown in the topology diagram, and cable as necessary.

### Configure initial settings for each router and the Layer 3 switch.

Console into each device, enter global configuration mode, and apply the initial settings for the lab if the devices are not already configured.

### Verify connectivity in the network.

From R1, ping the loopback interfaces of R2 and R3. All pings should be successful. This verifies that full connectivity exists in the underlay, or transport, network.

Open configuration window

R1# ping 192.168.2.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.3.1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/1 ms

R1# ping 192.168.3.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.3.1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/1 ms

Close configuration window

      - From R2 traceroute to the Loopback 1 interface of R3. You should see the following output. Note that traffic from the R2 spoke router travels through the R1 hub router to reach R3. This is the DMVPN Phase 1 traffic flow. Repeat the traceroute to verify this result.

Open configuration window

R2# traceroute 172.16.3.1

Type escape sequence to abort.

Tracing the route to 172.16.3.1

VRF info: (vrf in name/id, vrf out name/id)

1 100.100.100.1 1 msec 1 msec 1 msec

2 100.100.100.3 1 msec *  2 msec

On the spoke routers, issue the show ip route command. Note that EIGRP shows the next hop for the Loopback 1 networks to be the R1 tunnel interface address. That is, both spoke routers will use R1 to send traffic on the overlay network to the LANs.

R2# show ip route eigrp | begin Gateway

Gateway of last resort is not set

172.16.0.0/16 is variably subnetted, 3 subnets, 2 masks

D        172.16.3.0/24 [90/102400640] via 100.100.100.1, 00:21:14, Tunnel1

192.0.2.0/30 is subnetted, 1 subnets

D        192.0.2.0 [90/15360] via 198.51.100.1, 00:21:21, GigabitEthernet0/0/1

D     192.168.3.0/24

[90/16000] via 198.51.100.1, 00:21:21, GigabitEthernet0/0/1

203.0.113.0/30 is subnetted, 1 subnets

D        203.0.113.0

[90/15360] via 198.51.100.1, 00:21:21, GigabitEthernet0/0/1

Close configuration window

open configuration window

R3# show ip route eigrp | begin Gateway

Gateway of last resort is not set

172.16.0.0/16 is variably subnetted, 3 subnets, 2 masks

D        172.16.2.0/24 [90/102400640] via 100.100.100.1, 00:22:49, Tunnel1

192.0.2.0/30 is subnetted, 1 subnets

D        192.0.2.0 [90/15360] via 203.0.113.1, 00:22:59, GigabitEthernet0/0/1

D     192.168.2.0/24

[90/16000] via 203.0.113.1, 00:22:59, GigabitEthernet0/0/1

198.51.100.0/30 is subnetted, 1 subnets

D        198.51.100.0

[90/15360] via 203.0.113.1, 00:22:59, GigabitEthernet0/0/1

Close configuration window

## Configure DMVPN Phase 3

In this part of the lab, you will configure DMVPN Phase 3 to create DMVPN tunnels between the spoke routers. Initially, the services of the hub are required to begin data transfer over the VPN. However, the hub multipoint tunnel interface detects that traffic is “hairpinning,” or exiting the interface on which it was received. The hub router sends redirect messages to the two spoke routers that are communicating. The spoke routers use the redirect messages to dynamically create a tunnel to connect to each other. In this way, the hub router does not need to forward spoke-to-spoke traffic after the setup of the spoke-to-spoke tunnel.

### Modify the tunnel interfaces on the spoke routers.

This lab builds on the Phase 1 lab that was previously completed. On the spoke routers R2 and R3, modify the configurations of the Tunnel 1 interfaces. The network will essentially be configured for DMVPN Phase 2 at the conclusion of this step.

On R2, set the tunnel mode to mGRE. You will see an error message that states that point-to-multipoint tunnels can not have static tunnel destinations configured.

Open configuration window

R2(config)# interface tunnel 1

R2(config-if)# tunnel mode gre multipoint

Tunnel set mode failed. p2mp tunnels cannot have a tunnel destination.

Remove the tunnel destination configuration. You will see the interface go down and the EIGRP adjacency with the hub router be lost. Reconfigure the tunnel mode as GRE multipoint. You will see the interface come back up and the EIGRP adjacency will be restored.

R2(config-if)# no tunnel destination

*Mar 27 12:46:31.496: %LINEPROTO-5-UPDOWN: Line protocol on Interface Tunnel1, changed state to down

*Mar 27 12:46:31.499: %DUAL-5-NBRCHANGE: EIGRP-IPv4 68: Neighbor 100.100.100.1 (Tunnel1) is down: interface down

R2(config-if)# tunnel mode gre multipoint

*Mar 27 12:46:46.629: %LINEPROTO-5-UPDOWN: Line protocol on Interface Tunnel1, changed state to up

*Mar 27 12:46:48.008: %DUAL-5-NBRCHANGE: EIGRP-IPv4 68: Neighbor 100.100.100.1 (Tunnel1) is up: new adjacency

Close configuration window

Repeat this process for the R3 tunnel interface. At this point, you have essentially configured DMVPN Phase 2. You may want to verify the R1 can ping R2 and R3 loopbacks again.

R1# ping 192.168.2.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.2.1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/1 ms

R1# ping 192.168.3.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.3.1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/1 ms

Verify the tunnel interface configuration is now using mGRE with the show interface tunnel 1 command.

Open configuration window

R3# show interface tunnel 1 | include Tunnel protocol

Tunnel protocol/transport multi-GRE/IP

Close configuration window

### Modify the configuration on the spoke routers to enable NHRP routing shortcuts.

Configure the spoke router interfaces on R2 and R3 to enable NHRP to add spoke networks as next hops in the EIGRP routing table. The hub router will send NHRP redirect messages to the spokes. NHRP uses information in the redirect messages to manipulate the EIGRP routing table and create the next hop shortcuts.

Open configuration window

R2(config)# interface tunnel 1

R2(config-if)# ip nhrp shortcut

Close configuration window

Open configuration window

R3(config)# interface tunnel 1

R3(config-if)# ip nhrp shortcut

Close configuration window

### Modify the configuration of hub router to send NHRP redirect messages.

The Tunnel 1 interface of the hub router needs to be configured to send NHRP redirect messages to the spokes when it detects spoke-to-spoke traffic. Issue the ip nhrp redirect command on R1.

Open configuration window

R1(config)# interface tunnel 1

R1(config-if)# ip nhrp redirect

Close configuration window

## Verify DMVPN Phase 3

Now that DMVPN Phase 3 is complete, you can test dynamic spoke-to-spoke tunnel creation.

### Observe dynamic tunnel creation.

Return to R2. Initiate a traceroute to the simulated LAN interface on R3. The path will pass through R1 as it does in DMVPN Phase 1. But you will see console messages indicating that a dynamic tunnel was established and NHRP resolution occurred between the DMVPN peers (R2 and R3) during the trace.

Open configuration window

R2# traceroute 172.16.3.1

Type escape sequence to abort.

Tracing the route to 172.16.3.1

VRF info: (vrf in name/id, vrf out name/id)

1 100.100.100.1 1 msec 0 msec 0 msec

*Nov 10 14:27:00.032: %DMVPN-5-NHRP_NHP_UP: Tunnel1: Next Hop NHP : (Tunnel: 100.100.100.3 NBMA: 192.168.3.1) for (Tunnel: 100.100.100.2 NBMA: 192.168.2.1) is UP

*Nov 10 14:27:00.079: %DMVPN-7-NHRP_RES:  Tunnel1: Host with (Tunnel: 100.100.100.2 NBMA: 192.168.2.1) Received Resolution Req from  (Tunnel: 100.100.100.3 NBMA: 192.168.3.1)

2  * 100.100.100.3 1 msec *

Issue the traceroute command again. You will now see that R1 has enabled direct spoke-to-spoke communication between R2 and R3. This tunnel will expire after ten minutes by default. The tunnel dynamically reopens after data is sent to the spoke router again.

R2# traceroute 172.16.3.1

Type escape sequence to abort.

Tracing the route to 172.16.3.1

VRF info: (vrf in name/id, vrf out name/id)

1 100.100.100.3 1 msec *  1 msec

Close configuration window

Repeat the traceroute commands on R3 to 172.16.2.1. Do the trace twice because R3 does not automatically create its own NHRP mapping for R2 just because R2 initiated a connection. The NHRP cache and tunnel establishment are triggered by outgoing traffic.

### View the routing table.

On R2, use the command to view EIGRP routes in the routing table and compare it with the DMVPN Phase 1 routing from earlier in this lab. You should observe that the EIGRP route to the 172.16.3.0 network still shows the same next hop interface, but it is now flagged with a % symbol, indicating that NHRP has overridden the next hop entry with its own value. Additionally, a new NHRP route now appears in the routing table, showing that R3's tunnel interface is considered directly connected by NHRP, which enables more efficient spoke-to-spoke communication.

Note: You may need to reissue the traceroute commands from the previous step if the NHRP mappings expired on R2.

Open configuration window

R2# show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

100.0.0.0/8 is variably subnetted, 3 subnets, 2 masks

C        100.100.100.0/29 is directly connected, Tunnel1

L        100.100.100.2/32 is directly connected, Tunnel1

H        100.100.100.3/32 is directly connected, 00:03:53, Tunnel1

172.16.0.0/16 is variably subnetted, 3 subnets, 2 masks

C        172.16.2.0/24 is directly connected, Loopback1

L        172.16.2.1/32 is directly connected, Loopback1

D   %    172.16.3.0/24 [90/102400640] via 100.100.100.1, 02:53:00, Tunnel1

192.0.2.0/30 is subnetted, 1 subnets

D        192.0.2.0 [90/15360] via 198.51.100.1, 03:38:15, GigabitEthernet0/0/1

192.168.2.0/24 is variably subnetted, 2 subnets, 2 masks

C        192.168.2.0/24 is directly connected, Loopback0

L        192.168.2.1/32 is directly connected, Loopback0

D     192.168.3.0/24

[90/16000] via 198.51.100.1, 03:38:15, GigabitEthernet0/0/1

198.51.100.0/24 is variably subnetted, 2 subnets, 2 masks

C        198.51.100.0/30 is directly connected, GigabitEthernet0/0/1

L        198.51.100.2/32 is directly connected, GigabitEthernet0/0/1

203.0.113.0/30 is subnetted, 1 subnets

D        203.0.113.0

[90/15360] via 198.51.100.1, 03:38:15, GigabitEthernet0/0/1

View the value that is actually used for the next hop address. It is marked with NHO for next hop override. This indicates that the next hop to 172.16.3.0 is R3 through the spoke-to-spoke tunnel.

R2# show ip route next-hop-override | begin Gateway

Gateway of last resort is not set

100.0.0.0/8 is variably subnetted, 3 subnets, 2 masks

C        100.100.100.0/29 is directly connected, Tunnel1

L        100.100.100.2/32 is directly connected, Tunnel1

H        100.100.100.3/32 is directly connected, 00:17:39, Tunnel1

172.16.0.0/16 is variably subnetted, 3 subnets, 2 masks

C        172.16.2.0/24 is directly connected, Loopback1

L        172.16.2.1/32 is directly connected, Loopback1

D   %    172.16.3.0/24 [90/102400640] via 100.100.100.1, 00:17:41, Tunnel1

[NHO][90/255] via 100.100.100.3, 00:17:39, Tunnel1

192.0.2.0/30 is subnetted, 1 subnets

D        192.0.2.0 [90/15360] via 198.51.100.1, 01:02:56, GigabitEthernet0/0/1

192.168.2.0/24 is variably subnetted, 2 subnets, 2 masks

C        192.168.2.0/24 is directly connected, Loopback0

L        192.168.2.1/32 is directly connected, Loopback0

D     192.168.3.0/24

[90/16000] via 198.51.100.1, 01:02:56, GigabitEthernet0/0/1

198.51.100.0/24 is variably subnetted, 2 subnets, 2 masks

C        198.51.100.0/30 is directly connected, GigabitEthernet0/0/1

L        198.51.100.2/32 is directly connected, GigabitEthernet0/0/1

203.0.113.0/30 is subnetted, 1 subnets

D        203.0.113.0

[90/15360] via 198.51.100.1, 01:02:56, GigabitEthernet0/0/1

### Verify the DMVPN

View the DMVPN information with the spoke-to-spoke tunnel open. The first entry in the table is the static tunnel between R2 and the hub router. The DLX flag indicates that the entry is the local network. The DT1 entry is the route through the overlay network, and the DT2 entry the next hop override to the target network. If the spoke-to-spoke tunnel has closed, generate traffic to reopen it with traceroute.

R2# show dmvpn detail

Legend: Attrb --> S - Static, D - Dynamic, I - Incomplete

N - NATed, L - Local, X - No Socket

T1 - Route Installed, T2 - Nexthop-override

C - CTS Capable, I2 - Temporary

# Ent --> Number of NHRP entries with same NBMA peer

NHS Status: E --> Expecting Replies, R --> Responding, W --> Waiting

UpDn Time --> Up or Down Time for a Tunnel

==========================================================================

Interface Tunnel1 is up/up, Addr. is 100.100.100.2, VRF ""

Tunnel Src./Dest. addr: 192.168.2.1/Multipoint, Tunnel VRF ""

Protocol/Transport: "multi-GRE/IP", Protect ""

Interface State Control: Disabled

nhrp event-publisher : Disabled

IPv4 NHS:

100.100.100.1  RE priority = 0 cluster = 0

Type:Spoke, Total NBMA Peers (v4/v6): 3

# Ent  Peer NBMA Addr Peer Tunnel Add State  UpDn Tm Attrb    Target Network

----- --------------- --------------- ----- -------- ----- -----------------

1 192.0.2.1         100.100.100.1    UP 03:22:47     S   100.100.100.1/32

1 192.168.2.1       100.100.100.2    UP 00:01:36   DLX   100.100.100.2/32

2 192.168.3.1       100.100.100.3    UP 00:01:36   DT1   100.100.100.3/32

192.168.3.1       100.100.100.3    UP 00:01:36   DT2      172.16.3.0/24

Crypto Session Details:

--------------------------------------------------------------------------------

Pending DMVPN Sessions:

Close configuration window

Repeat this command on R3. Again, you may need to reissue the traceroute command to R2 to bring the tunnel backup.

R3# show dmvpn detail | begin Interface

Interface Tunnel1 is up/up, Addr. is 100.100.100.3, VRF "global"

Tunnel Src./Dest. addr: 192.168.3.1/Multipoint, Tunnel VRF "global"

Protocol/Transport: "multi-GRE/IP", Protect ""

Interface State Control: Disabled

nhrp event-publisher : Disabled

IPv4 NHS:

100.100.100.1  RE  priority = 0 cluster = 0

Type:Spoke, Total NBMA Peers (v4/v6): 3

# Ent  Peer NBMA Addr Peer Tunnel Add State  UpDn Tm Attrb    Target Network

----- --------------- --------------- ----- -------- ----- -----------------

1 192.0.2.1         100.100.100.1    UP 00:21:52     S   100.100.100.1/32

1 192.168.3.1       100.100.100.3    UP 00:06:10   DLX      172.16.3.0/24

2 192.168.2.1       100.100.100.2    UP 00:00:35   DT1   100.100.100.2/32

192.168.2.1       100.100.100.2    UP 00:00:35   DT2      172.16.2.0/24

### View NHRP Mappings

Note: If it has been more than 10 minutes since you completed your traceroutes, repeat them now to re-establish the spoke-to-spoke tunnels.

On R2 and R3, issue the show ip nhrp detail command to view the NHRP information for the routers. Note the correspondence between the entries in the output of this command with the output from the show dmvpn detail command. The first entry is the static entry to the hub router. The second entry is the local route, corresponding to the DTX entry in output above. The third entry corresponds to the DT1 entry and is flagged with router nhop rib. This indicates that the router has an explicit method to reach the tunnel IP address using an NBMA address and has an associated route installed in the routing table.

The flag rib nho indicates that the router has found an identical route in the routing table that belongs to a different protocol (EIGRP in this case.). NHRP has overridden the other protocol’s next-hop entry for the network by installing a next-hop shortcut in the routing table. This corresponds to the DT2 entry in the show dmvpn detail output.

Open configuration window

R2# show ip nhrp detail

100.100.100.1/32 via 100.100.100.1

Tunnel1 created 04:03:01, never expire

Type: static, Flags:

NBMA address: 192.0.2.1

Preference: 255

100.100.100.2/32 via 100.100.100.2

Tunnel1 created 00:00:21, expire 00:09:38

Type: dynamic, Flags: router unique local

NBMA address: 192.168.2.1

Preference: 255

(no-socket)

Requester: 100.100.100.3 Request ID: 9

100.100.100.3/32 via 100.100.100.3

Tunnel1 created 00:00:21, expire 00:09:38

Type: dynamic, Flags: router nhop rib

NBMA address: 192.168.3.1

Preference: 255

172.16.3.0/24 via 100.100.100.3

Tunnel1 created 00:00:21, expire 00:09:38

Type: dynamic, Flags: router rib nho

NBMA address: 192.168.3.1

Preference: 255

Close configuration window

You have successfully configured a DMVPN Phase 3 network.

End of document

# Device Configs - Final

# Router R1

enable

configure terminal

hostname R1

no ip domain lookup

banner motd # R1, Implement DMVPN Hub #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

exit

interface g0/0/1

ip address 192.0.2.1 255.255.255.252

no shutdown

exit

interface tunnel 1

tunnel mode gre multipoint

tunnel source g0/0/1

tunnel key 999

ip address 100.100.100.1 255.255.255.248

ip nhrp network-id 1

ip nhrp authentication NHRPauth

ip nhrp map multicast dynamic

ip nhrp redirect

bandwidth 4000

ip mtu 1400

ip tcp adjust-mss 1360

exit

router eigrp DMVPN_TUNNEL_NET

address-family ipv4 unicast autonomous-system 68

eigrp router-id 1.1.1.1

network 100.100.100.0 255.255.255.248

af-interface tunnel 1

no split-horizon

router eigrp DMVPN_TRANS_NET

address-family ipv4 unicast autonomous-system 168

eigrp router-id 10.1.1.1

network 192.0.2.0 255.255.255.252

end

# Router R2

enable

configure terminal

hostname R2

no ip domain lookup

banner motd # R2, Implement DMVPN Spoke 1 #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

exit

interface g0/0/1

ip address 198.51.100.2 255.255.255.252

no shutdown

exit

interface loopback 0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface loopback 1

ip address 172.16.2.1 255.255.255.0

no shutdown

exit

interface tunnel 1

tunnel mode gre multipoint

tunnel source loopback 0

no tunnel destination

tunnel key 999

ip address 100.100.100.2 255.255.255.248

ip nhrp network-id 1

ip nhrp authentication NHRPauth

ip nhrp nhs 100.100.100.1

ip nhrp map multicast 192.0.2.1

ip nhrp map 100.100.100.1 192.0.2.1

ip nhrp shortcut

ip mtu 1400

ip tcp adjust-mss 1360

router eigrp DMVPN_TUNNEL_NET

address-family ipv4 unicast autonomous-system 68

eigrp router-id 2.2.2.2

network 100.100.100.0 255.255.255.248

network 172.16.2.0 255.255.255.0

eigrp stub connected

router eigrp DMVPN_TRANS_NET

address-family ipv4 unicast autonomous-system 168

eigrp router-id 20.2.2.2

network 198.51.100.0 255.255.255.252

network 192.168.2.0 255.255.255.0

end

# Router R3

enable

configure terminal

hostname R3

no ip domain lookup

banner motd # R3, Implement DMVPN Spoke 2 #

line con 0

exec-timeout 0 0

logging synchronous

exit

line vty 0 4

privilege level 15

password cisco123

exec-timeout 0 0

logging synchronous

login

exit

interface g0/0/1

ip address 203.0.113.2 255.255.255.252

no shutdown

exit

interface loopback 0

ip address 192.168.3.1 255.255.255.0

no shutdown

exit

interface loopback 1

ip address 172.16.3.1 255.255.255.0

no shutdown

exit

interface tunnel 1

tunnel mode gre multipoint

tunnel source loopback 0

no tunnel destination

tunnel key 999

ip address 100.100.100.3 255.255.255.248

ip nhrp network-id 1

ip nhrp authentication NHRPauth

ip nhrp nhs 100.100.100.1

ip nhrp map multicast 192.0.2.1

ip nhrp map 100.100.100.1 192.0.2.1

ip nhrp shortcut

ip mtu 1400

ip tcp adjust-mss 1360

router eigrp DMVPN_TUNNEL_NET

address-family ipv4 unicast autonomous-system 68

eigrp router-id 3.3.3.3

network 100.100.100.0 255.255.255.248

network 172.16.3.0 255.255.255.0

eigrp stub connected

router eigrp DMVPN_TRANS_NET

address-family ipv4 unicast autonomous-system 168

eigrp router-id 30.3.3.3

network 203.0.113.0 255.255.255.252

network 192.168.3.0 255.255.255.0

eigrp stub connected

end