# Lab - Manipulate Route Redistribution (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

# Topology

# Addressing Table

| Device | Interface | IPv4 Address/Mask | IPv6 Address/Prefix | IPv6 Link Local |
| --- | --- | --- | --- | --- |
| R1 | G0/0/0 | 10.1.12.1/24 | 2001:db8:acad:12::1/64 | fe80::12:1 |
| R1 | G0/0/1 | 10.1.11.1/24 | 2001:db8:acad:11::1/64 | fe80::11:1 |
| R1 | Loopback | 10.1.1.1/24 | 2001:db8:acad:1::1/64 | fe80::1:1 |
| R2 | G0/0/0 | 10.1.12.2/24 | 2001:db8:acad:12::2/64 | fe80::12:2 |
| R2 | G0/0/1 | 10.1.23.2/24 | 2001:db8:acad:23::2/64 | fe80::23:2 |
| R3 | G0/0/0 | 10.1.23.3/24 | 2001:db8:acad:23::3/64 | fe80::23:3 |
| R3 | G0/0/1 | 10.1.32.1/24 | 2001:db8:acad:32::3/64 | fe80::32:3 |
| R3 | Loopback 0 | 10.3.3.3/24 | 2001:db8:acad:3::3/64 | fe80::3:3 |
| D1 | G1/0/11 | 10.1.11.2/24 | 2001:db8:acad:11::2/64 | fe80::11:2 |
| D1 | Loopback 0 | 209.165.201.1/25 | 2001:db8:209:165:201::1/80 | fe80::209:1 |
|  | Additional Loopbacks | Refer to the Topology |  |  |
| D2 | G1/0/11 | 10.1.32.2/24 | 2001:db8:acad:32::2/64 | fe80::32:2 |
| D2 | Loopback 0 | 198.51.100.1/25 | 2001:db8:198:51:100::1/80 | fe80::198:1 |
|  | Additional Loopbacks | Refer to the Topology |  |  |

# Objectives

Part 1: Build the Network and Configure Basic Device Settings

Part 2: Verify OSPFv3 AF Neighborships and Routing for IPv4 and IPv6

Part 3: Verify EIGRP Neighborships and Routing for IPv4 and IPv6

Part 4: Configure Redistribution from OSPFv3 to EIGRP

Part 5: Configure Redistribution from EIGRP to OSPFv3

Part 6: Configure Policies to Manipulate Route Redistribution

# Background / Scenario

In the first part of this lab, you will review the configuration of redistribution from OSPF into EIGRP for IPv4 and IPv6, and redistribution of EIGRP into OSPF for IPv4 and IPv6. Much of this lab reviews what you did in the lab Configure Route Redistribution Between EIGRP and OSPF. Refer to that lab if you need a reference for how to configure redistribution in Part 4 and Part 5.

In Part 6, you will manipulate route redistribution using a variety of methods including ACLs, prefix lists, and route maps.

D1, R1 and R2 are configured with OSPFv3 for IPv4 and IPv6 address families, while R2, R3 and D2 are configured with EIGRP using named mode for IPv4 and IPv6 address families.

Note: D1 is configured to originate and advertise a default route into the OSPFv3 domain for both IPv4 and IPv6 address families.

Note: This lab is an exercise in configuring and verifying two-way route redistribution on R2. Route redistribution in this lab does not reflect networking best practices.

Note: The routers used with CCNP hands-on labs are Cisco 8200L with Cisco IOS XE Release 17.12.05 (universalk9 image). The switches used in the labs are Cisco Catalyst 9300L with Cisco IOS XE Release 17.12.05 (universalk9 image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs.

Note: Make sure that all the devices have been erased and have no startup configurations. If you are unsure, contact your instructor.

Instructor Note: Refer to the Instructor Lab Manual for the procedures to initialize and reload devices.

# Required Resources

3 Routers (Cisco 8200L with Cisco IOS XE Release 17.12.05c universal image or comparable)

2 Switches (Cisco 9300L with Cisco IOS XE release 17.12.05 universal image or comparable)

1 PC (Choice of operating system with terminal emulation program installed)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

# Instructions

## Build the Network and Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings.

### Cable the network as shown in the topology.

Attach the devices as shown in the topology diagram and cable as necessary.

### Configure basic settings for each device.

Console into each device, enter global configuration mode, and apply the basic settings for the lab. Initial configurations for each device are listed below.

Open configuration window

Router R1

hostname R1

no ip domain lookup

ipv6 unicast-routing

banner motd # R1, Manipulate Redistribution #

line con 0

exec-timeout 0 0

logging synchronous

exit

router ospfv3 1

router-id 1.1.1.1

exit

interface g0/0/0

ip address 10.1.12.1 255.255.255.0

ipv6 address FE80::12:1 link-local

ipv6 address 2001:DB8:ACAD:12::1/64

ospfv3 1 ipv6 area 0

ospfv3 1 ipv4 area 0

no shutdown

exit

interface g0/0/1

ip address 10.1.11.1 255.255.255.0

ipv6 address fe80::11:1 link-local

ipv6 address 2001:db8:acad:11::1/64

ospfv3 1 ipv6 area 11

ospfv3 1 ipv4 area 11

no shutdown

exit

interface loopback 0

ip address 10.1.1.1 255.255.255.0

ipv6 address FE80::1:1 link-local

ipv6 address 2001:DB8:ACAD:1::1/64

ospfv3 network point-to-point

ospfv3 1 ipv4 area 0

ospfv3 1 ipv6 area 0

no shutdown

exit

router ospfv3 1

address-family ipv4 unicast

passive-interface Loopback0

exit-address-family

address-family ipv6 unicast

passive-interface Loopback0

exit-address-family

end

Router R2

hostname R2

no ip domain lookup

ipv6 unicast-routing

banner motd # R2, Manipulate Redistribution #

line con 0

exec-timeout 0 0

logging synchronous

exit

router ospfv3 1

router-id 2.2.2.2

address-family ipv4 unicast

exit-address-family

address-family ipv6 unicast

exit-address-family

interface g0/0/0

ip address 10.1.12.2 255.255.255.0

ipv6 address FE80::12:2 link-local

ipv6 address 2001:DB8:ACAD:12::2/64

ospfv3 1 ipv6 area 0

ospfv3 1 ipv4 area 0

no shutdown

exit

interface g0/0/1

ip address 10.1.23.2 255.255.255.0

ipv6 address fe80::23:2 link-local

ipv6 address 2001:db8:acad:23::2/64

no shutdown

exit

router eigrp CISCO

address-family ipv4 unicast autonomous-system 64512

af-interface default

shutdown

exit-af-interface

af-interface GigabitEthernet0/0/1

no shutdown

exit-af-interface

topology base

exit-af-topology

network 10.1.23.0 0.0.0.255

eigrp router-id 2.2.2.2

exit-address-family

address-family ipv6 unicast autonomous-system 64512

af-interface default

shutdown

exit-af-interface

af-interface GigabitEthernet0/0/1

no shutdown

exit-af-interface

topology base

exit-af-topology

exit-address-family

end

Router R3

hostname R3

no ip domain lookup

ipv6 unicast-routing

banner motd # R3, Manipulate Redistribution #

line con 0

exec-timeout 0 0

logging synchronous

exit

interface g0/0/0

ip address 10.1.23.3 255.255.255.0

ipv6 address fe80::23:3 link-local

ipv6 address 2001:db8:acad:23::3/64

no shutdown

exit

interface g0/0/1

ip address 10.1.32.3 255.255.255.0

ipv6 address fe80::32:3 link-local

ipv6 address 2001:db8:acad:32::3/64

no shutdown

exit

interface loopback 0

ip address 10.3.3.3 255.255.255.0

ipv6 address fe80::3:3 link-local

ipv6 address 2001:db8:acad:3::3/64

no shutdown

exit

router eigrp CISCO

address-family ipv4 unicast autonomous-system 64512

af-interface default

shutdown

exit-af-interface

af-interface GigabitEthernet0/0/0

no shutdown

exit-af-interface

af-interface GigabitEthernet0/0/1

no shutdown

exit-af-interface

af-interface Loopback0

no shutdown

exit-af-interface

topology base

exit-af-topology

network 10.1.23.0 0.0.0.255

network 10.1.32.0 0.0.0.255

network 10.3.3.0 0.0.0.255

eigrp router-id 3.3.3.3

exit-address-family

address-family ipv6 unicast autonomous-system 64512

af-interface default

shutdown

exit-af-interface

af-interface GigabitEthernet0/0/0

no shutdown

exit-af-interface

af-interface GigabitEthernet0/0/1

no shutdown

exit-af-interface

af-interface Loopback0

no shutdown

exit-af-interface

topology base

exit-af-topology

eigrp router-id 3.3.3.3

exit-address-family

end

Switch D1

hostname D1

no ip domain lookup

ip routing

ipv6 unicast-routing

banner motd # D1, Manipulate Redistribution #

line con 0

exec-timeout 0 0

logging synchronous

exit

router ospfv3 1

router-id 11.11.11.11

exit

interface range g1/0/1-24

shutdown

exit

interface g1/0/11

no switchport

ip address 10.1.11.2 255.255.255.0

ipv6 address fe80::11:2 link-local

ipv6 address 2001:db8:acad:11::2/64

ospfv3 1 ipv6 area 11

ospfv3 1 ipv4 area 11

no shutdown

exit

interface loopback 0

ip address 209.165.201.1 255.255.255.128

ipv6 address fe80::209:1 link-local

ipv6 address 2001:db8:209:165:201::1/80

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

ospfv3 1 ipv6 area 11

no shutdown

exit

interface Loopback1

ip address 10.10.41.1 255.255.255.0

ipv6 enable

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

no shutdown

exit

interface Loopback2

ip address 10.10.42.1 255.255.255.0

ipv6 enable

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

no shutdown

exit

interface Loopback3

ip address 10.10.43.1 255.255.255.0

ipv6 enable

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

no shutdown

exit

interface Loopback4

ip address 10.10.44.1 255.255.255.0

ipv6 enable

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

no shutdown

exit

interface loopback 41

ip address 172.16.41.1 255.255.255.0

ipv6 address fe80:41::1 link-local

ipv6 address 2001:db8:0:41::1/64

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

ospfv3 1 ipv6 area 11

no shutdown

exit

interface loopback 42

ip address 172.16.42.1 255.255.255.0

ipv6 address fe80:42::1 link-local

ipv6 address 2001:db8:0:42::1/64

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

ospfv3 1 ipv6 area 11

no shutdown

exit

interface loopback 43

ip address 172.16.43.1 255.255.255.0

ipv6 address fe80:43::1 link-local

ipv6 address 2001:db8:0:43::1/64

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

ospfv3 1 ipv6 area 11

no shutdown

exit

interface loopback 44

ip address 172.16.44.1 255.255.255.0

ipv6 address fe80:44::1 link-local

ipv6 address 2001:db8:0:44::1/64

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

ospfv3 1 ipv6 area 11

no shutdown

exit

router ospfv3 1

address-family ipv4 unicast

passive-interface loopback0

default-information originate

exit-address-family

address-family ipv6 unicast

passive-interface loopback0

default-information originate

exit-address-family

ip route 0.0.0.0 0.0.0.0 Loopback0

ipv6 route ::/0 Loopback0

end

Switch D2

hostname D2

no ip domain lookup

ip routing

ipv6 unicast-routing

banner motd # D2, Manipulate Redistribution #

line con 0

exec-timeout 0 0

logging synchronous

exit

interface range g1/0/1-24

shutdown

exit

interface g1/0/11

no switchport

ip address 10.1.32.2 255.255.255.0

ipv6 address fe80::32:2 link-local

ipv6 address 2001:db8:acad:32::2/64

no shutdown

exit

interface loopback 0

ip address 198.51.100.1 255.255.255.128

ipv6 address fe80::198:2 link-local

ipv6 address 2001:db8:198:51:100::1/80

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

no shutdown

interface loopback 1

ip address 10.10.21.1 255.255.255.0

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

no shutdown

interface loopback 2

ip address 10.10.22.1 255.255.255.0

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

no shutdown

interface loopback 3

ip address 10.10.23.1 255.255.255.0

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

no shutdown

interface loopback 4

ip address 10.10.24.1 255.255.255.0

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

no shutdown

interface loopback 21

ip address 172.16.21.1 255.255.255.0

ipv6 address fe80:21::1 link-local

ipv6 address 2001:db8:0:21::1/64

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

ospfv3 1 ipv6 area 11

no shutdown

interface loopback 22

ip address 172.16.22.1 255.255.255.0

ipv6 address fe80:22::1 link-local

ipv6 address 2001:db8:0:22::1/64

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

ospfv3 1 ipv6 area 11

no shutdown

interface loopback 23

ip address 172.16.23.1 255.255.255.0

ipv6 address fe80:23::1 link-local

ipv6 address 2001:db8:0:23::1/64

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

ospfv3 1 ipv6 area 11

no shutdown

interface loopback 24

ip address 172.16.24.1 255.255.255.0

ipv6 address fe80:24::1 link-local

ipv6 address 2001:db8:0:24::1/64

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

ospfv3 1 ipv6 area 11

no shutdown

router eigrp CISCO

address-family ipv4 unicast autonomous-system 64512

af-interface default

shutdown

exit-af-interface

af-interface Loopback0

no shutdown

passive-interface

exit-af-interface

af-interface Loopback1

no shutdown

exit-af-interface

af-interface Loopback2

no shutdown

exit-af-interface

af-interface Loopback3

no shutdown

exit-af-interface

af-interface Loopback4

no shutdown

exit-af-interface

af-interface Loopback21

no shutdown

exit-af-interface

af-interface Loopback22

no shutdown

exit-af-interface

af-interface Loopback23

no shutdown

exit-af-interface

af-interface Loopback24

no shutdown

exit-af-interface

af-interface GigabitEthernet1/0/11

no shutdown

exit-af-interface

topology base

exit-af-topology

network 10.1.32.0 0.0.0.255

network 198.51.100.0 0.0.0.127

network 10.10.21.0 255.255.255.0

network 10.10.22.0 255.255.255.0

network 10.10.23.0 255.255.255.0

network 10.10.24.0 255.255.255.0

network 172.16.21.0 255.255.255.0

network 172.16.22.0 255.255.255.192

network 172.16.22.64 255.255.255.192

network 172.16.22.128 255.255.255.192

eigrp router-id 22.22.22.22

exit-address-family

address-family ipv6 unicast autonomous-system 64512

af-interface default

shutdown

exit-af-interface

af-interface Loopback0

no shutdown

passive-interface

exit-af-interface

af-interface Loopback1

no shutdown

exit-af-interface

af-interface Loopback2

no shutdown

exit-af-interface

af-interface Loopback3

no shutdown

exit-af-interface

af-interface Loopback4

no shutdown

exit-af-interface

af-interface Loopback21

no shutdown

exit-af-interface

af-interface Loopback22

no shutdown

exit-af-interface

af-interface Loopback23

no shutdown

exit-af-interface

af-interface Loopback24

no shutdown

exit-af-interface

af-interface GigabitEthernet1/0/11

no shutdown

exit-af-interface

topology base

exit-af-topology

eigrp router-id 22.22.22.22

exit-address-family

end

Set the clock on all devices to UTC time.

Save the running configuration to startup-config on all devices.

Close configuration window

## Verify OSPFv3 AF Neighborships and Routing for IPv4 and IPv6

In this part, you will verify that OSPF has established neighbor relationships and routing for IPv4 and IPv6. The verification commands are identical to what you did in the previous lab. Refer to that lab if you need a refresher.

### Verify OSPFv3 AF neighborships on R1.

Verify R1 has OSPFv3 neighbors: two neighbors from IPv4 address family and two from IPv6 address family. Neighbor State should be FULL/DR or FULL/BDR.

Open configuration window

R1# show ospfv3 neighbor

OSPFv3 1 address-family ipv4 (router-id 1.1.1.1)

Neighbor ID     Pri   State           Dead Time   Interface ID    Interface

2.2.2.2           1   FULL/BDR        00:00:36    6               GigabitEthernet0/0/0

11.11.11.11       1   FULL/BDR        00:00:31    38              GigabitEthernet0/0/1

OSPFv3 1 address-family ipv6 (router-id 1.1.1.1)

Neighbor ID     Pri   State           Dead Time   Interface ID    Interface

2.2.2.2           1   FULL/BDR        00:00:39    6               GigabitEthernet0/0/0

11.11.11.11       1   FULL/BDR        00:00:39    38              GigabitEthernet0/0/1

Close configuration window

### Verify the IPv4 OSPFv3 routing table on R2.

Verify the OSPFv3 IPv4 routing table on R2. You should see the default route from D1 as well as all the intra-area and inter-area OSPF routes are received and installed from 10.1.12.1, which is R1.

Open configuration window

R2# show ip route ospfv3 | begin Gateway

Gateway of last resort is 10.1.12.1 to network 0.0.0.0

O*E2  0.0.0.0/0 [110/1] via 10.1.12.1, 00:02:44, GigabitEthernet0/0/0

10.0.0.0/8 is variably subnetted, 10 subnets, 2 masks

O        10.1.1.0/24 [110/2] via 10.1.12.1, 01:25:40, GigabitEthernet0/0/0

O IA     10.1.11.0/24 [110/2] via 10.1.12.1, 01:25:40, GigabitEthernet0/0/0

O IA     10.10.41.0/24 [110/3] via 10.1.12.1, 01:25:40, GigabitEthernet0/0/0

O IA     10.10.42.0/24 [110/3] via 10.1.12.1, 01:25:40, GigabitEthernet0/0/0

O IA     10.10.43.0/24 [110/3] via 10.1.12.1, 01:25:40, GigabitEthernet0/0/0

O IA     10.10.44.0/24 [110/3] via 10.1.12.1, 01:25:40, GigabitEthernet0/0/0

172.16.0.0/24 is subnetted, 4 subnets

O IA     172.16.41.0 [110/3] via 10.1.12.1, 01:25:40, GigabitEthernet0/0/0

O IA     172.16.42.0 [110/3] via 10.1.12.1, 01:25:40, GigabitEthernet0/0/0

O IA     172.16.43.0 [110/3] via 10.1.12.1, 01:25:40, GigabitEthernet0/0/0

O IA     172.16.44.0 [110/3] via 10.1.12.1, 01:25:40, GigabitEthernet0/0/0

209.165.201.0/25 is subnetted, 1 subnets

O IA     209.165.201.0 [110/3] via 10.1.12.1, 01:25:40, GigabitEthernet0/0/0

From R2, ping the Loopback 0 address on D1. The ping should be successful.

R2# ping 209.165.201.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 209.165.201.1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/2 ms

### Verify IPv6 OSPFv3 routing table on R2.

Verify the OSPFv3 IPv6 routing table on R2. You should see the default route from D1 as well as all the intra-area and inter-area OSPF routes are received and installed from fe80::12:1, which is R1.

R2# show ipv6 route ospf | begin ::/0

OE2 ::/0 [110/1], tag 1

via FE80::12:1, GigabitEthernet0/0/0

OI  2001:DB8:0:41::/64 [110/3]

via FE80::12:1, GigabitEthernet0/0/0

OI  2001:DB8:0:42::/64 [110/3]

via FE80::12:1, GigabitEthernet0/0/0

OI  2001:DB8:0:43::/64 [110/3]

via FE80::12:1, GigabitEthernet0/0/0

OI  2001:DB8:0:44::/64 [110/3]

via FE80::12:1, GigabitEthernet0/0/0

OI  2001:DB8:209:165:201::/80 [110/3]

via FE80::12:1, GigabitEthernet0/0/0

O   2001:DB8:ACAD:1::/64 [110/2]

via FE80::12:1, GigabitEthernet0/0/0

OI  2001:DB8:ACAD:11::/64 [110/2]

via FE80::12:1, GigabitEthernet0/0/0

From R2, ping the IPv6 Loopback 0 address on D1. The ping should be successful.

R2# ping 2001:db8:209:165:201::1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 2001:DB8:209:165:201::1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/3 ms

Close configuration window

## Verify EIGRP Neighborships and Routing for IPv4 and IPv6

In this part, you will verify that EIGRP has established neighbor relationships and routing for IPv4 and IPv6. The verification commands are identical to what you did in the previous lab. Refer to that lab if you need a refresher.

### Verify EIGRP for IPv4 neighborships on R3.

Issue the command to verify EIGRP has two IPv4 neighbors.

Open configuration window

R3# show ip eigrp neighbors

EIGRP-IPv4 VR(CISCO) Address-Family Neighbors for AS(64512)

H   Address                 Interface              Hold Uptime   SRTT   RTO  QSeq

(sec)         (ms)       CntNum

1   10.1.32.2               Gi0/0/1                  10 20:13:56    3   100  013

0   10.1.23.2               Gi0/0/0                  13 20:31:08    1   100  019

### Verify the EIGRP for IPv6 neighborships on R3.

Issue the command to verify EIGRP has two IPv6 neighbors .

R3# show ipv6 eigrp neighbors

EIGRP-IPv6 VR(CISCO) Address-Family Neighbors for AS(64512)

H   Address                 Interface              Hold Uptime   SRTT   RTO  Q  Seq

(sec)         (ms)       Cnt Num

1   Link-local address:     Gi0/0/1                  13 00:10:34    2   100  0  3

FE80::32:2

0   Link-local address:     Gi0/0/0                  13 00:13:47    1   100  0  8

FE80::23:2

Close configuration window

### Verify EIGRP for IPv4 routing table on R2.

Issue the command to display the EIGRP IPv4 routing table on R2, as shown.

Open configuration window

R2# show ip route eigrp | begin 10.0

10.0.0.0/8 is variably subnetted, 12 subnets, 2 masks

D        10.1.32.0/24 [90/15360] via 10.1.23.3, 01:37:55, GigabitEthernet0/0/1

D        10.3.3.0/24 [90/10880] via 10.1.23.3, 01:37:55, GigabitEthernet0/0/1

D        10.10.21.0/24

[90/16000] via 10.1.23.3, 01:37:55, GigabitEthernet0/0/1

D        10.10.22.0/24

[90/16000] via 10.1.23.3, 01:37:55, GigabitEthernet0/0/1

D        10.10.23.0/24

[90/16000] via 10.1.23.3, 01:37:55, GigabitEthernet0/0/1

D        10.10.24.0/24

[90/16000] via 10.1.23.3, 01:37:55, GigabitEthernet0/0/1

172.16.0.0/24 is subnetted, 2 subnets

D        172.16.21.0 [90/16000] via 10.1.23.3, 01:37:55, GigabitEthernet0/0/1

D        172.16.22.0 [90/16000] via 10.1.23.3, 01:37:55, GigabitEthernet0/0/1

198.51.100.0/25 is subnetted, 1 subnets

D        198.51.100.0 [90/16000] via 10.1.23.3, 01:37:55, GigabitEthernet0/0/1

### Verify EIGRP for IPv6 routing table on R2.

Issue the command to display the IPv6 EIGRP routing table on R2, as shown.

R2# show ipv6 route eigrp | begin 2001

D   2001:DB8:0:21::/64 [90/16000]

via FE80::23:3, GigabitEthernet0/0/1

D   2001:DB8:0:22::/64 [90/16000]

via FE80::23:3, GigabitEthernet0/0/1

D   2001:DB8:0:23::/64 [90/16000]

via FE80::23:3, GigabitEthernet0/0/1

D   2001:DB8:0:24::/64 [90/16000]

via FE80::23:3, GigabitEthernet0/0/1

D   2001:DB8:198:51:100::/80 [90/16000]

via FE80::23:3, GigabitEthernet0/0/1

D   2001:DB8:ACAD:3::/64 [90/10880]

via FE80::23:3, GigabitEthernet0/0/1

D   2001:DB8:ACAD:32::/64 [90/15360]

via FE80::23:3, GigabitEthernet0/0/1

## Configure Redistribution from OSPFv3 to EIGRP

In this part, you will redistribute OSPFv3 routes into EIGRP for IPv4. The configuration is identical to what you did in the previous lab. Refer to that lab if you need a refresher.

### Redistribute OSPFv3 into EIGRP for IPv4.

The name of the EIGRP process is CISCO. The AS is 64512. You are redistributing OSPFv3 process 1 routes. Use metric values of 1000000, 10, 255, 1, and 1500.

R2(config)# router eigrp CISCO

R2(config-router)# address-family ipv4 autonomous-system 64512

R2(config-router-af)# topology base

R2(config-router-af-topology)# redistribute ospfv3 1 metric 1000000 10 255 1 1500

R2(config-router-af-topology)# end

close configuration window

### On D2, verify redistribution of OSPFv3.

Issue the show ip route eigrp on D2. You should see that the gateway of last resort is now set and D2 has learned all the external EIGRP for IPv4 routes which originated from OSPFv3.

Open configuration window

D2# show ip route eigrp | begin Gateway

Gateway of last resort is 10.1.32.3 to network 0.0.0.0

D*EX  0.0.0.0/0 [170/66560] via 10.1.32.3, 00:00:20, GigabitEthernet1/0/11

10.0.0.0/8 is variably subnetted, 19 subnets, 2 masks

D EX     10.1.1.0/24

[170/66560] via 10.1.32.3, 00:00:20, GigabitEthernet1/0/11

D EX     10.1.11.0/24

[170/66560] via 10.1.32.3, 00:00:20, GigabitEthernet1/0/11

D EX     10.1.12.0/24

[170/66560] via 10.1.32.3, 00:00:20, GigabitEthernet1/0/11

D        10.1.23.0/24

[90/15360] via 10.1.32.3, 00:17:29, GigabitEthernet1/0/11

D        10.3.3.0/24 [90/10880] via 10.1.32.3, 00:17:29, GigabitEthernet1/0/11

D EX     10.10.41.0/24

[170/66560] via 10.1.32.3, 00:00:20, GigabitEthernet1/0/11

D EX     10.10.42.0/24

[170/66560] via 10.1.32.3, 00:00:20, GigabitEthernet1/0/11

D EX     10.10.43.0/24

[170/66560] via 10.1.32.3, 00:00:20, GigabitEthernet1/0/11

D EX     10.10.44.0/24

[170/66560] via 10.1.32.3, 00:00:20, GigabitEthernet1/0/11

172.16.0.0/16 is variably subnetted, 12 subnets, 2 masks

D EX     172.16.41.0/24

[170/66560] via 10.1.32.3, 00:00:20, GigabitEthernet1/0/11

D EX     172.16.42.0/24

[170/66560] via 10.1.32.3, 00:00:20, GigabitEthernet1/0/11

D EX     172.16.43.0/24

[170/66560] via 10.1.32.3, 00:00:20, GigabitEthernet1/0/11

D EX     172.16.44.0/24

[170/66560] via 10.1.32.3, 00:00:20, GigabitEthernet1/0/11

209.165.201.0/25 is subnetted, 1 subnets

D EX     209.165.201.0

[170/66560] via 10.1.32.3, 00:00:20, GigabitEthernet1/0/11

close configuration window

### Redistribute OSPFv3 into EIGRP for IPv6.

The name of the EIGRP process is CISCO. The AS is 64512. You are redistributing OSPFv3 process 1 routes. Use metric values of 1000000, 10, 255, 1, and 1500. Be sure to add include-connected to the redistribute command.

Note: When redistributing OSPFv3 routes into EIGRP, IPv6 requires the include-connected keyword to ensure connected IPv6 interfaces participating in OSPFv3 are redistributed. IPv4 connected routes are included by default.

R2(config)# router eigrp CISCO

R2(config-router)# address-family ipv6 autonomous-system 64512

R2(config-router-af)# topology base

R2(config-router-af-topology)# default-metric 1000000 10 255 1 1500

R2(config-router-af-topology)# redistribute ospf 1 include-connected

### On D2 Verify OSPFv3 redistribution for IPv6.

Issue the command to view the IPv6 routing table for EIGRP. You should see that the gateway of last resort is now set and D2 has learned all the external EIGRP for IPv6 routes, including 2001:db8:acad:12::/64, which originated from OSPFv3.

Open configuration window

D2# show ipv6 route eigrp | begin ::/0

EX  ::/0 [170/66560], tag 1

via FE80::32:3, GigabitEthernet1/0/11

EX  2001:DB8:0:41::/64 [170/66560]

via FE80::32:3, GigabitEthernet1/0/11

EX  2001:DB8:0:42::/64 [170/66560]

via FE80::32:3, GigabitEthernet1/0/11

EX  2001:DB8:0:43::/64 [170/66560]

via FE80::32:3, GigabitEthernet1/0/11

EX  2001:DB8:0:44::/64 [170/66560]

via FE80::32:3, GigabitEthernet1/0/11

EX  2001:DB8:209:165:201::/80 [170/66560]

via FE80::32:3, GigabitEthernet1/0/11

EX  2001:DB8:ACAD:1::/64 [170/66560]

via FE80::32:3, GigabitEthernet1/0/11

D   2001:DB8:ACAD:3::/64 [90/10880]

via FE80::32:3, GigabitEthernet1/0/11

EX  2001:DB8:ACAD:11::/64 [170/66560]

via FE80::32:3, GigabitEthernet1/0/11

EX  2001:DB8:ACAD:12::/64 [170/66560]

via FE80::32:3, GigabitEthernet1/0/11

D   2001:DB8:ACAD:23::/64 [90/15360]

via FE80::32:3, GigabitEthernet1/0/11

close configuration window

close configuration window

## Configure Redistribution from EIGRP for IPv4 into OSPFv3

In this part, you will perform EIGRP for IPv4 redistribution into OSPFv3. The configuration is identical to what you did in the previous lab. Refer to that lab if you need a refresher.

### On R2, redistribute EIGRP into OSPFv3.

The OSPFv3 process is 1. The EIGRP autonomous system is 64512. Set the metric-type to 1.

Note: Using metric-type 1 causes redistributed EIGRP routes to be advertised as OSPF external type 1 (E1) routes. With E1 routes, the OSPF metric includes both the external cost assigned at the ASBR and the internal OSPF cost to reach that ASBR, allowing routers to prefer the closest exit point when multiple ASBRs exist.

Open configuration window

R2(config)# router ospfv3 1

R2(config-router)# address-family ipv4 unicast

R2(config-router-af)# redistribute eigrp 64512 metric-type 1

close configuration window

### Verify redistribution on D1.

Issue the show ip route ospfv3 on D1 to see the external OSPF routes.

Open configuration window

D1# show ip route ospfv3 | begin Gateway

Gateway of last resort is 0.0.0.0 to network 0.0.0.0

10.0.0.0/8 is variably subnetted, 19 subnets, 2 masks

O IA     10.1.1.0/24 [110/2] via 10.1.11.1, 02:04:44, GigabitEthernet1/0/11

O IA     10.1.12.0/24 [110/2] via 10.1.11.1, 02:04:44, GigabitEthernet1/0/11

O E1     10.1.23.0/24 [110/22] via 10.1.11.1, 00:00:27, GigabitEthernet1/0/11

O E1     10.1.32.0/24 [110/22] via 10.1.11.1, 00:00:27, GigabitEthernet1/0/11

O E1     10.3.3.0/24 [110/22] via 10.1.11.1, 00:00:27, GigabitEthernet1/0/11

O E1     10.10.21.0/24 [110/22] via 10.1.11.1, 00:00:27, GigabitEthernet1/0/11

O E1     10.10.22.0/24 [110/22] via 10.1.11.1, 00:00:27, GigabitEthernet1/0/11

O E1     10.10.23.0/24 [110/22] via 10.1.11.1, 00:00:27, GigabitEthernet1/0/11

O E1     10.10.24.0/24 [110/22] via 10.1.11.1, 00:00:27, GigabitEthernet1/0/11

172.16.0.0/16 is variably subnetted, 10 subnets, 2 masks

O E1     172.16.21.0/24

[110/22] via 10.1.11.1, 00:00:27, GigabitEthernet1/0/11

O E1     172.16.22.0/24

[110/22] via 10.1.11.1, 00:00:27, GigabitEthernet1/0/11

198.51.100.0/25 is subnetted, 1 subnets

O E1     198.51.100.0 [110/22] via 10.1.11.1, 00:00:27, GigabitEthernet1/0/11

Close configuration window

From D2 ping the Loopback 0 interface on D1 using Loopback 0 interface of D2 as the source. The ping should be successful. This verifies successful two-way redistribution on R2 and end-to-end connectivity for IPv4.

Open configuration window

D2# ping 209.165.201.1 source loopback 0

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 209.165.201.1, timeout is 2 seconds:

Packet sent with a source address of 198.51.100.1

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 2/2/3 ms

Close configuration window

### Configure redistribution of EIGRP for IPv6 routes into OSPFv3 using a route map.

Next, you will redistribute EIGRP for IPv6 routes into OSPFv3 using a route map to set the external LSA to a metric type 1.

Create a route map with the name E20, and then set the metric type to 1 for E1.

Open configuration window

R2(config)# route-map E2O permit 10

R2(config-route-map)# set metric-type type-1

R2(config-route-map)# exit

Next, you access the OSPFv3 IPv6 address family. Then you issue the redistribute command and specify the route map name. Ensure to add the include-connected after the route map name.

R2(config)# router ospfv3 1

R2(config-router)# address-family ipv6

R2(config-router-af)# redistribute eigrp 64512 route-map E2O include-connected

R2(config-router-af)# exit

Close configuration window

### On D1 verify that routes from EIGRP for IPv6 are imported into OSPFv3 with the external metric type 1.

Issue the show ipv6 route ospf on D1 to see the external EIGRP routes. Notice the highlighted external E1 OSPF routes.

Open configuration window

D1# show ipv6 route ospf | begin 2001

OE1 2001:DB8:0:21::/64 [110/22]

via FE80::11:1, GigabitEthernet1/0/11

OE1 2001:DB8:0:22::/64 [110/22]

via FE80::11:1, GigabitEthernet1/0/11

OE1 2001:DB8:0:23::/64 [110/22]

via FE80::11:1, GigabitEthernet1/0/11

OE1 2001:DB8:0:24::/64 [110/22]

via FE80::11:1, GigabitEthernet1/0/11

OE1 2001:DB8:198:51:100::/80 [110/22]

via FE80::11:1, GigabitEthernet1/0/11

OI  2001:DB8:ACAD:1::/64 [110/2]

via FE80::11:1, GigabitEthernet1/0/11

OE1 2001:DB8:ACAD:3::/64 [110/22]

via FE80::11:1, GigabitEthernet1/0/11

OI  2001:DB8:ACAD:12::/64 [110/2]

via FE80::11:1, GigabitEthernet1/0/11

OE1 2001:DB8:ACAD:23::/64 [110/22]

via FE80::11:1, GigabitEthernet1/0/11

OE1 2001:DB8:ACAD:32::/64 [110/22]

via FE80::11:1, GigabitEthernet1/0/11

close configuration window

From D2, ping the Loopback address on D1 using Loopback address of D2. The ping should be successful. This verifies full successful two-way redistribution on R2 and end-to-end connectivity for IPv6.

Open configuration window

D2# ping 2001:db8:209:165:201::1 source loopback 0

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 2001:DB8:209:165:201::1, timeout is 2 seconds:

Packet sent with a source address of 2001:DB8:198:51:100::1

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 2/4/9 ms

close configuration window

## Configure Policies to Manipulate Route Redistribution

In this part, you will apply policies to manipulate route redistribution using ACLs, prefix lists, and route maps.

### Use distribute-list with an access list to filter IPv4 routes that are redistributed from OSPF into EIGRP.

Assume the policy is to not advertise the 10.10.x.x networks into EIGRP. You can do this with a standard ACL using a statement to deny each of the 10.10.x.x networks. Then you apply the ACL to a distribute-list inside the EIGRP for IPv4 address family.

R2(config)# access-list 5 deny 10.10.41.0 0.0.0.255

R2(config)# access-list 5 deny 10.10.42.0 0.0.0.255

R2(config)# access-list 5 deny 10.10.43.0 0.0.0.255

R2(config)# access-list 5 deny 10.10.44.0 0.0.0.255

R2(config)# access-list 5 permit any

R2(config)# router eigrp CISCO

R2(config-router)# address-family ipv4 unicast autonomous-system 64512

R2(config-router-af)# topology base

R2(config-router-af-topology)# distribute-list 5 out g0/0/1

R2(config-router-af-topology)#

*Jan  9 00:26:54.193: %DUAL-5-NBRCHANGE: EIGRP-IPv4 64512: Neighbor 10.1.23.3 (GigabitEthernet0/0/1) is resync: intf route configuration changed

After configuring access list 5 to deny the specified networks, configure the distribute-list under the EIGRP IPv4 address family topology configuration mode on R2. To prevent networks from being advertised, use deny statements in the access list. The out keyword in the distribute-list command specifies that subnets matching access list 5 will be filtered as routing updates leave the GigabitEthernet 0/0/1 interface towards R3. The in keyword would allow control of routes entering the routing table.

Note: The EIGRP neighbor relationship between R2 and D1 will reset after applying the distribute-list, as indicated by the DUAL-5-NBRCHANGE message.

Verify that the 10.10.x.x networks are no longer in the routing table of D2.

D2# show ip route eigrp | begin Gateway

Gateway of last resort is 10.1.32.3 to network 0.0.0.0

D*EX  0.0.0.0/0 [170/66560] via 10.1.32.3, 01:22:14, GigabitEthernet1/0/11

10.0.0.0/8 is variably subnetted, 15 subnets, 2 masks

D EX     10.1.1.0/24

[170/66560] via 10.1.32.3, 01:22:14, GigabitEthernet1/0/11

D EX     10.1.11.0/24

[170/66560] via 10.1.32.3, 01:22:14, GigabitEthernet1/0/11

D EX     10.1.12.0/24

[170/66560] via 10.1.32.3, 01:22:14, GigabitEthernet1/0/11

D        10.1.23.0/24

[90/15360] via 10.1.32.3, 01:39:23, GigabitEthernet1/0/11

D        10.3.3.0/24 [90/10880] via 10.1.32.3, 01:39:23, GigabitEthernet1/0/11

172.16.0.0/16 is variably subnetted, 12 subnets, 2 masks

D EX     172.16.41.0/24

[170/66560] via 10.1.32.3, 01:22:14, GigabitEthernet1/0/11

D EX     172.16.42.0/24

[170/66560] via 10.1.32.3, 01:22:14, GigabitEthernet1/0/11

D EX     172.16.43.0/24

[170/66560] via 10.1.32.3, 01:22:14, GigabitEthernet1/0/11

D EX     172.16.44.0/24

[170/66560] via 10.1.32.3, 01:22:14, GigabitEthernet1/0/11

209.165.201.0/25 is subnetted, 1 subnets

D EX     209.165.201.0

[170/66560] via 10.1.32.3, 01:22:14, GigabitEthernet1/0/11

### Use prefix-list to filter IPv4 routes that are redistributed from EIGRP into OSPF.

Prefix-list commands filter traffic by matching exact prefix lengths or ranges using these operators:

- ge (greater than or equal) - Matches prefix lengths from the specified value up to /32

- le (less than or equal) - Matches prefix lengths up to the specified value

- ge and le combined - Matches prefix lengths within the specified range

Routes must match both the base prefix/length and any ge/le range parameters specified.

Configuration Examples

ip prefix-list ANY_ROUTE permit 0.0.0.0/0 le 32   # Matches any route

ip prefix-list ONLY_DEFAULT permit 0.0.0.0/0      # Matches only default route

ip prefix-list List1 permit 192.168.0.0/16 le 20  # Matches /16 to /20 inclusive

ip prefix-list List2 permit 192.168.0.0/16 ge 18  # No match between /16 and /18

Key Points

- Without ge/le keywords, prefix lists require exact matches

- With ge/le keywords, prefix lists match any prefix within the specified address space if the subnet mask length falls within the defined range

- This provides more flexible filtering than using network/length arguments alone

Assume the policy is to only allow subnets belonging to 172.16.0.0/16 with prefix lengths up to /24 into OSPF. You can do this with a prefix list using a permit statement for 172.16.0.0/16 with the le 24 parameter, which matches any subnet within the 172.16.0.0/16 range that has a mask between /16 and /24. Then you apply the prefix list to a distribute-list inside the OSPFv3 process for IPv4 address family, filtering IPv4 routes redistributed from EIGRP 64512.

R2(config)# ip prefix-list EIGRP_TO_OSPF seq 5 permit 172.16.0.0/16 le 24

R2(config)# router ospfv3 1

R2(config-router)# address-family ipv4 unicast

R2(config-router-af)# distribute-list prefix EIGRP_TO_OSPF out eigrp 64512

Use the show ip prefix-list detail command to verify hits on the prefix list. The 2 hits correspond to the 172.16.13.0/24 and 172.16.21.0/24 prefixes that were redistributed from EIGRP into OSPF.

R2# show ip prefix-list detail

Prefix-list with the last deletion/insertion: EIGRP_TO_OSPF

ip prefix-list EIGRP_TO_OSPF:

count: 1, range entries: 1, sequences: 5 - 5, refcount: 3

seq 5 permit 172.16.0.0/16 le 24 (hit count: 2, refcount: 1)

Verify that D1 is no longer receiving routes for 172.16.0.0/16 subnets with a mask greater than /24. These routes are assigned a cost of 22, which is the default 20 for redistributed OSPF routes plus a cost of 2 to reach the ASBR, which is R2. Also, notice that D1 is also no longer receiving routes for any of the other IPv4 networks in the EIGRP for IPv4 routing process.

D1# show ip route ospfv3 | begin Gateway

Gateway of last resort is 0.0.0.0 to network 0.0.0.0

10.0.0.0/8 is variably subnetted, 12 subnets, 2 masks

O IA     10.1.1.0/24 [110/2] via 10.1.11.1, 03:56:12, GigabitEthernet1/0/11

O IA     10.1.12.0/24 [110/2] via 10.1.11.1, 03:56:12, GigabitEthernet1/0/11

172.16.0.0/16 is variably subnetted, 10 subnets, 2 masks

O E1     172.16.21.0/24

[110/22] via 10.1.11.1, 00:00:52, GigabitEthernet1/0/11

O E1     172.16.22.0/24

[110/22] via 10.1.11.1, 00:00:52, GigabitEthernet1/0/11

### Use route-map to filter IPv4 routes that are redistributed from EIGRP into OSPF.

Assume that the policy is to use route maps for packet and route manipulation. We can quickly change the above configuration to apply the prefix list in a route map. Then refer to the route map in the redistribute command. In the following configuration, we also use set commands to change the default OSPF metric for redistributed routes from 20 to 25.

Note: The ip prefix-list is already configured. It is shown here again for completeness.

R2(config)# ip prefix-list EIGRP_TO_OSPF seq 5 permit 172.16.0.0/16 le 24

R2(config)# route-map RED_TO_OSPF permit 10

R2(config-route-map)# match ip address prefix-list EIGRP_TO_OSPF

R2(config-route-map)# set metric 25

R2(config-route-map)# exit

R2(config)# router ospfv3 1

R2(config-router)# address-family ipv4 unicast

R2(config-router-af)# no distribute-list prefix EIGRP_TO_OSPF out eigrp 64512

R2(config-router-af)# redistribute eigrp 64512 route-map RED_TO_OSPF

Verify that the routing table on D1 has the same routes as before when we used the distribute-list method of route manipulation. Notice that the metric is now 27 instead of 22 for the two redistributed EIGRP networks

D1# show ip route ospfv3 | begin Gateway

Gateway of last resort is 0.0.0.0 to network 0.0.0.0

10.0.0.0/8 is variably subnetted, 12 subnets, 2 masks

O IA     10.1.1.0/24 [110/2] via 10.1.11.1, 20:31:32, GigabitEthernet1/0/11

O IA     10.1.12.0/24 [110/2] via 10.1.11.1, 20:31:32, GigabitEthernet1/0/11

172.16.0.0/16 is variably subnetted, 10 subnets, 2 masks

O E1     172.16.21.0/24

[110/27] via 10.1.11.1, 00:00:06, GigabitEthernet1/0/11

O E1     172.16.22.0/24

[110/27] via 10.1.11.1, 00:00:06, GigabitEthernet1/0/11

# Device Configs – Final

# Router R1

R1# show run

Building configuration...

Current configuration : 2330 bytes

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R1

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

no ip domain lookup

login on-success log

ipv6 unicast-routing

subscriber templating

multilink bundle-name authenticated

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

hash sha512

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

diagnostic bootup level minimal

license udi pid ISR4331/K9 sn FDO2218007Z

memory free low-watermark processor 61443

spanning-tree extend system-id

redundancy

mode none

interface Loopback0

ip address 10.1.1.1 255.255.255.0

ipv6 address FE80::1:1 link-local

ipv6 address 2001:DB8:ACAD:1::1/64

ospfv3 network point-to-point

ospfv3 1 ipv6 area 0

ospfv3 1 ipv4 area 0

interface GigabitEthernet0/0/0

ip address 10.1.12.1 255.255.255.0

negotiation auto

ipv6 address FE80::12:1 link-local

ipv6 address 2001:DB8:ACAD:12::1/64

ospfv3 1 ipv4 area 0

ospfv3 1 ipv6 area 0

interface GigabitEthernet0/0/1

ip address 10.1.11.1 255.255.255.0

negotiation auto

ipv6 address FE80::11:1 link-local

ipv6 address 2001:DB8:ACAD:11::1/64

ospfv3 1 ipv4 area 11

ospfv3 1 ipv6 area 11

interface GigabitEthernet0/0/2

no ip address

negotiation auto

interface Serial0/1/0

no ip address

interface Serial0/1/1

no ip address

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

negotiation auto

router ospfv3 1

router-id 1.1.1.1

address-family ipv4 unicast

passive-interface Loopback0

exit-address-family

address-family ipv6 unicast

passive-interface Loopback0

exit-address-family

ip forward-protocol nd

no ip http server

ip http secure-server

ip ssh bulk-mode 131072

control-plane

banner motd ^C R1, Manipulate Redistribution ^C

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line aux 0

line vty 0 4

login

transport input ssh

line vty 5 14

login

transport input ssh

end

# Router R2

R2# show running-config

Building configuration...

Current configuration : 3293 bytes

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R2

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

no ip domain lookup

login on-success log

ipv6 unicast-routing

subscriber templating

multilink bundle-name authenticated

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

hash sha512

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

diagnostic bootup level minimal

license udi pid ISR4321/K9 sn FDO20410713

memory free low-watermark processor 61443

spanning-tree extend system-id

redundancy

mode none

interface GigabitEthernet0/0/0

ip address 10.1.12.2 255.255.255.0

negotiation auto

ipv6 address FE80::12:2 link-local

ipv6 address 2001:DB8:ACAD:12::2/64

ospfv3 1 ipv4 area 0

ospfv3 1 ipv6 area 0

interface GigabitEthernet0/0/1

ip address 10.1.23.2 255.255.255.0

negotiation auto

ipv6 address FE80::23:2 link-local

ipv6 address 2001:DB8:ACAD:23::2/64

interface Serial0/1/0

no ip address

interface Serial0/1/1

no ip address

interface Serial0/2/0

no ip address

interface Serial0/2/1

no ip address

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

negotiation auto

router eigrp CISCO

address-family ipv4 unicast autonomous-system 64512

af-interface default

shutdown

exit-af-interface

af-interface GigabitEthernet0/0/1

no shutdown

exit-af-interface

topology base

distribute-list 5 out GigabitEthernet0/0/1

redistribute ospfv3 1 metric 1000000 10 255 1 1500

exit-af-topology

network 10.1.23.0 0.0.0.255

eigrp router-id 2.2.2.2

exit-address-family

address-family ipv6 unicast autonomous-system 64512

af-interface default

shutdown

exit-af-interface

af-interface GigabitEthernet0/0/1

no shutdown

exit-af-interface

topology base

default-metric 1000000 10 255 1 1500

redistribute ospf 1 include-connected

exit-af-topology

exit-address-family

router ospfv3 1

router-id 2.2.2.2

address-family ipv4 unicast

redistribute eigrp 64512 metric-type 1 route-map RED_TO_OSPF

exit-address-family

address-family ipv6 unicast

redistribute eigrp 64512 route-map E2O include-connected

exit-address-family

ip forward-protocol nd

no ip http server

ip http secure-server

ip ssh bulk-mode 131072

ip prefix-list EIGRP_TO_OSPF seq 5 permit 172.16.0.0/16 le 24

ip access-list standard 5

10 deny   10.10.41.0 0.0.0.255

20 deny   10.10.42.0 0.0.0.255

30 deny   10.10.43.0 0.0.0.255

40 deny   10.10.44.0 0.0.0.255

50 permit any

route-map E2O permit 10

set metric-type type-1

route-map RED_TO_OSPF permit 10

match ip address prefix-list EIGRP_TO_OSPF

set metric 25

control-plane

banner motd ^C R2, Manipulate Redistribution ^C

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line aux 0

line vty 0 4

login

transport input ssh

line vty 5 14

login

transport input ssh

end

# Router R3

R3# show running-config

Building configuration...

Current configuration : 3064 bytes

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R3

boot-start-marker

boot-end-marker

no aaa new-model

no ip domain lookup

login on-success log

ipv6 unicast-routing

subscriber templating

multilink bundle-name authenticated

crypto pki trustpoint TP-self-signed-3903873913

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-3903873913

revocation-check none

rsakeypair TP-self-signed-3903873913

hash sha512

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

hash sha512

crypto pki certificate chain TP-self-signed-3903873913

certificate self-signed 01

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

diagnostic bootup level minimal

license udi pid ISR4221/K9 sn FGL23313183

memory free low-watermark processor 61616

spanning-tree extend system-id

enable secret 9 $9$J62a4JDYgOtrjk$CVwsib4TTukzGmJCcCiEenDGMEgXhMjG62U63uFrbSQ

redundancy

mode none

interface Loopback0

ip address 10.3.3.3 255.255.255.0

ipv6 address FE80::3:3 link-local

ipv6 address 2001:DB8:ACAD:3::3/64

interface GigabitEthernet0/0/0

ip address 10.1.23.3 255.255.255.0

negotiation auto

ipv6 address FE80::23:3 link-local

ipv6 address 2001:DB8:ACAD:23::3/64

interface GigabitEthernet0/0/1

ip address 10.1.32.3 255.255.255.0

negotiation auto

ipv6 address FE80::32:3 link-local

ipv6 address 2001:DB8:ACAD:32::3/64

interface Serial0/1/0

no ip address

shutdown

interface Serial0/1/1

no ip address

shutdown

router eigrp CISCO

address-family ipv4 unicast autonomous-system 64512

af-interface default

shutdown

exit-af-interface

af-interface GigabitEthernet0/0/0

no shutdown

exit-af-interface

af-interface GigabitEthernet0/0/1

no shutdown

exit-af-interface

af-interface Loopback0

no shutdown

exit-af-interface

topology base

exit-af-topology

network 10.1.23.0 0.0.0.255

network 10.1.32.0 0.0.0.255

network 10.3.3.0 0.0.0.255

eigrp router-id 3.3.3.3

exit-address-family

address-family ipv6 unicast autonomous-system 64512

af-interface default

shutdown

exit-af-interface

af-interface GigabitEthernet0/0/0

no shutdown

exit-af-interface

af-interface GigabitEthernet0/0/1

no shutdown

exit-af-interface

af-interface Loopback0

no shutdown

exit-af-interface

topology base

exit-af-topology

eigrp router-id 3.3.3.3

exit-address-family

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip ssh bulk-mode 131072

control-plane

banner motd ^C R3, Manipulate Redistribution ^C

line con 0

exec-timeout 0 0

logging synchronous

transport input none

stopbits 1

line aux 0

line vty 0 4

login

transport input ssh

line vty 5 14

login

transport input ssh

end

# Switch D1

D1# show running-config

Building configuration...

Current configuration : 7240 bytes

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform punt-keepalive disable-kernel-core

hostname D1

vrf definition Mgmt-vrf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

switch 1 provision c9300l-24t-4g

ip routing

no ip domain lookup

login on-success log

ipv6 nd cache expire refresh

ipv6 unicast-routing

crypto pki trustpoint TP-self-signed-1929813547

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-1929813547

revocation-check none

rsakeypair TP-self-signed-1929813547

hash sha512

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

hash sha512

crypto pki certificate chain TP-self-signed-1929813547

certificate self-signed 01

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

license boot level network-advantage addon dna-advantage

memory free low-watermark processor 130178

diagnostic bootup level minimal

spanning-tree mode rapid-pvst

spanning-tree extend system-id

redundancy

mode sso

crypto engine compliance shield disable

transceiver type all

monitoring

class-map match-any system-cpp-police-ewlc-control

description EWLC Control

class-map match-any system-cpp-police-topology-control

description Topology control

class-map match-any system-cpp-police-sw-forward

description Sw forwarding, L2 LVX data packets, LOGGING, Transit Traffic

class-map match-any system-cpp-default

description EWLC Data, Inter FED Traffic

class-map match-any system-cpp-police-sys-data

description Openflow, Exception, EGR Exception, NFL Sampled Data, RPF Failed

class-map match-any system-cpp-police-punt-webauth

description Punt Webauth

class-map match-any system-cpp-police-l2lvx-control

description L2 LVX control packets

class-map match-any system-cpp-police-forus

description Forus Address resolution and Forus traffic

class-map match-any system-cpp-police-multicast-end-station

description MCAST END STATION

class-map match-any system-cpp-police-high-rate-app

description High Rate Applications

class-map match-any system-cpp-police-multicast

description MCAST Data

class-map match-any system-cpp-police-l2-control

description L2 control

class-map match-any system-cpp-police-dot1x-auth

description DOT1X Auth

class-map match-any system-cpp-police-data

description ICMP redirect, ICMP_GEN and BROADCAST

class-map match-any system-cpp-police-stackwise-virt-control

description Stackwise Virtual OOB

class-map match-any non-client-nrt-class

class-map match-any system-cpp-police-routing-control

description Routing control and Low Latency

class-map match-any system-cpp-police-protocol-snooping

description Protocol snooping

class-map match-any system-cpp-police-dhcp-snooping

description DHCP snooping

class-map match-any system-cpp-police-ios-routing

description L2 control, Topology control, Routing control, Low Latency

class-map match-any system-cpp-police-system-critical

description System Critical and Gold Pkt

class-map match-any system-cpp-police-ios-feature

description ICMPGEN,BROADCAST,ICMP,L2LVXCntrl,ProtoSnoop,PuntWebauth,MCASTData,Transit,DOT1XAuth,Swfwd,LOGGING,L2LVXData,ForusTraffic,ForusARP,McastEndStn,Openflow,Exception,EGRExcption,NflSampled,RpfFailed

policy-map system-cpp-policy

interface Loopback0

ip address 209.165.201.1 255.255.255.128

ipv6 address FE80::209:1 link-local

ipv6 address 2001:DB8:209:165:201::1/80

ospfv3 network point-to-point

ospfv3 1 ipv6 area 11

ospfv3 1 ipv4 area 11

interface Loopback1

ip address 10.10.41.1 255.255.255.0

ipv6 enable

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

interface Loopback2

ip address 10.10.42.1 255.255.255.0

ipv6 enable

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

interface Loopback3

ip address 10.10.43.1 255.255.255.0

ipv6 enable

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

interface Loopback4

ip address 10.10.44.1 255.255.255.0

ipv6 enable

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

interface Loopback41

ip address 172.16.41.1 255.255.255.0

ipv6 address FE80:41::1 link-local

ipv6 address 2001:DB8:0:41::1/64

ospfv3 network point-to-point

ospfv3 1 ipv6 area 11

ospfv3 1 ipv4 area 11

interface Loopback42

ip address 172.16.42.1 255.255.255.0

ipv6 address FE80:42::1 link-local

ipv6 address 2001:DB8:0:42::1/64

ospfv3 network point-to-point

ospfv3 1 ipv6 area 11

ospfv3 1 ipv4 area 11

interface Loopback43

ip address 172.16.43.1 255.255.255.0

ipv6 address FE80:43::1 link-local

ipv6 address 2001:DB8:0:43::1/64

ospfv3 network point-to-point

ospfv3 1 ipv6 area 11

ospfv3 1 ipv4 area 11

interface Loopback44

ip address 172.16.44.1 255.255.255.0

ipv6 address FE80:44::1 link-local

ipv6 address 2001:DB8:0:44::1/64

ospfv3 network point-to-point

ospfv3 1 ipv6 area 11

ospfv3 1 ipv4 area 11

interface GigabitEthernet0/0

vrf forwarding Mgmt-vrf

no ip address

negotiation auto

interface GigabitEthernet1/0/1

shutdown

interface GigabitEthernet1/0/2

shutdown

interface GigabitEthernet1/0/3

shutdown

interface GigabitEthernet1/0/4

shutdown

interface GigabitEthernet1/0/5

shutdown

interface GigabitEthernet1/0/6

shutdown

interface GigabitEthernet1/0/7

shutdown

interface GigabitEthernet1/0/8

shutdown

interface GigabitEthernet1/0/9

shutdown

interface GigabitEthernet1/0/10

shutdown

interface GigabitEthernet1/0/11

no switchport

ip address 10.1.11.2 255.255.255.0

ipv6 address FE80::11:2 link-local

ipv6 address 2001:DB8:ACAD:11::2/64

ospfv3 1 ipv4 area 11

ospfv3 1 ipv6 area 11

interface GigabitEthernet1/0/12

shutdown

interface GigabitEthernet1/0/13

shutdown

interface GigabitEthernet1/0/14

shutdown

interface GigabitEthernet1/0/15

shutdown

interface GigabitEthernet1/0/16

shutdown

interface GigabitEthernet1/0/17

shutdown

interface GigabitEthernet1/0/18

shutdown

interface GigabitEthernet1/0/19

shutdown

interface GigabitEthernet1/0/20

shutdown

interface GigabitEthernet1/0/21

shutdown

interface GigabitEthernet1/0/22

shutdown

interface GigabitEthernet1/0/23

shutdown

interface GigabitEthernet1/0/24

shutdown

interface GigabitEthernet1/1/1

interface GigabitEthernet1/1/2

interface GigabitEthernet1/1/3

interface GigabitEthernet1/1/4

interface AppGigabitEthernet1/0/1

interface Vlan1

no ip address

router ospfv3 1

router-id 11.11.11.11

address-family ipv4 unicast

passive-interface Loopback0

default-information originate

exit-address-family

address-family ipv6 unicast

passive-interface Loopback0

default-information originate

exit-address-family

ip forward-protocol nd

ip http server

ip http secure-server

ip route 0.0.0.0 0.0.0.0 Loopback0

ip ssh bulk-mode 131072

ipv6 route ::/0 Loopback0

control-plane

service-policy input system-cpp-policy

banner motd ^C D1, Manipulate Redistribution ^C

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line vty 0 4

login

transport input ssh

line vty 5 31

login

transport input ssh

end

# Switch D2

D2# show running-config

Building configuration...

Current configuration : 8919 bytes

version 17.12

service timestamps debug datetime msec

service timestamps log datetime msec

platform punt-keepalive disable-kernel-core

hostname D2

vrf definition Mgmt-vrf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

switch 1 provision c9300l-24t-4g

ip routing

no ip domain lookup

login on-success log

ipv6 nd cache expire refresh

ipv6 unicast-routing

crypto pki trustpoint TP-self-signed-1507993010

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-1507993010

revocation-check none

rsakeypair TP-self-signed-1507993010

hash sha512

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

hash sha512

crypto pki certificate chain TP-self-signed-1507993010

certificate self-signed 01

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

license boot level network-advantage addon dna-advantage

memory free low-watermark processor 130178

diagnostic bootup level minimal

spanning-tree mode rapid-pvst

spanning-tree extend system-id

redundancy

mode sso

crypto engine compliance shield disable

transceiver type all

monitoring

class-map match-any system-cpp-police-ewlc-control

description EWLC Control

class-map match-any system-cpp-police-topology-control

description Topology control

class-map match-any system-cpp-police-sw-forward

description Sw forwarding, L2 LVX data packets, LOGGING, Transit Traffic

class-map match-any system-cpp-default

description EWLC Data, Inter FED Traffic

class-map match-any system-cpp-police-sys-data

description Openflow, Exception, EGR Exception, NFL Sampled Data, RPF Failed

class-map match-any system-cpp-police-punt-webauth

description Punt Webauth

class-map match-any system-cpp-police-l2lvx-control

description L2 LVX control packets

class-map match-any system-cpp-police-forus

description Forus Address resolution and Forus traffic

class-map match-any system-cpp-police-multicast-end-station

description MCAST END STATION

class-map match-any system-cpp-police-high-rate-app

description High Rate Applications

class-map match-any system-cpp-police-multicast

description MCAST Data

class-map match-any system-cpp-police-l2-control

description L2 control

class-map match-any system-cpp-police-dot1x-auth

description DOT1X Auth

class-map match-any system-cpp-police-data

description ICMP redirect, ICMP_GEN and BROADCAST

class-map match-any system-cpp-police-stackwise-virt-control

description Stackwise Virtual OOB

class-map match-any non-client-nrt-class

class-map match-any system-cpp-police-routing-control

description Routing control and Low Latency

class-map match-any system-cpp-police-protocol-snooping

description Protocol snooping

class-map match-any system-cpp-police-dhcp-snooping

description DHCP snooping

class-map match-any system-cpp-police-ios-routing

description L2 control, Topology control, Routing control, Low Latency

class-map match-any system-cpp-police-system-critical

description System Critical and Gold Pkt

class-map match-any system-cpp-police-ios-feature

description ICMPGEN,BROADCAST,ICMP,L2LVXCntrl,ProtoSnoop,PuntWebauth,MCASTData,Transit,DOT1XAuth,Swfwd,LOGGING,L2LVXData,ForusTraffic,ForusARP,McastEndStn,Openflow,Exception,EGRExcption,NflSampled,RpfFailed

policy-map system-cpp-policy

interface Loopback0

ip address 198.51.100.1 255.255.255.128

ipv6 address FE80::198:2 link-local

ipv6 address 2001:DB8:198:51:100::1/80

ospfv3 network point-to-point

ospfv3 1 ipv4 area 11

interface Loopback1

ip address 10.10.21.1 255.255.255.0

ospfv3 network point-to-point

interface Loopback2

ip address 10.10.22.1 255.255.255.0

ospfv3 network point-to-point

interface Loopback3

ip address 10.10.23.1 255.255.255.0

ospfv3 network point-to-point

interface Loopback4

ip address 10.10.24.1 255.255.255.0

ospfv3 network point-to-point

interface Loopback21

ip address 172.16.21.1 255.255.255.0

ipv6 address FE80:21::1 link-local

ipv6 address 2001:DB8:0:21::1/64

ospfv3 network point-to-point

ospfv3 1 ipv6 area 11

ospfv3 1 ipv4 area 11

interface Loopback22

ip address 172.16.22.1 255.255.255.0

ipv6 address FE80:22::1 link-local

ipv6 address 2001:DB8:0:22::1/64

ospfv3 network point-to-point

ospfv3 1 ipv6 area 11

ospfv3 1 ipv4 area 11

interface Loopback23

ip address 172.16.23.1 255.255.255.0

ipv6 address FE80:23::1 link-local

ipv6 address 2001:DB8:0:23::1/64

ospfv3 network point-to-point

ospfv3 1 ipv6 area 11

ospfv3 1 ipv4 area 11

interface Loopback24

ip address 172.16.24.1 255.255.255.0

ipv6 address FE80:24::1 link-local

ipv6 address 2001:DB8:0:24::1/64

ospfv3 network point-to-point

ospfv3 1 ipv6 area 11

ospfv3 1 ipv4 area 11

interface GigabitEthernet0/0

vrf forwarding Mgmt-vrf

no ip address

negotiation auto

interface GigabitEthernet1/0/1

shutdown

interface GigabitEthernet1/0/2

shutdown

interface GigabitEthernet1/0/3

shutdown

interface GigabitEthernet1/0/4

shutdown

interface GigabitEthernet1/0/5

shutdown

interface GigabitEthernet1/0/6

shutdown

interface GigabitEthernet1/0/7

shutdown

interface GigabitEthernet1/0/8

shutdown

interface GigabitEthernet1/0/9

shutdown

interface GigabitEthernet1/0/10

shutdown

interface GigabitEthernet1/0/11

no switchport

ip address 10.1.32.2 255.255.255.0

ipv6 address FE80::32:2 link-local

ipv6 address 2001:DB8:ACAD:32::2/64

interface GigabitEthernet1/0/12

shutdown

interface GigabitEthernet1/0/13

shutdown

interface GigabitEthernet1/0/14

shutdown

interface GigabitEthernet1/0/15

shutdown

interface GigabitEthernet1/0/16

shutdown

interface GigabitEthernet1/0/17

shutdown

interface GigabitEthernet1/0/18

shutdown

interface GigabitEthernet1/0/19

shutdown

interface GigabitEthernet1/0/20

shutdown

interface GigabitEthernet1/0/21

shutdown

interface GigabitEthernet1/0/22

shutdown

interface GigabitEthernet1/0/23

shutdown

interface GigabitEthernet1/0/24

shutdown

interface GigabitEthernet1/1/1

interface GigabitEthernet1/1/2

interface GigabitEthernet1/1/3

interface GigabitEthernet1/1/4

interface AppGigabitEthernet1/0/1

interface Vlan1

no ip address

router eigrp CISCO

address-family ipv4 unicast autonomous-system 64512

af-interface default

shutdown

exit-af-interface

af-interface Loopback0

no shutdown

passive-interface

exit-af-interface

af-interface Loopback1

no shutdown

exit-af-interface

af-interface Loopback2

no shutdown

exit-af-interface

af-interface Loopback3

no shutdown

exit-af-interface

af-interface Loopback4

no shutdown

exit-af-interface

af-interface Loopback21

no shutdown

exit-af-interface

af-interface Loopback22

no shutdown

exit-af-interface

af-interface Loopback23

no shutdown

exit-af-interface

af-interface Loopback24

no shutdown

exit-af-interface

af-interface GigabitEthernet1/0/11

no shutdown

exit-af-interface

topology base

exit-af-topology

network 10.1.32.0 0.0.0.255

network 10.10.21.0 0.0.0.255

network 10.10.22.0 0.0.0.255

network 10.10.23.0 0.0.0.255

network 10.10.24.0 0.0.0.255

network 172.16.21.0 0.0.0.255

network 172.16.22.0 0.0.0.63

network 172.16.22.64 0.0.0.63

network 172.16.22.128 0.0.0.63

network 198.51.100.0 0.0.0.127

eigrp router-id 22.22.22.22

exit-address-family

address-family ipv6 unicast autonomous-system 64512

af-interface default

shutdown

exit-af-interface

af-interface Loopback0

no shutdown

passive-interface

exit-af-interface

af-interface Loopback1

no shutdown

exit-af-interface

af-interface Loopback2

no shutdown

exit-af-interface

af-interface Loopback3

no shutdown

exit-af-interface

af-interface Loopback4

no shutdown

exit-af-interface

af-interface Loopback21

no shutdown

exit-af-interface

af-interface Loopback22

no shutdown

exit-af-interface

af-interface Loopback23

no shutdown

exit-af-interface

af-interface Loopback24

no shutdown

exit-af-interface

af-interface GigabitEthernet1/0/11

no shutdown

exit-af-interface

topology base

exit-af-topology

eigrp router-id 22.22.22.22

exit-address-family

router ospfv3 1

address-family ipv4 unicast

exit-address-family

address-family ipv6 unicast

exit-address-family

ip forward-protocol nd

ip http server

ip http secure-server

ip ssh bulk-mode 131072

control-plane

service-policy input system-cpp-policy

banner motd ^C D2, Manipulate Redistribution ^C

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line vty 0 4

login

transport input ssh

line vty 5 31

login

transport input ssh

end